3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304708

# Online, April 17 – 26, 2023

Source:	LG Electronics

Title:	Discussion on L1-RSRP measurement for FR2 multi-Rx chains

Agenda item:	5.8.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on L1-RSRP measurement for multi-Rx chains.

Discussion

Beam sweeping factor reduction

Group-based reporting is one of the methods enabling simultaneous signal reception from multi-TRP. The description of group-based reporting is

- For Rel-17 : UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRI selecting on CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

According to the description, if group-based reporting is configured, a UE reports the best beam pairs based on each resource set in consideration of simultaneous reception. To find the best beam pair, a UE may need to perform full Rx beam sweeping to measure the signal from each TRP since each UE antenna panel is not paired to each TRP. If multi-Rx chain is activated during the measurement or evaluation period, the UE can receive measurement resource signal using multi-Rx chain depending on UE implementation. In that case, Rx beam sweeping factor for measurement or evaluation period can be reduced, and the capability for the number of Rx beam sweeping might be needed. It would depend on UE implementation for beam operation, so the number of Rx beam sweeping factor including existing value should be reported by UE.

- Proposal 1: Introduce reduced Rx beam sweeping factor for L1 measurements along with the capability of supported Rx beam sweeping factor.

For the requirements definition, since intra-cell based mTRP is considered in this release, there would be no simultaneous transmission of SSB or CSI-RS from the multi-TRP. Therefore, no separated requirements for group based reporting is needed, but if beam sweeping factor reduction is introduced, the reduced beam sweeping factor should be reflected in SSB based L1-RSRP measurement requirements.

- Proposal 2: Do not introduce the requirements for group based reporting since intra-cell based mTRP is considered. If redueced beam sweeping factor is introduced, it should be reflected in SSB based L1-RSRP measurement requirements.

Conclusion

In this contribution, we provide our views on L1-RSRP measurement for multi-Rx, and we propose

- Proposal 1: Introduce reduced Rx beam sweeping factor for L1 measurements along with the capability of supported Rx beam sweeping factor.

- Proposal 2: Do not introduce the requirements for group based reporting since intra-cell based mTRP is considered. If redueced beam sweeping factor is introduced, it should be reflected in SSB based L1-RSRP measurement requirements.

Reference