3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304717

# Electronic meeting, 17th - 26th, April, 2023

Source: 	Samsung

Title: 	Views on signaling design for lower MSD

Agenda Item:	5.5.2.3.1

Document for:	Discussion

- Introduction

In RAN#99, the study phase of Lower MSD was concluded, and the new objective was approved as [1]. More details of the achievement in RAN4 could be found in [2].

Enable lower MSD capability for inter-band CA/EN-DC/DC combinations [RAN4, RAN2]

- Continue the study on approaches for a UE to indicate the improved lower MSD performance based on the progress of the study phase and specify the related signalling

- Based on the outcome of the signalling study, specify the UE RF requirements to support lower MSD capability

In this paper, we share our views on the remaining issue captured in [3] and some new issues identified.

- 2. Discussion

2.1 Conditions to indicate Lower MSD capability

Issue 2-1-2: Conditions to indicate the lower MSD capability

Option 1: UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. Additionally, it is unnecessary to report the Lower MSD values in case the specified MSD itself is small or the MSD improvement is not significant. However, if UE is willing to report the values under these cases, it should not be prohibited.

Option 2: measures must be discussed to avoid a situation that UE can report lower MSD capability without any actual improvement.

Option 3: UE report improved MSD as long as the real MSD is below the largest value range, for example 20dB. The band combinations with less than [5dB] MSD requirements in the spec doesn’t need to report the improved MSD.

Option 4: Others

- WF

- UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved.

- The amount of MSD improvement necessary for indication needs further study

In last meeting, companies had extensive discussion on whether there is necessity to “quantify” improvement, that is to say whether we need to set some thresholds to clarify what does “small MSD” mean, and what does “small MSD improvement” mean, separately.

At first glance, it seems unnecessary to report the Lower MSD values in case the specified MSD itself is small or the MSD improvement is not significant, and we think in some scenarios UE may also have less interest to report the Lower MSD capability in these two cases since this capability is expected to have several capability classes, indicating this capability is not necessarily link to a very low actual MSD nor all kinds of MSD are improved, NW may still need to further check. And in some scenarios, even 1 dB MSD improvement is important, which may bring in significant SNR improvement.

In addition, we think it might be challenging to quantify both “small MSD” and “small MSD improvement”, so left UE more flexibility to make the decision is preferred and there would be no threshold to quantify “small MSD” and “small MSD improvement”.

Proposal 1：Remove the sub-bullet in the agreement of last meeting.

- UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved.

- The amount of MSD improvement necessary for indication needs further study

2.2 How to derive and verify Lower MSD capability

2.2.1 Further refinement of last meeting’s agreement

In last meeting, there are two agreements made regarding the aggressor orders for lower MSD and the conditions to derive and verify Lower MSD capability.

For the interference/aggressor orders considered for low MSD, the agreement was reproduced as below.

Issue 2-1-5: Interference/aggressor orders considered for lower MSD

AH Agreement

Option 2: A UE should be allowed to report the low MSD capability for any MSD requirements that have been defined in the 3GPP specifications for a given band combination.

- The reported low MSD should be tested againt the existing test configuations.

From the title of Issue 2-1-5 it is clear the agreement is for MSD orders although the description of the agreement says “…for any MSD requirement….” in which seems include both orders and all the configurations in this order, but we think it is enough to derive and verify Lower MSD capability under the worst case configuration which corresponds to the largest MSD, for each MSD order, which is captured in another agreement as below:

Issue 2-3-3: Test configurations for lower MSD

- WF

- For all impairments the same UL/DL configurations and test points as for the minimum requirements

- If the same impairment order (e.g. IMD5, H3 etc.) has multiple test configurations, the one having largest MSD is chosen to be tested

- For cross-band isolation this applies per band combination

One point deserve further consideration is for the UE does not support the specified configuration (neither TP1# , TP#2 nor TP#3) for cross band isolation, for which the worst case configuration (mandatory to be defined) is under the maximum CBW of aggressor band and minimum DL CBW of victim band of the band combination, but what if one UE does not support the maximum CBW specified for aggressor band of this band combination? For example, one band combination has both BCS0 and BCS1, and when BCS1 was introduced with wider CBW of the aggressor band than BCS0, the cross band isolation requirement for BCS1 would override the requirement for BCS0, but chances are some UE does not support BSC1 but only BCS0, so there would be no straightforward MSD and test point to refer to in the latest spec. Let alone in future there might be ＜5MHz channel bandwidth to be introduced which may lead to a huge update for MSD table. Note that this is a common issue for both specified MSD and lower MSD capability in terms of conformance test, i.e., when UE does not support the specified MSD configuration, which configuration and MSD value to assume for conformance test? For the specified MSD conformance test, it is a legacy issue (from Rel-17) which might could be addressed either in maintenance WI or in this WI by sending LS to RAN5 with further clarification, for lower MSD, even after the maintenance issue is addressed, we also need to discuss the condition for deriving and verifying lower MSD capability which should be captured in RAN4 spec.

With above, we think the agreement made above probably need more discussion in terms of cross band isolation

Proposal 2: It is suggested to further discuss the test configuration for the specified minimum MSD for conformance test, i.e., How to deal with the case UE does not support the specified configuration?

An alternative is: Send LS to RAN5 with clarification that for one band combination, in case UE does not support any of the specified configuration, the worst case configuration the UE supported itself for this band combination should be chosen as test configuration and conform to the largest MSD specified (cross band isolation could be an example).

Proposal 3: For cross band isolation, in terms of lower MSD capability:

- If UE supports the specified worst case configuration which corresponds to the largest MSD, it should be chosen to verify lower MSD capability

- If UE does not support the specified worst case configuration，FFS on the test configuration for lower MSD

Alt 1: The worst case configuration the UE supports itself is chosen as test configuration (→ But may lead to new test point against the exiting specified test point for conformance test)

Alt 2: Others

An example for illustrating proposal 3 is shown below, if one UE does not support 50MHz for n1 within n1-n38, how to derive and verify lower MSD capability?

| n1 | n38 | 1955 | 50 | 15 | 128 (RBstart=142) | 2572.5 | 5 | 2.9 | >ACLR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | n38 | 1955 | 50 | 15 | 128 (RBstart=142) | 2590 | 40 | 2.9 | >ACLR2 |

2.3 Essential information included in the lower MSD capability.

2.2.1 Differentiation of the lower MSD capability for power classes

In RAN4#104-bis-e meeting, we proposed that “Lower MSD capability is applicable for PC1.5, PC2 and PC3. Particularly, for one band combination with specific UL and DL, Lower MSD capability is subject to the power class the band combination indicated”, the justification is that UE would only report one PC the UE supported for a BC, rather than the enumeration of all PCs, therefore we thought the Lower MSD capability is along with the PC the UE indicated for the BC.

At the moment we think more issues require due consideration. It is identified that NW may have demand to get the Lower MSD information for different power classes, and as [4] analyzed and we also agree, it is impractical for NW to derive the Lower MSD capability for lower power classes according to the Lower MSD capability for the power class the UE indicated, since the delta MSD differs for different band combinations especially for IMD. For example, as we know HPUE is about to be allowed in Japan for TDD next year but FDD not yet, and if a higher power UE roams to Japan whose power is limited by p-max, the Lower MSD capability cannot be used if this capability is only supported for the power class the UE indicated. Hence, we made below proposal.

Proposal 4: Lower MSD capability is applicable for PC1.5, PC2 and PC3. Allow UE to report Lower MSD capability for different power classes.

2.3.2 Essential information included in the lower MSD capability

In last meeting, the victim band, the MSD type with orders and MSD values/thresholds were agreed to be included in the essential information to be made aware to NW. Besides them, we think power class should also be included as one essential information with the justification elaborated in clause 2.3.1.

For aggressor UL and victim DL bandwidth, we think they are not necessary to be included, since it is expected the condition to derive and verify lower MSD capability for a band combination would be clarified in RAN4 spec with some general description or some notes added in the REFERENS exception tables, perhaps something like:

For harmonic, the condition to derive and verify the lower MSD capability for one band combination for certain power class is under the worst case UL/DL configuration and test point which corresponds to the largest MSD in this table (or Table XXX).

With above, it would be clear to NW what is the aggressor UL and victim DL CBW for one band combination in terms of lower MSD capability. In case a UE does not support the specified worst case configuration, the NW is still aware that the lower MSD capability for this UE for certain MSD mechanism is attached to the worst case configuration the UE itself supported for this MSD mechanism.

In addition, in our view, it would be good to leave the detailed signalling approach for RAN2 decision with RAN4’ s input on what information should be made aware to NW.

Proposal 5: Power class could be included as one essential information to be made aware to NW, while CBW of aggressor UL and victim DL are not necessary to be included.

2.3 MSD thresholds

It was agreed in last meeting that identical lower MSD thresholds are applicable for each MSD mechanism, and regarding the exact values of the threshold set, consideration on the maximum threshold and the granularity (linear steps or nonlinear steps) are necessary.

From our side, in RAN4#104-bis-e meeting we proposed a table for “2-bit-signalling solution” for thresholds based on our analysis of MSD improvement for the group reference, and thanks for Nokia and CHTTL pointed out that “-” wastes one bit, and in the following meetings we continued to suggest 0/5/10/15 as thresholds.

| Bit | Maximum allowed actual MSD (i.e. Thresholds) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 00 | 0dB | Ⅰ | Actual MSD = 0 |
| 01 | 5 dB | Ⅱ | Actual MSD ≤ 5 |
| 10 | 10 dB | Ⅲ | Actual MSD ≤ 10 |
| 11 | 15 dB | IV | Actual MSD ≤ 15 |

If absent, it means either the actual MSD is larger than the maximum threshold, or the actual MSD has minor improvement (also including the case the specified MSD itself is already very small), so UE choose not to report the capability class.

Proposal 6: It is suggested to consider 0/5/10/15dB as thresholds applicable for all kinds of MSD.

2.4 Applicability of the lower MSD thresholds for different MSD type

AH Agreement

Identical Lower MSD thresholds are applicable for each MSD mechanism

Above is the agreement made in last meeting, which could be interpreted as different power classes share one set of MSD threshold, as well as different orders for one kind of MSD also share one set of MSD threshold, and we think this interpretation is fine to serve simplifying capability purpose, i.e., there is just one set of MSD threshold in total.

Proposal 7: One set of MSD threshold could be applicable to different MSD mechanism and different orders within this mechanism, as well as different power classes.

- 3. Conclusion

Proposal 1：Remove the sub-bullet in the agreement of last meeting.

- UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved.

- The amount of MSD improvement necessary for indication needs further study

Proposal 2: It is suggested to further discuss the test configuration for the specified minimum MSD for conformance test, i.e., How to deal with the case UE does not support the specified configuration?

An alternative is: Send LS to RAN5 with clarification that for one band combination, in case UE does not support any of the specified configuration, the worst case configuration the UE supported itself for this band combination should be chosen as test configuration and conform to the largest MSD specified (cross band isolation could be an example).

Proposal 3: For cross band isolation, in terms of lower MSD capability:

- If UE supports the specified worst case configuration which corresponds to the largest MSD, it should be chosen to verify lower MSD capability

- If UE does not support the specified worst case configuration，FFS on the test configuration for lower MSD

Alt 1: The worst case configuration the UE supports itself is chosen as test configuration (→ But may lead to new test point against the exiting specified test point for conformance test)

Alt 2: Others

Proposal 4: Lower MSD capability is applicable for PC1.5, PC2 and PC3. Allow UE to report Lower MSD capability for different power classes.

Proposal 5: Power class could be included as one essential information to be made aware to NW, while CBW of aggressor UL and victim DL are not necessary to be included.

Proposal 6: It is suggested to consider 0/5/10/15dB as thresholds applicable for all kinds of MSD.

Proposal 7: One set of MSD threshold could be applicable to different MSD mechanism and different orders within this mechanism, as well as different power classes.

- 4. Reference

[1] RP-230753, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), RAN#99, Huawei, HiSilicon

[2] RP-230169, On concluding the study phase of lower MSD, RAN#99, Samsung

[3] R4-2303695, WF on study for lower MSD, RAN4#106, Huawei, HiSilicon

[4] R4-2218555, Power Class aspect on lower MSD, Nokia, Nokia Shanghai Bell, RAN4#105