3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2304721

# Electronic meeting,17th - 26th, April, 2023

Source: 	Samsung, TELUS, Bell Mobility

Title: 	Views on low band 4Rx for handheld UE

Agenda Item:	4.29.2

Document for:	Discussion

- Introduction

A new WID was approved in RAN#98 meeting while further revised as [1] in RAN#99, in which study the feasibility of low band 4Rx for handheld UE and specify the requirement if confirmed as feasible is included as one objective. In last meeting, the group had some initial discussion on the feasibility and the relevant requirement, the agreements are captured in [2].

In this paper, we share our views on the methodology to introduce low band 4Rx for handheld UE and the relevant requirements.

- Discussion

Working Methodology

The agreement for the feasibility study on low band 4Rx for handheld UE is copied as below.

Issue 2-1-1: Feasibility of supporting 4Rx in low bands (<1GHz)

Agreement: (agreed in Main session)

- Supporting 4Rx in low bands may be feasible at least for some handheld UE and complexity might be high or gain might be low for some other UE which is UE implementation dependent.

- Supporting 4Rx in low bands is an optional feature.

- Interested companies are encouraged to provide more simulation or measurement data.

Indeed, pursuit of implementing more low antennas in smartphone may lead to less CA and/or MIMO layers, or at the sacrifice of other features, but the gain and loss should be left to vendors and operators to measure and determine. It is observed there are already some handheld UE supporting 4 low Rx in the market, with that we think implementing 4Rx operation for low band is feasible at least for some handheld UE. In addition, in terms of the working methodology, once the feasibility of low band 4Rx for handheld UE is confirmed, 4Rx operation should be endowed for all the bands included and to be included in the WI, rather than only enabling this feature for n28. Similarly, 8Rx feature is to be enabled for all requested bands in FR1_enh.

Proposal 1: Once the feasibility of low band 4Rx for handheld UE is confirmed, 4Rx is supposed to be endowed for all the requested low bands included and to be included in the WI.

Delta TRxSRS

- Proposals

  - Option 1: Doesn’t need to specify with reasons below

    - All the requested lower bands currently are FDD bands in the WID, and SRS antenna switching is designed for TDD bands.

- WF

  - FFS whether specific delta TRxSRS for FDD low bands need to be defined in this WI.

The popular perception in RAN4 is SRS antenna switching is used for TDD bands for which the spectrum of DL and UL are the same, so DL performance could be evaluated by gNB through SRS antenna switching of UE. However, this capability is not specifically for TDD in RAN1 spec, furthermore, RAN1 designed FDD partial reciprocity codebook in Rel-17 which makes SRS antenna switching for FDD bands also usable.

From RAN4 spec perspective, it also not mentioned delta TRxSRS requirement is for TDD bands and/or FDD bands, but 3dB for bands whose FUL_high is lower than n79 and 4.5dB for bands whose FUL_high is higher than n79, when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16. With above, we think if in future SRS antenna switching is also enabled for FDD from implementation perspective, 3dB could be used as requirement as well and currently there is no action (spec update) needed in RAN4.

Proposal 2: No action (spec update) needed in RAN4 though SRS antenna switching is also possible for FDD bands.

Signalling to differentiate requirements for handheld UE and FWA

- Proposals

  - Option 1: New signaling is not introduced, no matter the delta RIB,4R requirement is same or different between handheld UE and FWA, considering the benefit of such signaling is unclear.

- WF

  - Whether new signalling is needed to differentiate requirements for FWA and handheld UE can be discussed after the requirements are defined.

We support Option 1, considering NW does not care about the UE type, correspondingly there is no need to define a new signalling to differentiate RF requirements for different UE types, as always.

Proposal 3: No new signalling is expected to differentiate the requirements for different UE types.

- 3. Conclusion

Proposal 1: Once the feasibility of low band 4Rx for handheld UE is confirmed, 4Rx is supposed to be endowed for all the requested low bands included and to be included in the WI.

Proposal 2: No action (spec update) needed in RAN4 though SRS antenna switching is also possible for FDD bands.

Proposal 3: No new signalling is expected to differentiate the requirements for different UE types.

- 4. Reference

[1] RP-230161, 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#99

[2] WF on 4Rx requirements for low operating bands, OPPO, RAN4#106