3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2304722

# Electronic meeting,17th - 26th, April, 2023

Source: 	Samsung

Title: 	Views on LP-WUR architectures

Agenda Item:	5.21.2

Document for:	Discussion

- Introduction

A new SID has been established to study the lower-power wake-up signal and receiver for NR while further revised as [1]. In last RAN4 meeting, the reply LS [2] was sent out to RAN1 with some initial assumption and agreement reached in RAN4.

In this paper, we further discuss the remaining issues identified.

- 2. Discussion

2.1 Criteria for architecture selection

In our view, the coverage of LP-WUS should be comparable with the legacy signals of the main receiver, for sake of extensive application of LP-WUS. If the coverage of LP-WUS is more degraded compared with existing NR signal, it is anticipated that only UE at the cell center could benefit from this new feature, unfortunately the UE at the cell edge who suffers more sever power issue and has more urgent demand to prong the battery life is not able to utilize LP-WUS.

In the reply LS [2], RAN4 asks the feedback from RAN1 on the targets of power consumption, coverage and SNR. Our expectation is that within the targeted power consumption range and coverage range, the architecture design (including component selection) for LP-WUR should strive to comparable coverage/REFSENS with legacy signals. To achieve comparable coverage with the existing signal of main receiver, LP-WUR may not achieve maximum energy efficiency, however, the power consumption could still be lower than main radio due to the adoption of non-coherent OOK demodulation which is widely used for ultra-low power receivers. In addition, the average power consumption of LP-WUR can be further reduced by duty cycle monitoring mechanism of LP-WUS.

With above, we re-propose our proposal of last meeting.

Proposal 1: The architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, which is considered as one criteria for architecture selection.

2.2 Down-selection of WUR-architectures

RAN4 had discussed the pros and cons of the three architectures recommend by RAN1 in last meeting. More details could be found in [3], in summary, though the architecture with RF-ED could achieve the lowest power consumption compared with the other two, the poor receiver sensitivity/coverage of this architecture has no possibility to match with existing NR signals. In addition, it was agreed that it is more suitable for single-band operation while we think multi-band supporting (LP-WUS is multiplexed with existing NR signal in frequency) is the main interest.

Therefore, we re-propose our proposal of last meeting.

Proposal 2: It is proposed to exclude RF envelop detection architecture for LP-WUR.

2.3 The feasible Noise Figure

As below equation of REFSENS shown, NF (Noise figure) is a determined factor for evaluation of sensitivity, along with other parameters such as channel bandwidth, the required SNR (signal to noise radio) and the implementation margin.

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM

As mentioned above RF-ED has the worst REFSENS due to the large NF and the poor performance of interference suppression, as we proposed above, it is suggested to exclude this architecture, hence we do not discuss the feasible NF for this architecture here.

The agreement in RAN1 regards to NF is reproduced as below, the options are from 9 to 24. In RAN4, the common assumption of NF for REFSENS evaluation is 9dB for TDD band and FDD band with large frequency gap, 12dB for high frequency FDD band or FDD band with small frequency gap. It should also be noted that the NF is mainly dominated by the components at the beginning of the chain, i.e. RF filter and RF LNA. Very likely WUS would share RFFE with legacy NR signals, then different signals would be routed to main receiver or WUR separately, additionally main receiver and WUR could also be integrated. With above, it is expected that NF is not going to be much larger than that for legacy NR signals with the assumption the detectable SNR is similar (around -1dB).

On the other hand, we also agree the clarification in [4] that the assumed NF also depends on the targeted SNR where the wake-up signal will be designed to be detectable. NF could be higher if the target SNR is low enough without negative impact to the system operation. So realistically the exact noise figure may not be agreed for the moment since SNR is still under evaluation in RAN1.

To achieve comparable REFSENS with existing NR signal, it is expected that LNA/AMP/filters cannot be omitted for LP-WUR, while a more power-saving LO might be adopted. Hence it is expected that the NF would be worse for LP-WUR than for main radio, but for sake of comparable REFSENS with existing NR signal, huge NF should not be pursued. Therefore, in our view, the feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to achieve comparable REFSENS with legacy NR signals.

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Observation 1: Realistically the exact noise figure may not be agreed for the moment.

Proposal 3: The feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to comparable REFSENS with legacy NR signals.

2.4 WUS location

It was agreed to FFS on whether WUS can be located in a band separate from the UE’s NR band. Tough it may alleviate the interference issue at both UE side and NW side, while it calls for additional implementation of this dedicated band at NW side for both hardware and software, which means the legacy NW is not able to support LP-WUS functionality that makes the new feature less meaningful.

Proposal 4: It is not preferable to assume a dedicated band for WUS.

- 3. Conclusion

Proposal 1: The architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, which is considered as one criteria for architecture selection.

Proposal 2: It is proposed to exclude RF envelop detection architecture for LP-WUR.

Observation 1: Realistically the exact noise figure may not be agreed for the moment.

Proposal 3: The feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to comparable REFSENS with legacy NR signals.

Proposal 4: It is not preferable to assume a dedicated band for WUS.

- 4. Reference

[1] RP-222644, Revised SID on lower-power WUS WUR for NR, vivo, RAN#97e

[2] R4-2303712, Reply LS to RAN1 on LP-WUR architectures, vivo, RAN4#106

[3] R4-2301104, Views on LP-WUR architectures, Samsung, RAN4#106

[4] R4-2302276, LP-WUR RF RAN4, Qualcomm, RAN4#106