3GPP TSG-RAN WG4 Meeting # 106bis-e				                     R4-2304790

Online, April 17 – April 26, 2023

Agenda Item:	5.8.3.3

Source: 	Xiaomi

Title: 	on the RLM and BFD CBD requirement

Document for:	Approval

- Introduction

During the RAN4#106 meeting discussion, the WF [1] has been agreed. The RLM and BFD CBD requirements are discussed and also open issues are captured in the discussion summary [2]. In this paper, we give further discussion on this topic.

- Discussion

Based on our parallel discussion paper on the L1-RSRP measurement delay [3] is proposed to be enhanced for SSB based L1-RSRP and for CSI-RS based L1-RSRP, it is proposed not to further enhance the measurement delay due to the different CSI-RS resource set configurations counting the repetition and number of CSI-RS resources per resource set.

For the CSI-RS based CBD, The value of TEvaluate_CBD_CSI-RS is defined with scaling factor N=8 for FR2-1 and N=12 for FR2-2. These beam sweeping factor N is the same for SSB based CBD which is also N=8 for FR2-1 and N=12 for FR2-2. In this case, the scaling factor can be similarly enhanced as SSB based L1-RSRP with capability signalling.

Observation 1: For both CSI-RS based and SSB based CBD, the scaling factor N=8 for FR2-1 and N=12 for FR2-2.

Proposal 1: For both CSI-RS based and SSB based CBD, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement.

For the CSI-RS based RLM, the value of TEvaluate_RLM_CSI-RS is defined with scaling factor equals to 1 and hence the scaling factor cannot be enhanced. For SSB-based RLM, TEvaluate_RLM_SSB is defined with scaling factor equals to 8 and the similar capability signalling as the same as SSB-based L1-RSRP measurement can be used for measurement delay enhancement.

Proposal 2: For CSI-RS based RLM, scaling factor cannot be enhanced while for SSB-based RLM, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement

For BFD, currently there is requirements defined for TRP specific BFD. This BFD is mainly focus on the inter-cell scenario to cover the cell with different PCI. As currently the multi-RX chain requirements have been agreed to be based on intra-cell m-TRP scenario, it is suggested not to enhance the TRP specific BFD in Rel-18.

Proposal 3: It is proposed not to enhance the TRP specific BFD in Rel-18.

For cell-specific BFD, TEvaluate_BFD_CSI-RS is defined with scaling factor equals to 1 and hence the scaling factor cannot be enhanced. For SSB-based BFD, TEvaluate_BFD_SSB is defined with scaling factor equals to 8 and the similar capability signalling as the same as SSB-based L1-RSRP measurement can be used for measurement delay enhancement.

Proposal 4: For cell specific BFD, for CSI-RS based BFD, scaling factor cannot be enhanced while for SSB-based BFD, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement.

3	Conclusions

In this paper, we give further discussion on RLM BFD and CBD, the observations and proposals are captured as below:

Observation 1: For both CSI-RS based and SSB based CBD, the scaling factor N=8 for FR2-1 and N=12 for FR2-2.

Proposal 1: For both CSI-RS based and SSB based CBD, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement.

Proposal 2: For CSI-RS based RLM, scaling factor cannot be enhanced while for SSB-based RLM, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement

Proposal 3: It is proposed not to enhance the TRP specific BFD in Rel-18.

Proposal 4: For cell specific BFD, for CSI-RS based BFD, scaling factor cannot be enhanced while for SSB-based BFD, the scaling factor can be enhanced with capability signalling as the same as SSB-based L1-RSRP measurement.

4 References

[1]R4-2303302		WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), VIVO

[2]R4-2302767		Topic summary for [106][209] FR2_multiRx_part1, Moderator (vivo)