3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304794

Online, April 17 – April 26, 2023

Agenda item:	4.32.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for the satellite L-/S-band

Document for:	Approval

Introduction

In RAN#98e meeting, the work item [RP-223485] on Introduction of the satellite L-/S-band was approved. Part of the objective is as follows:

-	Specify a new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz;

-	Support channel bandwidths and SCS as presented in Table 4.1-1 below;

-	Support channel raster points at step of 100kHz;

NOTE: Support for channel raster points at step of 5kHz can be considered later based on the outcome of the corresponding RAN WG4 for potential channel raster enhancements.

-	Support UE Power class 3 (+23dBm);

NOTE: Work on UE Power class 2 can commence based on the RAN#99 decision.

-	Introduce the corresponding SAN and UE RF core requirements;

-	Introduce the corresponding RRM requirements.

Table 4.1-1: Channel bandwidth and SCS system parameters.

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 |  |
| 30 |  | 10 | 15 |  |
| 60 |  | 10 | 15 |  |

- In this contribution, we want to share some initial views on UE RF requirements for Introduction of the satellite L-/S-band.

Discussion

## 2.1. Operating bands

The following agreement has been approved to reserve band n254 for WI NR_NTN_Lsband.

Agreement: - n254 is reserved for the band defined for WI NR_NTN_LSband

According to the regulation of band number that the NTN satellite bands are numbered in descending order from n256, the reserved band number n254 should be applied to the new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz.

Table 2.1-1: NTN satellite bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n254 | 1610 MHz-1626.5MHz | 2483.5MHz-2500MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Proposal 1: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

## 2.2 The type of satellites applied to n254

According to the objective of WI NR_NTN_Lsband, the new band is not limited for GEO. There is no technical reasons to indicate that the new band is not suitable for LEO, so the new band should not be limited for GEO, especially in early research stage.

Proposal 2: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should not be limited for GEO.

## 2.3 UE channel bandwidth per operating band

The wider bandwidth can be used to schedule multiple users and the feasibility for wider bandwidth will not be affected. Therefore, according to the WID objective, the supported channel bandwidths and SCS of band n254 can be defined as Table 2.2-1.

Table 2.2-1: Channel bandwidths for each NTN satellite band

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 |  |
| n254 | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the UE channel bandwidth per operating band should be defined as Table 2.2-1.

### 2.3.1 Asymmetric channel bandwidths

The following proposal was listed in Topic summary[2]:

## Asymmetric channel bandwidth combinations: - Option 1: Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band. - Option 2: (none at this meeting)

The introduction of asymmetric channel bandwidth combinations can reduce the complexity of UE implementation. We are open to this feature and this feature can be further studied.

Proposal 4: The introduction of asymmetric channel bandwidth combinations can be further studied.

## 2.4 Channel raster entries for each operating band

According to the WID objective, the granularity of the channel raster is 100kHz and the granularity of the global frequency raster is 5kHz. Therefore, the NR-ARFCN can be defined as Table 2.3-1.

Table 2.3-1: Applicable NR-ARFCN per operating band

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |

Proposal 5: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the NR-ARFCN should be defined as Table 2.3-1.

## 2.5 Synchronization raster entries for each operating band

The detailed spectrum status between NTN band and surrounding TN bands are shown as following Figure 2.1-1.

Figure 2.1-1 NTN band and surrounding TN bands

Based on the above adjacent spectrum for NTN band, the applicable SS Block SCS and SS Block pattern of TN band can be reused. For example, the applicable SS Block SCS and SS Block pattern of band n24 can be reused to n255. Similarly, the applicable SS Block SCS and SS Block pattern of band n41 can be reused to n254. The applicable SS Block SCS and SS Block pattern of TN band n24/41/66 and NTN band n256/255 are listed in Table 2.4-1 and Table 2.4-2.

Table 2.4-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |

Table 2.4-2: Applicable SS raster entries per operating band

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Therefore, the synchronization raster for operating bands n254 is specified in Table2.4-3.

Table 2.4-3: Applicable SS raster entries per operating band

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Proposal 6: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster should be defined as Table 2.4-1.

## 2.6 TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands n254 is specified in Table 2.5-1.

Table 2.5-1: UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n254 | 873.5 MHz |

Proposal 7: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation should be defined as Table 2.5-1.

## 2.7 UE maximum output power

According to the WID objective, the new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz supports UE Power class 3 (+23dBm).

Table 2.6-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

Proposal 8: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the UE maximum output power should be defined as Table 2.6-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

Proposal 2: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should not be limited for GEO.

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the UE channel bandwidth per operating band should be defined as Table 2.2-1.

Proposal 4: The introduction of asymmetric channel bandwidth combinations can be further studied.

Proposal 5: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the NR-ARFCN should be defined as Table 2.3-1.

Proposal 6: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster should be defined as Table 2.4-1.

Proposal 7: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation should be defined as Table 2.5-1.

Proposal 8: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the UE maximum output power should be defined as Table 2.6-1.

References

- RP-223485, “New WID on Introduction of the satellite L-/S-ban”, Globalstar, Apple.

- R4-2302814, Topic summary for [106][121] NR_NTN_LSband, Apple