3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304806

# Online, April 17 – April 26, 2023

Source: 	vivo

Title: 	Discussion on L3 aspects of FR2 SCell activation enhancements

Agenda Item:	5.9.2.1

Document for:	Discussion

Introduction

In WID[1], the scope of ‘FR2 SCell activation delay reduction’ is provided in [1]

- FR2 SCell activation delay reduction

  - Identify cases where FR2 SCell activation delay can be reduced (e.g., unknown target cell cases), and specify reduced delay requirements for such cases, including but not limited to [RAN4]

    - Study and, if feasible, enhance cell detection for unknown SCell and time/frequency tracking

    - Study and, if feasible, enhance L1-RSRP measurement delay reduction on target SCell

    - Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable.

  - Specify if needed, reference signal enhancement and/or signaling enhancement for the UE to meet the enhanced delay requirements [RAN4, RAN2]

    - Note: No RAN1 work, i.e. introducing new RS, is expected.

  - Note: the technical solutions can be extended to FR1, when applicable

  - Note: RAN1/RAN2 work can be triggered by RAN4 LS only

As discussed in RAN4 #105 and RAN4 #106, the WF [2], [3] are approved.

Based on all above information, we provide our views on L3 aspects of SCell activation enhancements.

Discussion

<On how to configure/trigger/report L3 measurement results for unknown SCell>

In last meeting, the following issues are discussed and agreements are achieved.

Issue 1-1-3: FFS on how to configure/trigger/report L3 measurement results for unknown FR2 SCell activation enhancement

- Suggestion for next meeting discussion:

  - This is the first meeting to discuss this issue and proposals are quite diverse. Please focus the discussion on following aspects:

    - How and when to configure L3 measurement report: via RRC in SCell addition or other? Any difference between periodic or aperiodic reporting configuration?

    - How to trigger L3 measurement report: by SCell activation command or by other MAC-CE independent from SCell activation command?

    - How to report L3 measurement result: by L3 RRC or L2 MAC CE or L1 UCI? How to get the UL grant for report via L2 MAC CE or L3 RRC?

    - Whether needs LS to check with RAN1/2?

Deactivated SCell measurement can be regarded as one intra-frequency layer and the CSSF factor is calculated based on the assumption of sharing one searcher for all SCC(s) and PSCC. In legacy L3 measurement procedure, UE may have detected SCell, but no L3 measurement report has been sent. Based on current RAN4 discussion in efeRRM WI, the target scenario for enhancement on SCell activation delay reduction, is the scenario that

- A L3 MO of the deactivated SCell frequency layer is configured to UE, but no L3 measurement report is sent by UE on this layer before gNB wants to trigger SCell activation.

Therefore, generally, the scenario can be categorized into following 3 sub-scenarios.

Scenario-1:  UE cannot detect this cell before the SCell activation MAC-CE is received, because the Es/Iot of the SCell SSBs does meet the corresponding detectable condition.

Scenario-2:  UE has detected this cell within a predefined period before the SCell activation MAC-CE is received, but the cell quality of the SCell does not meet the criteria for event-triggered L3 MR reporting.

Scenario-3:  UE has detected this cell, and at that time the cell quality of the SCell does not meet the criteria for event-triggered L3 MR reporting, but UE has not performed the L3 cell measurement/identification for the frequency layer within a predefined period before the SCell activation MAC-CE is received.

In legacy known/unknown classification, all above 3 scenarios are regarded as unknown, and corresponding SCell activation requirements for these 3 scenarios are the same. Note that there could be no solution in R18 for Scenario-1. In R18 enhancement for SCell activation delay reduction, at least to solve the issue of Scenario-2, the target unknown scenario can be either at the time when setting up RRC connection for the corresponding cell group (i.e. direct SCell activation), or when the SpCell has been in RRC connected state for a quite long period and UE has performed L3 measurement on the MO of the SCC. Therefore, the cell detection of target SCell can be performed either in idle mode (i.e. EMR) or in connected mode. To ensure that UE is aware of potentially-needed early reporting of the measurement, and allow NW to trigger the corresponding reporting whenever it is needed, it is suggested to configure the additional early reporting association in either measConfig or measIdleConfigSIB. For example, if the additional early reporting association is configured in measIdleConfigSIB, UE would be able to continue the measurement on the detected cell of the SCC at the exact time when the SpCell enters the RRC connected mode (which is discussed and agreed in R18 feMob WI) and allows NW to trigger the corresponding measurement reporting when the measurement result is ready. Note that this reporting can be even before the MeasConfig, which includes the corresponding MO of the SCC, is provided.

Proposal 1  To enable enhancement to direct SCell activation, and to allow NW to trigger the reporting whenever it is needed, e.g. even before the SCell addition, it is suggested to configure the additional early L3 measurement report association in either measConfig or measIdleConfigSIB.

In the R15 SCell known condition, it is shown that the TCI state can be configured based on L3 MR. If the FR2 SCell is known, the delay components do not include L3 measurement procedure.

TS 38.133 v18.0.0:

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Firstly, SCell activation is normally L1L2 procedure. In our understanding, in legacy known SCell activation, the gNB L2 determines whether to activate the SCell based on scheduler decision inside itself, and the availability of SCell, including the SSB index information to configure TCI state, is forwarded to it by gNB L3 after gNB L3 receives MR from UE. For R18 SCell activation procedure, the similar reporting can be re-used.

Proposal 2  In R18 enhancement, the early reporting of measurement results can be carried by L3 signaling, e.g., L3 MR or EMR reporting.

On the other hand, to enhance the case of unknown SCell activation, which means gNB L3 has not received MR from the UE, the reporting does not need to be sent to gNB L3, but can directly send to gNB L2. Note that there is already some similar discussion and agreement in LTM discussion happened in feMob WI. In our understanding, the conclusion there can also be re-used here. Therefore, for the enhancement, the measurement result based on rough sync and rough Rx beam, should be reporting in L1/L2 signaling, e.g. in uplink UCI.

Moreover, as discussed above, we do not think the L1L2 reporting of measurement results can only be triggered after FR2 SCell activation command is sent. As discussed above, without the SSB index information that forwarded by gNB L3, gNB L2 may also try to obtain the TCI related information by itself before the SCell is activated. Note that this is also similar to the discussion happened in R18 feMob WI.

If the reporting of the results is carried by uplink L1L2 signaling, then following common procedure of L1L2 reporting, if the reporting occasion is available, but there is no valid measurement result, UE may report -Inf value in the reporting to network. For example, if the L3 measurement reporting mapping specified in 10.1.6 of TS 38.133 is re-used, UE reports ‘RSRP_0’, i.e. RSRP<-156dBm.

Proposal 3  To speed up FR2 SCell activation, after decoding of the MAC-CE-based SCell activation command, the reporting of the results obtained by rough-Rx-beam-based measurement can also be carried by L1/L2 signaling.

Proposal 4  To allow gNB to obtain measurement results before the decision of SCell activation, the triggering of measurement result reporting can also be a separate MAC CE command different from SCell activation command.

Proposal 5  If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’.

Regarding the uplink grant needed for the reporting, in our understanding it can follow legacy procedures. As long as UE has UL data, UE may send BSR/SR/RACH to ask for the UL grant. There is no strong motivation for any enhancement for this part. However, a timer can be introduced by RAN2, supervising the longest period between gNB triggering the reporting and the actual result is sent by the UE in PUSCH. If either grant is not provided by gNB or UE is not able to form the reporting content before the timer expires, the early reporting is considered un-successful, and the legacy unknown requirements are taken as the fall back solution.

Proposal 6  Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires.

Therefore, how to enhance requirements of SCell activation delay depends on the definition of ‘valid’ in the agreements. RAN4 needs to clarify whether the ‘valid measurement result’ is the measurement result obtained when measurable condition specified in TS 38.133 is met or meeting some specific criterion based on network configuration, similar to the ‘event’ defined for legacy L3 reporting. In our view the latter one is preferred, as the Es/Iot for SCell activation is >-2dB, which is different from measurable condition of L3 measurement. It would be better if network may have flexibility in determine the criteria for determining the validity of the result, and the configuration criteria for determining validity of the results should be different from L3 MR reporting criteria.

Observation 1  Legacy RRM requirements for unknown SCell activation delay are only applicable to the case of Es/Iot > -2dB, which are different from the side condition for L3 measurement.

Proposal 7  The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network.

Another issue is how to obtain such measurement results at UE side, especially for Scenario-3 above. In our view, UE should prioritize the measurement on the corresponding frequency layer after the SCell activation command is received, so as to get a result within the configured maximum delta-time. In this case, the searcher of the UE is used for the prioritized measurement, hence no L3 RRM requirements would be applicable during the period before UE reporting the corresponding measurement result that triggered by SCell activation command.

Proposal 8  If a UE is configured to report measurement results after the SCell activation command is received, the measurement requirements specified in TS 38.133 clause 9.2 for all SCCs and PSCC are not applicable until UE finishes the corresponding measurement reporting.

In case the valid measurement result is reported by L1L2 signaling before SCell activation command is sent, the requirement for SCell activation may follow the known case, i.e. network can perform TCI activation based on the L1L2 reporting, and L3/L1 parts can be skipped. This state can be defined as either sub-state of known or unknown, i.e. either from L3 perspective or from L1/L2 perspective.

Proposal 9  If valid L3 measurement results are reported, L3 and L1 parts can be skipped.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  To enable enhancement to direct SCell activation, and to allow NW to trigger the reporting whenever it is needed, e.g. even before the SCell addition, it is suggested to configure the additional early L3 measurement report association in either measConfig or measIdleConfigSIB.

Proposal 2  In R18 enhancement, the early reporting of measurement results can be carried by L3 signaling, e.g., L3 MR or EMR reporting.

Proposal 3  To speed up FR2 SCell activation, after decoding of the MAC-CE-based SCell activation command, the reporting of the results obtained by rough-Rx-beam-based measurement can also be carried by L1/L2 signaling.

Proposal 4  To allow gNB to obtain measurement results before the decision of SCell activation, the triggering of measurement result reporting can also be a separate MAC CE command different from SCell activation command.

Proposal 5  If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’.

Proposal 6  Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires.

Observation 1  Legacy RRM requirements for unknown SCell activation delay are only applicable to the case of Es/Iot > -2dB, which are different from the side condition for L3 measurement.

Proposal 7  The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network.

Proposal 8  If a UE is configured to report measurement results after the SCell activation command is received, the measurement requirements specified in TS 38.133 clause 9.2 for all SCCs and PSCC are not applicable until UE finishes the corresponding measurement reporting.

Proposal 9  If valid L3 measurement results are reported, L3 and L1 parts can be skipped.

- References

[1]  RP-220977 	New WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, OPPO.  RAN #95-e

[2]  R4-2220440  WF on R18 eFeRRM - FR2 SCell activation enhancement, Apple.  RAN4 105

[3]  R4-2303228  WF on R18 RRM enhancement (part 1), Apple.  RAN4 106