3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304812

# Online, April 17 – April 26, 2023

Title:	[DRAFT] LS to RAN2 on the measurement framework in R18 LTM

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 Yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #106-bis-e meetings, RAN4 have further discussed the measurement framework for R18 LTM. Based on discussion RAN4 have reached the following agreements.

[Note: Highlighted parts will be updated based on agreements in RAN4 106-bis-e]

In R18, RAN4 agree to introduce a new type of L1 measurement on top of existing measurements:

- Existing L3 measurement, while the reporting container is L3 MR.

- Existing Type-1 L1 measurement (for beam management), while the reporting container is UCI.

- New Type-2 L1 measurement (for mobility), while the UE measurement behaviour is the same as L3 measurement without L3 filtering, and new reporting mechanism are to be introduced by RAN2/RAN1.

The following summary regarding the above 3 types of measurement is provided for information.

Table 1  Summary of inter-cell measurements considered in R18 LTM

|  | L3 measurement | Type-2 L1 measurement | Type-1 L1 measurement |
| --- | --- | --- | --- |
| Supported RS | SSB, CSI-RS for mobility | SSB | SSB, NZP CSI-RS, IMR |
| UE Rx beam assumption Note 1 | Rough Rx beam, beam sweeping is always needed. | Rough Rx beam, beam sweeping is always needed. | Fine Rx beam, beam sweeping is not needed for CSI-RS based L1 measurements |
| Configuration container | MeasConfig | FFS in RAN2/RAN1 | CSI-MeasConfig |
| Measurement periodicity | SMTC periodicity | SMTC periodicity | SSB periodicity |
| Reporting container | L3 MR | UCI or MAC CE (Details FFS in RAN1/RAN2) | UCI |
| L3 filtering | Needed | Not needed | Not needed. |
| Intra-frequency measurements outside active BWP | Scenarios are valid. - In some scenarios (e.g. CD-SSB is outside the active BWP of the SpCell), the measurement is performed within gaps, - In some scenarios, (e.g. deactivated SCell measurement), the measurement is performed without gaps. | Scenarios are valid. - In some scenarios (e.g. CD-SSB is outside the active BWP of the SpCell), the measurement is performed within gaps, - In some scenarios, (e.g. deactivated SCell measurement), the measurement is performed without gaps. | Scenarios are not valid |
| Inter-frequency measurement | Scenarios are valid | Scenarios are valid | Not needed. |
| DL synchronization before measurement | Not needed | Not needed | Needed |
| Neighbor cell detection | Performed parallel with the measurement | Performed parallel with the measurement | Not needed |
| Support of RTD > CP between different cells in the same frequency layer | No need for UE capability | No need for UE capability | UE capability is needed. |
| Purpose | HO/ PSCell_addition/ PSCell_change/ SCell_addition/ SCell_activation | LTM cell switch/ Indicating start of fine-Rx-beam-based L1 measurement on a given cell | Beam management |
| Note 1: Only applicable to FR2. |  |  |  |

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take above conclusions in consideration, specify the configuration and reporting mechanism for the new type of measurement agreed by RAN4, and provide feedback to RAN4 if any.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #107	May 22 – May 26	Incheon, Korea

TSG-RAN4 Meeting #108	Aug. 21 – Aug. 25	Toulouse, France