3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304813

# Online, April 17 – April 26, 2023

Source: 	vivo

Title: 	Discussion on other RRM impacts in R18 NR MIMO evolution WI

Agenda Item:	5.30.3.1

Document for:	Discussion

Introduction

In RAN4 #106, RAN4 has discussed the RRM impacts of R18 MIMO evolution WI. The status of the discussion is captured in [1].

Based on [1], we provide our views on the aspects besides 2-TAs and Unified TCI enhancement.

Discussion

<On TDCP related RRM impacts>

In the WID [2], the following objective is included.

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

For this objective, the following agreements have been made in RAN1 #112.

Agreement in RAN1 #112

For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows:

- Basic feature: Y=1 with delay ≤ Dbasic symbols, only wideband quantized normalized amplitude is reported

  - FFS: Candidate values for delay

- Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported

  - For Y>1, the phase can be configured to be absent for all the Y delays

  - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource

  - TBD: Candidate value(s) for Y>1

- FFS: Value of Dbasic

RAN1 has also agreed that the TDCP reporting only supports aperiodic reporting that triggered by gNB.

Based on above RAN1 conclusions, TDCP reporting will be a new L1 measurement quantity, similar to current L1-RSRP/L1-SINR reporting. To ensure the quality of the report in the deployment, it would be reasonable to introduce UE measurement delay and accuracy requirements for R18 TDCP reporting.

Proposal 1  Define L1-TDCP measurement delay and accuracy requirements in R18 MIMO evolution WI.

RAN1 has performed link level analysis regarding the TDCP reporting. The confidence level of TDCP reporting, and the UE behaviour when the quality of TDCP measurement is not sufficiently high, are still being discussed in RAN1. In the early evaluation for TDCP reporting in RAN1, RMS error, Std Deviation/Bias are used as the metric for TDCP reporting quality. For UE supporting the basic feature, the reporting quantity would be the normalized amplitude of the time domain profile for a single delay. Note that the delay here is not the delay-tap in channel model, but the symbol-level ‘gap’ between the referenced TRS symbols for TDCP calculation. Then comes to the question for RAN4 on the testability issue, e.g. how to set-up/calibrate the channel model for the testing of TDCP accuracy and how to reduce the uncertainty of testing. Moreover, RAN4 may also need to further discuss the reasonable UE implementation margin for the requirement, taking a reasonable UE behaviour assumption.

Proposal 2  RAN4 further discuss the following issues regarding the L1-TDCP delay and accuracy requirements:

- The metric for TDCP measurement accuracy

- The testability issues, e.g. channel model calibration and test uncertainty

- Basic UE behaviour assumption that may impact the delay/accuracy requirements

<On SRS enhancement>

In the WID [2], the following objectives are included.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

For the SRS capacity enhancement and SRS interference randomization targeting TDD CJT scenario, we do not see the impact to the RRM requirements.

For the 8Tx SRS enhancement, in R16/R17 RAN4 has studied and specified the interruption requirements for SRS antenna switching, and SRS carrier switching. In those studies, at least from our understanding, are targeting handheld UE. For example, in the discussion of interruptions for SRS antenna switching, the discussion focuses on the interruptions on the bands signalled in txSwitchImpactToRx or txSwitchWithAnotherBand. In our understanding these signallings are introduced for handheld UE in FR1 only. For the CPE/FWA/vehicle UE types, in our understanding there is no need to report impacted DL/UL band in CA/DC combination, since in most cases it is unnecessary to re-use the RF chains across bands. Normally only handheld UEs need to re-use RF chains across bands so as to solve the form factor and cost issues. Based on this understanding, we do not think the R17 interruption requirements need to be extended to the 8 Tx cases.

Proposal 3  In R18 MIMO WI, not to consider extending R17 SRS AS interruption requirements to cover the new scenarios of 8Tx related SRS enhancement, due to the fact that they are not targeted for handheld UE.

<On Enhanced Uplink Transmission>

In last meeting, the following is discussed and captured in [1].

Issue 3-4-1: How to consider RRM requirements by simultaneous UL transmission with multi-panels?

- Proposals

  - P1: The enhancement on simultaneous UL transmission with multi-panel need to be studied based on the enhancement on simultaneous DL receptions with multi-panel. (Huawei)

  - P2: Suggest to discuss multi-TX panel related requirement in future release. (Intel)

  - P3: The unified TCI state switching requirement will be impacted to extent the multi-TRP case and the STxMP feature. (Xiaomi)

  - P4: FFS on whether to specify the RRM requirements for maximum timing difference for multi-panel transmission. (Samsung)

  - P5: No RRM impact. (Apple, Ericsson)

In our view, P1 and P2 are supported. We do not yet see the need to discuss simultaneous UL transmission if simultaneous DL transmission based on more than one panel is still being discussed.

Proposal 4  Suggest to discuss and/or capture the spec impact due to simultaneous UL transmission in future release, because the enhancement on simultaneous DL receptions with multi-panel is under discussion in R18.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  Define L1-TDCP measurement delay and accuracy requirements in R18 MIMO evolution WI.

Proposal 2  RAN4 further discuss the following issues regarding the L1-TDCP delay and accuracy requirements:

- The metric for TDCP measurement accuracy

- The testability issues, e.g. channel model calibration and test uncertainty

- Basic UE behaviour assumption that may impact the delay/accuracy requirements

Proposal 3  In R18 MIMO WI, not to consider extending R17 SRS AS interruption requirements to cover the new scenarios of 8Tx related SRS enhancement, due to the fact that they are not targeted for handheld UE.

Proposal 4  Suggest to discuss and/or capture the spec impact due to simultaneous UL transmission in future release, because the enhancement on simultaneous DL receptions with multi-panel is under discussion in R18.

- References

[1]  R4-2303258	WF on R18 NR MIMO RRM requirements, Samsung.  RAN4 #106

[2]  RP-223276	WID Update: MIMO Evolution for Downlink and Uplink, Samsung.  RAN #98-e