3GPP TSG RAN WG4 Meeting #106-bis-e		 R4-2304871

# 17 April - 26 April 2023, e-Meeting

Agenda Item:	x.x.x

Source: 	MediaTek Inc.

Title: 	Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41

Document for:	Discussion

1. Introduction

In RAN4 #106, simultaneous Rx/Tx operation for CA_n40-n41 was extensively discussed and WF[1] had been agreed. The contribution provides continue discussion and calculation on the requirements for simultaneous Rx/Tx operation for CA_n40-n41

2. Discussion

The CA_n40-n41 has been discussed in last RAN4 meeting. It was found there may be MSD due to cross band isolation if simultaneous Rx/Tx operation is required. One of the WI[1] scope is to harmonize the requirements for NR CA, EN-DC and NR SUL corresponding combos. We provide further discussion on MSD for CA_n40-n41. We re-calculate MSD on n40/n41 with the components data in our vendor pool and calculation assumption listed below

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| PA ACP2 | -50 | dBc |
| Front-end loss | 4 | dB |
| n41 filter rejection on n40 range | 30 | dB |
| n40 filter rejection on n41 range | 36 | dB |
| PA RXBN noise | -117 | dBm/Hz |
| Thermal noise at n41 RX ANT port | -165.1 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

In WF[1] test conditions has been agreed and copied below:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [FFS] | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [FFS] | ACLR2 |

- The MSD is evaluated for both PC3 and PC2.

For uplink TX in n40, since the BW is 100MHz and the lower frequency edge of DL victim BW of n41 starts from 2496MHz, there’s 1.5MHz overlap with ACP1 of n40 TX. For uplink TX in n41, since the BW is 100MHz and the higher frequency edge of DL victim BW of n40 starts from 2400MHz, there’s 1.5MHz overlap with ACP1 of n41 TX. The MSD test point are re-calculated as table below:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 35 | ACLR2 |

Table 2. Proposed MSD due to cross band isolation on n41

Proposal 1: MSD due to cross band isolation of CA_n40-n41 when enabling simultaneous Rx/Tx operation is proposed as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 35 | ACLR2 |

Proposal 2: To harmonize requirements as instructed in the WID, the MSD of SUL_n41-n97 is proposed to align CA_n40-n41 as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |

As for relaxation on the UE capability for n41 with configured simultaneous Rx/Tx, three options were left in last RAN4 meeting:

Option 1(Apple):

To reduce the antenna implementation challenge: Relax the 4Rx requirement for n41 for handhelds if simultaneous Rx/Tx carrier aggregation is configured with band n40.

- The relaxation does not apply for FWA UE.

- The relaxation does not apply to single band n41 configuration and for combinations with bands other than n40.

Option 2(CMCC):

Opening the door to more flexibility to implement 4Rx is not expected.

Option 3(MediaTek):

For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

It is usually required for one antenna to support multiple band frequency range. However, due to limited space for antenna design, there’s trade off that not all antennas can support full frequency range of all supported bands. In general, there’s less margin on transmit TRP requirement then receiving TIS requirements. There may also be some other features such as UL MIMO/TxD, SRS antenna switching and Tx switching need to be supported simultaneously. In our initial estimation, we found it is difficult to have all antennas to meet design requirements on both transmission and receiving of all the supporting bands and MRDC/NRDC combinations fulfil multiple UE capability at the same time.

Proposal 3: For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

3. Conclusion

Proposal 1: MSD due to cross band isolation of CA_n40-n41 when enabling simultaneous Rx/Tx operation is proposed as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 35 | ACLR2 |

Proposal 2: To harmonize requirements as instructed in the WID, the MSD of SUL_n41-n97 is proposed to align CA_n40-n41 as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |

Proposal 3: For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

Reference:

- R4-2303691 WF on simultaneous Rx/Tx transmission

- RP-223354 WID revision: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 RAN-P#98-e