3GPP TSG-RAN WG4 Meeting#106-bis-e						R4-2304881

17th Apr, 2023 – 26th Apr, 2023

Source: 	Qualcomm Incorporated

Title: 	NTN NB-IoT/eMTC demodulation performance requirements

Agenda item:	 6.8.7.1

Document for:	Discussion

- Introduction

In this paper, the demodulation performance requirements in NTN mode of both NBIoT and eMTC are discussed based on discussion in [1].

- NBIoT NTN Scenarios

Scenarios in the following table were considered in [3]. All numbers provided here are with 0.1 ppm maximum residual frequency error.

Table 2.1

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | -8.7 |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | 8ms | -12 |

Proposal 1: NBIoT: Propose SNR numbers in the Table 2.1 for alignment purpose

Proposal 2: For NBIoT test 2, propose NPDCCH repetition factor be 256

In test 2 with doppler spread as 10Hz, NPDCCH with repetition factor of 128 does not realize time diversity gain whereas every NPDSCH which is spread 128 * 4 ms = 512ms achieves diversity gain. Consequently, NPDCCH BLER performance does not have the same slope as that of NPDSCH. To ensure only NPDSCH performance is specified, we propose to have NPDCCH repetition as 256 with NPDSCH repetition as 128.

Please note that earlier agreement was to lower NPDSCH repetition to 128 from 256 in order to operate at a higher SNR regime.

- eMTC Scenarios

Proposal 3: eMTC: Propose SNR numbers in the Table 3.1 for alignment purpose

The shared numbers are from initial simulation results with 0.1 ppm maximum residual frequency error, and no drop in PDSCH when an overlap occurs with SIB.

Table 3.1

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms | 7.1 |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | -6.7 |

- Test Method

Proposal4: Propose to use AT commands and no GNSS receiver for UE location information

To decouple UE location accuracy of GNSS receiver and for ease of testing, we our of the view to program UE location via AT commands. This mode of test shall be useful for UE Tx timing requirement test cases.

- Conclusions

Following is the summary of the proposals.

Proposal 1: NBIoT: Proposed SNR numbers in Table 2.1 for alignment purpose

Proposal 2: For NBIoT test 2, propose NPDCCH repetition factor be 256

Proposal 3: eMTC: Proposed SNR numbers in Table 3.1 for alignment purpose

Proposal4: Propose to use AT commands and no GNSS receiver for UE location information

# References

[1] R4-2302932, “WF for IoT over NTN UE demodulation requirements”, #106, Feb 2023.

[2] 3GPP WID “New WID on NB-IoT/eMTC core & performance requirements for NTN,” 3GPP TSG RAN meeting#95e, online, March 17 – 23, 2022.

[3] 3GPP TS 36.101 version 17.6.0 “LTE; E-UTRA User Equipment (UE) radio transmission and reception”.