3GPP TSG-RAN WG4 Meeting #106bis-e                                           	R4-2304899

Online, April 17 – April 26, 2023

Source:	Samsung

Title:	discussion on 8Rx PDSCH requirements

Agenda item:	5.5.4.1.1

Document for:	Discussion

- Introduction

In last RAN4 #106 meeting, general test scope, simulation assumptions for FR1 8Rx PDSCH requirements have some agreements in WF [1]. In this contribution, we share our views on open issues of FR1 PDSCH requirements.

Issue 2-1: MIMO layers - Rank 2, 4 and 8 Issue 2-2: Antenna configuration - Rank 2: 2T8R - Rank 4: 4T8R - Rank 8: 8T8R Issue 2-3: Antenna correlation - Rank 2: ULA Medium B - Rank 4: ULA Low - Rank 8: ULA Low Issue 2-4: Propagation conditions - Rank 2: TDLC300-100 ,TDLA30-10 - Rank 4 and 8: TDLA30-10 - FFS on consideration of beamforming model on top of TDL channel model for two MCS configuration Issue 2-5: MCS configuration - The following MCS can be used starting point; Rank 2: - up to 256QAM: [MCS26] (table 2); (Low correlation) (Only forTDLA30-10) - MCS16 (Medium B); - MCS19(Medium B) - MCS23(Low) Rank 4: - up to 64QAM: MCS26; - MCS17; Rank 8: - up to 64QAM: MCS20; - MCS13; Issue 2-6: Two MCS configuration for Rank>4 - Option 1: Configure two MCSs for Rank>4 and discuss the spatial channel model to be used to guarantee different precoding gain for each layer - Option 2: Only consider single MCS Issue 2-7: Overhead - Overhead = 0 Issue 2-8: Mapping Type - PDSCH mapping Type A - FFS PDSCH mapping Type B Issue 2-9: TRS configuration - Configure TRS symbols in symbol#{5, 9} Issue 2-10: NZP-CSI-RS configuration - NZP CSI-RS configuration of row 6 for 8 layers. Issue 2-11: Whether consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx - Option 1: Yes, also define ‘Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH’ for 8Rx UE similar as 5.2.2.1.2, 5.2.3.1.2 in 38.101-4. - Option 2: No Issue 2-12: Special slot - Not schedule PDSCH in special slot for 8 layers cases Issue 2-13: N1 and N2 configuration for 8Tx case - (N1,N2)=(4,1) Issue 2-14: Test metric - Only 70% of maximum TP Issue 2-14: Simulation assumptions - Companies are encouraged to bring simulation results based on assumptions listed in Table2-1 and 2-2 Table 2-1: Common simulation assumptions Parameter; Unit; Value. Duplex mode; TDD. TDD pattern; 7D1S2U S=6D:4G:4U. SCS(kHz)/Bandwidth (MHz); 30/40. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PRB bundling type; Static. PRB bundling size; 2. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. Number of HARQ Processes; 8. Maximum HARQ transmissions; 4. Tx EVM (Explicitly modeled in the simulation); 64QAM:6% 256QAM:3%.

- Discussion

Two MCS configuration for Rank>4

For 8T8R 8layer case, total two codewords and each codeword is mapping with 4 layers, meanwhile the PDSCH requirements only focus on the final PDSCH performance instead of each codeword performance. Thus it is no need to consider different MCS levels for two codewords.

Proposal 1: Only consider single MCS for Rank 8 case.

Whether consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx

Firstly, the main target of introducing PDSCH requirement for 8Rx is to verify 8Rx performance. There is no need to introducing 8Rx test cases for all the features. Secondly, current test cases for minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH is only up to 2 ports. It seems no need to extend the requirements for CSI-RS ports larger than 4Tx.

Proposal 2: Do not consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx.

Maximum number of OFDM symbols for DL front loaded DMRS

In last meeting, the agreement for Maximum number of OFDM symbols for DL front loaded DMRS in simulation assumption is 1. But based on table 7.4.1.1.2-5 defined in 38.211 (as below), for rank larger than 4, we need to define the maximum number of OFDM symbols for DL front loaded DMRS configured as 2.

Table 7.4.1.1.2-5: PDSCH DM-RS time index l' and antenna ports p.

| Single or double symbol DM-RS | l' | Supported antenna ports p |  |
| --- | --- | --- | --- |
|  |  | Configuration type 1 | Configuration type 2 |
| single | 0 | 1000 – 1003 | 1000 – 1005 |
| double | 0, 1 | 1000 – 1007 | 1000 – 1011 |

Proposal 3: for rank larger than 4, Maximum number of OFDM symbols for DL front loaded DMRS should be configured as 2.

Performance requirements

According to the simulation results in [2], considering the impairment margin, we propose to introduce PDSCH performance requirements for 8Rx.

Proposal 4: introduce PDSCH performance requirements for 8Rx as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 7.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 8.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 10.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.42 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 7.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 10.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 2.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 14.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 20.74 |

- Conclusion

In this contribution, we provide analysis and views on 8Rx UE demodulation and CSI requirements for remaining open issues.

Proposal 1: Only consider single MCS for Rank 8 case.

Proposal 2: Do not consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx.

Proposal 3: for rank larger than 4, Maximum number of OFDM symbols for DL front loaded DMRS should be configured as 2.

Proposal 4: introduce PDSCH performance requirements for 8Rx as

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Low | 70 | 7.97 |
| 1-2 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Low | 70 | 8.60 |
| 1-3 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 8.58 |
| 1-4 | 40 / 30 | 16QAM, 0.64 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.25 |
| 1-5 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLA30-10 | 2x8, ULA Medium B | 70 | 10.37 |
| 1-6 | 40 / 30 | 64QAM, 0.51 | FR1.30-1 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.42 |
| 2-1 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 7.12 |
| 2-2 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 4x8, ULA Low | 70 | 10.93 |
| 3-1 | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 2.91 |
| 3-2 | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 14.73 |
| 3-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | TDLA30-10 | 8x8, ULA Low | 70 | 20.74 |

- Reference

[1] R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106 meeting

[2] R4-2304900, “simulation results on 8Rx UE demodulation requirements”, Samsung, 3GPP RAN4 #106bis-e meeting