3GPP TSG-RAN WG4 Meeting #106bis-e                                           	R4-2304904

Online, April 17 – April 26, 2023

Source:	Samsung

Title:	simulation results on absolute physical layer throughput with link adaptation

Agenda item:	5.19.2

Document for:	Discussion

- Introduction

In last RAN4 #106 meeting, test scope and simulation assumptions for absolute physical layer throughput with link adaption requirement have agreements in WF [1] as below. In this contribution, we share our simulation results.

Issue 1-1-1: Whether to cover FR2-2 - Focus on FR2-1 and FR1 in Rel-18 WI. Issue 1-2-1: Maximum rank and CSI-RS port number - Same as defined in Table 5.10.3-1 in TR 37.901-5, i.e., maximum rank 2 with 2 CSI-RS ports. Issue 1-2-2: Enable of outer loop link adaptation (OLLA) - Rel-18 focus to introduce ATP performance requirements without OLLA. - It’s not precluded to reconsider and evaluate in future release. Issue 1-2-3: Channel model - Reuse channel models in TR 37.901-5. Issue 1-2-4: Maximum number of HARQ transmission - Companies are encouraged to bring evaluation results with and without retransmission - Pending on the alignment outcome of further evaluation results with retransmission, if the feasibility concluded RAN4 can update the assumption with retransmission For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; for precoder following UE reported PMI

- Simulation Assumptions

Table 2-1. Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 and 4 | 1 and 4 | 1 and 4 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

- Simulation results

  - Simulation results without retransmission

Throughput, BLER, Median Rank and Median CQI results of Test 1-3 are in below table.

Table 3-1. Simulation results of Test 1 FR1 FDD cases

| 2x2 |  |  |  |  | 2x4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI | SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.37 | 0.04 | 1 | 3 | 0 | 6.02 | 0.07 | 1 | 4 |
| 2 | 5.25 | 0.09 | 1 | 4 | 2 | 7.51 | 0.08 | 1 | 4 |
| 4 | 6.99 | 0.08 | 1 | 4 | 4 | 10.40 | 0.08 | 1 | 5 |
| 6 | 9.21 | 0.07 | 1 | 6 | 6 | 13.10 | 0.07 | 2 | 5 |
| 8 | 10.53 | 0.05 | 1 | 6 | 8 | 16.93 | 0.07 | 2 | 6 |
| 10 | 12.66 | 0.08 | 1 | 7 | 10 | 21.09 | 0.05 | 2 | 7 |
| 12 | 14.66 | 0.06 | 1 | 8 | 12 | 25.48 | 0.06 | 2 | 7 |
| 14 | 16.91 | 0.06 | 1 | 8 | 14 | 27.84 | 0.04 | 2 | 8 |
| 16 | 19.52 | 0.08 | 1 | 8 | 16 | 32.21 | 0.03 | 2 | 9 |
| 18 | 22.57 | 0.10 | 2 | 9 | 18 | 36.80 | 0.05 | 2 | 10 |
| 20 | 25.75 | 0.11 | 2 | 10 | 20 | 39.22 | 0.06 | 2 | 11 |

Table 3-2. Simulation results of Test 2 FR1 TDD cases

| 2x2 |  |  |  |  | 2x4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI | SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 10.24 | 0.07 | 1 | 3 | 0 | 21.99 | 0.06 | 1 | 5 |
| 2 | 15.56 | 0.06 | 1 | 4 | 2 | 26.41 | 0.07 | 1 | 5 |
| 4 | 22.19 | 0.08 | 1 | 5 | 4 | 33.12 | 0.08 | 2 | 5 |
| 6 | 27.96 | 0.09 | 1 | 6 | 6 | 43.70 | 0.07 | 2 | 6 |
| 8 | 33.66 | 0.07 | 1 | 7 | 8 | 55.18 | 0.05 | 2 | 6 |
| 10 | 38.23 | 0.08 | 1 | 8 | 10 | 66.83 | 0.04 | 2 | 7 |
| 12 | 42.53 | 0.07 | 1 | 9 | 12 | 80.49 | 0.03 | 2 | 8 |
| 14 | 53.93 | 0.11 | 2 | 8 | 14 | 91.68 | 0.04 | 2 | 9 |
| 16 | 63.10 | 0.11 | 2 | 8 | 16 | 99.20 | 0.05 | 2 | 10 |
| 18 | 72.20 | 0.11 | 2 | 8 | 18 | 113.50 | 0.04 | 2 | 10 |
| 20 | 78.45 | 0.11 | 2 | 9 | 20 | 124.10 | 0.09 | 2 | 11 |

Table 3-3. Simulation results of Test 3 FR2 TDD cases

| 2x2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 19.35 | 0.08 | 1 | 5 |
| 2 | 27.79 | 0.07 | 1 | 7 |
| 4 | 37.77 | 0.07 | 1 | 8 |
| 6 | 49.02 | 0.08 | 1 | 9 |
| 8 | 54.65 | 0.07 | 1 | 10 |
| 10 | 64.73 | 0.06 | 1 | 11 |
| 12 | 76.01 | 0.08 | 1 | 12 |
| 14 | 84.86 | 0.09 | 1 | 11 |
| 16 | 101.02 | 0.10 | 2 | 11 |

  - Simulation results with retransmission

Throughput, BLER, Median Rank and Median CQI results of Test 1-3 are in below table.

Table 3-4. Simulation results of Test 1 FR1 FDD cases

| 2x2 |  |  |  |  | 2x4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI | SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.40 | 0.05 | 1 | 3 | 0 | 6.30 | 0.06 | 1 | 4 |
| 2 | 5.30 | 0.08 | 1 | 4 | 2 | 7.59 | 0.07 | 1 | 4 |
| 4 | 7.05 | 0.08 | 1 | 4 | 4 | 10.80 | 0.08 | 1 | 5 |
| 6 | 9.29 | 0.06 | 1 | 6 | 6 | 13.70 | 0.08 | 2 | 5 |
| 8 | 10.71 | 0.07 | 1 | 6 | 8 | 17.01 | 0.06 | 2 | 6 |
| 10 | 12.93 | 0.07 | 1 | 7 | 10 | 21.20 | 0.07 | 2 | 7 |
| 12 | 15.19 | 0.08 | 1 | 8 | 12 | 25.53 | 0.06 | 2 | 7 |
| 14 | 17.44 | 0.07 | 1 | 8 | 14 | 27.89 | 0.08 | 2 | 8 |
| 16 | 20.27 | 0.09 | 1 | 8 | 16 | 32.63 | 0.07 | 2 | 9 |
| 18 | 23.25 | 0.09 | 2 | 9 | 18 | 37.30 | 0.08 | 2 | 10 |
| 20 | 26.80 | 0.10 | 2 | 10 | 20 | 40.35 | 0.08 | 2 | 11 |

Table 3-5. Simulation results of Test 2 FR1 TDD cases

| 2x2 |  |  |  |  | 2x4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI | SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 10.39 | 0.06 | 1 | 3 | 0 | 22.12 | 0.09 | 1 | 5 |
| 2 | 15.73 | 0.07 | 1 | 4 | 2 | 26.50 | 0.07 | 1 | 5 |
| 4 | 22.52 | 0.05 | 1 | 5 | 4 | 33.27 | 0.08 | 2 | 5 |
| 6 | 28.49 | 0.07 | 1 | 6 | 6 | 43.91 | 0.06 | 2 | 6 |
| 8 | 34.47 | 0.06 | 1 | 8 | 8 | 55.31 | 0.07 | 2 | 6 |
| 10 | 40.01 | 0.08 | 1 | 9 | 10 | 66.79 | 0.06 | 2 | 7 |
| 12 | 45.10 | 0.07 | 1 | 9 | 12 | 80.51 | 0.07 | 2 | 8 |
| 14 | 55.38 | 0.1 | 2 | 8 | 14 | 92.37 | 0.06 | 2 | 9 |
| 16 | 65.81 | 0.11 | 2 | 9 | 16 | 106.70 | 0.08 | 2 | 10 |
| 18 | 75.37 | 0.06 | 2 | 9 | 18 | 119.40 | 0.09 | 2 | 10 |
| 20 | 85.12 | 0.09 | 2 | 10 | 20 | 132.93 | 0.07 | 2 | 11 |

Table 3-6. Simulation results of Test 3 FR2 TDD cases

| 2x2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| SNR(dB) | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 19.38 | 0.09 | 1 | 5 |
| 2 | 27.80 | 0.08 | 1 | 7 |
| 4 | 37.85 | 0.08 | 1 | 8 |
| 6 | 49.09 | 0.09 | 1 | 9 |
| 8 | 54.60 | 0.08 | 1 | 10 |
| 10 | 64.87 | 0.1 | 1 | 11 |
| 12 | 76.22 | 0.09 | 1 | 12 |
| 14 | 85.22 | 0.1 | 1 | 11 |
| 16 | 101.37 | 0.11 | 2 | 11 |

- Conclusion

According simulation results in section 3, we could get the simulation results of SNR vs % of max throughput as below table 4-1. And we also provide our impairment results.

Table 4-1. Simulation results of SNR vs % of max throughput [dB] without retransmission

|  | FR1 FDD |  |  |  | FR1 TDD |  |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  |
| % of Max throughput | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment |
| 10 | 3.4 | 5.4 | 0.6 | 2.6 | 3.6 | 5.6 |  |  | 1.2 | 3.2 |
| 15 | 6.7 | 8.7 | 3.5 | 5.5 | 7.1 | 9.1 | 3.4 | 5.4 | 3.8 | 5.8 |
| 20 | 10.2 | 12.2 | 5.9 | 7.9 | 11.5 | 13.5 | 5.6 | 7.6 | 5.9 | 7.9 |
| 25 | 13.3 | 15.3 | 7.6 | 9.6 | 13.6 | 15.6 | 7.4 | 9.4 | 9.2 | 11.2 |
| 30 | 15.9 | 17.9 | 9.2 | 11.2 | 15.8 | 17.8 | 9.2 | 11.2 | 11.5 | 13.5 |
| 35 | 18.0 | 20.0 | 10.7 | 12.7 | 18.1 | 20.1 | 10.8 | 12.8 | 14.0 | 16.0 |
| 40 |  |  | 12.3 | 14.3 |  |  | 12.4 | 14.4 | 15.6 | 17.6 |
| 45 |  |  | 14.6 | 16.6 |  |  | 14.4 | 16.4 |  |  |
| 50 |  |  | 16.0 | 18.0 |  |  | 16.6 | 18.6 |  |  |
| 55 |  |  | 17.4 | 19.4 |  |  | 18.1 | 20.1 |  |  |
| 60 |  |  | 19.6 | 21.6 |  |  |  |  |  |  |

Table 4-2. Simulation results of SNR vs % of max throughput [dB] with retransmission

|  | FR1 FDD |  |  |  | FR1 TDD |  |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  |
| % of Max throughput | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment |
| 10 | 3.3 | 5.3 | 0.2 | 2.2 | 3.5 | 5.5 |  |  | 1.2 | 3.2 |
| 15 | 6.6 | 8.6 | 3.3 | 5.3 | 6.9 | 8.9 | 3.4 | 5.4 | 3.7 | 5.7 |
| 20 | 10.0 | 12.0 | 5.5 | 7.5 | 10.6 | 12.6 | 5.5 | 7.5 | 5.9 | 7.9 |
| 25 | 12.8 | 14.8 | 7.5 | 9.5 | 13.3 | 15.3 | 7.4 | 9.4 | 9.2 | 11.2 |
| 30 | 15.4 | 17.4 | 9.1 | 11.1 | 15.3 | 17.3 | 9.2 | 11.2 | 11.4 | 13.4 |
| 35 | 17.6 | 19.6 | 10.6 | 12.6 | 17.4 | 19.4 | 10.8 | 12.8 | 14.0 | 16.0 |
| 40 | 19.4 | 21.4 | 12.2 | 14.2 | 19.6 | 21.6 | 12.4 | 14.4 | 15.5 | 17.5 |
| 45 |  |  | 14.5 | 16.5 |  |  | 14.1 | 16.1 |  |  |
| 50 |  |  | 15.8 | 17.8 |  |  | 15.6 | 17.6 |  |  |
| 55 |  |  | 17.2 | 19.2 |  |  | 17.2 | 19.2 |  |  |
| 60 |  |  | 18.9 | 20.9 |  |  | 18.7 | 20.7 |  |  |

Reference

[1] R4-2302930, “WF on absolute physical layer throughput requirements”, Intel Corporation, 3GPP RAN4 #106 meeting