3GPP TSG-RAN WG4 # 106bis-e	R4-2304915

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.5.4.1.2

Source:	MediaTek Inc.

Title:	Discussion on SDR requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106 meeting, a WF related to 8Rx UE SDR requirements was agreed [1]. In this contribution, we provide simulation results based on [1] and propose suitable MCS to define SDR requirements.

- Discussion

In RAN4#106, RAN4 agreed to configure additional DMRS symbol for 8Rx UE SDR requirements.

Issue 3-1: Whether to configure additional DMRS symbol - Yes Issue 3-2: Feasibility of 64QAM - It is feasible to define SDR test for 8 MIMO layer for 64QAM - Companies can bring the simulation results in next meeting for the maximum achievable MCS Issue 3-3: Feasibility of 256QAM - Evaluate SDR test cases for 2, 4 and 8 MIMO layers for 8Rx; FFS 6 MIMO layers - 2 and 4 MIMO layers: Use the existing MCS value defined in Table 5.5A-5 of TS 38.101-4 - 8 MIMO layers: FFS the max achievable MCS Issue 3-4: Feasibility of 1024QAM - FFS whether it is feasible to consider 1024QAM - If it is feasible to introduce requirements for 1024QAM - FFS the maximum achievable MIMO layers: Rank 2 or Rank 4 - FFS the maximum achievable MCS

Considering Tx EVM, the maximum achievable SNR for 64QAM, 256QAM are 24.4dB and 30.4dB. Also, we need to add some margin for RF and other receiver impairments. Therefore, based on simulation results in Table 1, we propose to define SDR requirements for 8Rx UE with the following MCS.

Proposal 1: Define SDR requirements for 8Rx UE with the following MCS

- For 64 QAM: MCS 25

- For 256 QAM: MCS 24

As we already agreed to define 8 layers SDR requirements, it is not necessary to define 6 layers SDR requirements for 8Rx UE.

Proposal 2: Do not introduce 6 layers SDR requirements for 8Rx UE.

Table 1. SNR point for 85% maximum throughput

| Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- |
| 8 | 8 | 27 | 28.7 |
|  |  | 26 | 27.7 |
|  |  | 25 | 26.7 |
|  |  | 24 | 25.8 |
|  |  | 23 | 24.8 |
|  | 6 | 27 | 21.7 |
|  |  | 26 | 20.7 |
|  |  | 25 | 19.7 |
|  |  | 24 | 18.6 |
| Note 1: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- Conclusion

In this contribution, we provide simulation results for SDR based on agreed simulation assumption in the last meeting.

Proposal 1: Define SDR requirements for 8Rx UE with the following MCS

- For 64 QAM: MCS 25

- For 256 QAM: MCS 24

Proposal 2: Do not introduce 6 layers SDR requirements for 8Rx UE.

- Reference

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon

- Appendix

Table 1: Simulation assumptions SDR requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1005} for 6 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |