3GPP TSG-RAN WG4 Meeting # 106bis	R4-2304923

Electronic Meeting, April 17 – April 26, 2023

Agenda Item: 5.25.2.1

Source: MediaTek Inc.

Title: Discussion on general aspects and scenarios of L1/L2 mobility

Document for: Discussion

- Introduction

RAN4#106 had some discussion on the general issues or R18L1/L2 mobility [1]. We will keep discussing related open issues in this Tdoc.

- Discussion

2.1 Definition of inter-frequency cell switch

In last meeting, RAN4 discussed the definition of inter-frequency cell switch as below.

Issue 2-2-1: Definition of inter-frequency cell switch < Wayforward >: FFS the following options - Option 1 (Apple, CTC, CATT, MTK, DOCOMO, OPPO, vivo, Huawei, Nokia, Ericsson): Inter-frequency cell switch is defined where the SSB of Pcell and/or PScell and the candidate target cell are on different frequency layers. - Option 2 (CATT, DOCOMO, CMCC, vivo): Inter-frequency cell switch is defined where the SSBs of active serving cell(s) and the corresponding candidate target cell(s) are on different frequency layers - Option 3 (CMCC): no need to have the definition of inter-frequency cell switch if cell switch delay requirements are agnostic for intra-frequency and inter-frequency, same as existing HO delay requirements. - Option 4 (Ericsson): Follow legacy procedure and do not define any explicit definition in the spec

We prefer option 1 which is aligned with L3 HO. We think there is no consensus on the definition of inter-frequency cell switch yet because there may be scell before cell switch which make it a bit complex. In our understanding, cell switch means SpCell changes to another cell. Whether it is intra-frequency cell switch or inter-frequency cell switch depends on whether the target cell is an intra- or inter- frequency cell of SpCell.

In last meeting, some companies have some concern on defining inter-frequency cell switch as we don’t have the definition for inter-frequency HO.

To our knowledge, the only difference between intra-frequency cell switch and inter-frequency cell switch may lie in that different time is needed for cell search Tsearch if the target cell is unknown. It is better to have the same definition as L3 HO. We can further discuss whether Tsearch is needed and what the exact value is for different cases. We think it is better to have a definition to align the understanding. Without such a definition, there will be some ambiguous if the target cell is a cell on the same frequency of Scell. But if there does have some concern, we are fine to discuss the requirements directly and make the description concise instead of defining inter-frequency cell switch.

Proposal 1: Inter-frequency cell switch is defined as SSBs of SpCell and the target cell are on different frequency layers.

2.2 Requirements for DL/UL synchronization before cell switch command

In RAN4#106, there are some proposals on DL/UL synchronization before cell switch command as below. We would like to provide our views.

Issue 2-4-1: Whether to specify requirements for downlink/uplink synchronisation before cell switch < Wayforward >: FFS the following options - Option 1 (Apple): No need to define specific requirements for downlink synchronisation before cell switch since it has already been covered by existing L3 measurement requirements. - Option 2 (Intel): If TCI state switch command can be sent before cell switch, depending on progress of RAN1, RAN4 may need to further discuss how to update current requirement for TCI activation, e.g. timing offset, active BWP. - Option 3 (QC): RAN4 to discuss whether and how to define delay and interruption requirements for PDCCH ordered PRACH transmission to LTM cell for which UE needs additional processing to build and load RF scripts. It is also up to decisions from other working groups. - Option 4 (Ericsson): - RAN4 to study interruption requirements due to PRACH transmission - RAN4 to discuss the DL synchronization requirements and the number of cells for which DL pre-synchronization can be maintained at the UE. - RAN4 to discuss downlink synchronisation requirements for UE before receiving cell switch command.

DL synchronization before cell switch command

RAN1 have agreed to support DL synchronization on target cell before cell switch command. In our understanding, there are several levels of DL synchronization:

- Level 1: obtain symbol boundary

- Level 2-1: obtain symbol index information and frame boundary

- Level 2-2: obtain SFN

- Level 3: T/F fine tracking

Through L3 measurement, UE can reach level 1 DL synchronization. If deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter-r17 is enabled, UE can reach level 2-1 and level 2-2 DL synchronization without reading SSB index in L3 measurement. Otherwise, UE needs to read SSB index in L3 measurement to reach level 2-1 DL synchronization. For FR1, as UE does not need to decode PBCH to get SSB index, UE would not reach level 2-2 DL synchronization through L3 measurement if deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter-r17 is not enabled.

In our understanding, Level 3 DL synchronization on non-serving cell is reached by activating a TCI state of non-serving cell. The purpose of DL synchronization on target cell before cell switch command is to omit the time for T/F fine tracking and decoding MIB to get SFN during cell switch procedure. UE can get SFN before cell switch through L3 measurement (reading SBI) in FR2 or NW configuration (deriveSSB-IndexFromCell or deriveSSB-IndexFromCellInter-r17 is enabled, or NW configures candidate cells’ SFN). If SFN is obtained through SBI reading in FR2, RAN4 already defined the requirements. If SFN is obtained from NW configuration, RAN4 does not need to define any related requirements. In summary, we don’t think RAN4 need to define any requirements for getting SFN before cell switch command. For T/F fine tracking, RAN1 have not agreed to support T/F fine tracking on neighbor cell yet to our information.

Proposal 2: RAN4 do not need to define any requirements for getting SFN of target cell before cell switch command.

Proposal 3: Further discuss whether and how to define requirements for T/F fine tracking on neighbor cell after RAN1 concludes supporting pre-T/F fine tracking on neighbor cell.

UL synchronization before cell switch command

RAN1 have agreed to support PDCCH ordered RACH for candidate cell(s). There would be some requirements. As shown below, RAN1 have defined some requirements for PDCCH ordered RACH for the serving cell, i.e., the delay to transmit PRACH after receiving PDCCH order and the scheduling restriction during PRACH transmission. We think it is better to define similar requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

From 38.213 If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - ∆BWPSwitching=0 if the active UL BWP does not change and ∆BWPSwitching is defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N2 assuming SCS configuration μ=0. For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

Observation 1: It is better to define similar delay and scheduling requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

Regarding the transmit timing accuracy, we think the legacy requirements apply as highlighted below.

38.133 cl.7.1.2 The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH. - when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive.

Proposal 4: The legacy transmit timing accuracy requirement in 38.133 cl.7.1.2 is also applicable to PDCCH ordered RACH transmission for candidate cell(s).

- Summary

In this paper, we provide some views on L1/L2 based inter-cell mobility. We have the following proposals and observation:

Proposal 1: Inter-frequency cell switch is defined as SSBs of SpCell and the target cell are on different frequency layers.

Proposal 2: RAN4 do not need to define any requirements for getting SFN of target cell before cell switch command.

Proposal 3: Further discuss whether and how to define requirements for T/F fine tracking on neighbor cell after RAN1 concludes supporting pre-T/F fine tracking on neighbor cell.

Observation 1: It is better to define similar requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

Proposal 4: The legacy transmit timing accuracy requirement in 38.133 cl.7.1.2 is also applicable to PDCCH ordered RACH transmission for candidate cell(s).

4 Reference

R4-2303175, “WF on NR Mobility Enhancements RRM requirements (part 1)”, MediaTek Inc., RAN4#106 meeting, 27 February –03 March. 2023.