3GPP TSG-RAN WG4 Meeting # 106bis	R4-2304930

Electronic Meeting, April 17 – April 26, 2023

Agenda Item: 5.33.1

Source: MediaTek Inc.

Title: Discussion on SL-RSRP and SD-RSRP

Document for: Discussion

- Introduction

RAN2 have sent RAN1 and RAN4 a LS asking about the comparison of SL-RSRP and SD-RSRP measurement [1]. In this contribution, we will provide our view on this issue.

- Discussion

RAN4 defined measurement accuracy requirements for SL-RSRP and SD-RSRP. When defining the accuracy requirements, the conditions are the same for SL-RSRP and SD-RSRP:

- both are one-shot measurement

- both are based on DMRS for PSCCH and/or PSSCH transmitted from one port

- with the same side condition, i.e., SNR=0 dB

The accuracy requirements are also the same for SL-RSRP and SD-RSRP.

Observation 1: Measurement accuracy requirements for SL-RSRP and SD-RSRP are the same.

However, as pointed in RAN2 LS, the transmission power of SL data and discovery signal may be different due to power control mechanism, this may make direct comparison of SD-RSRP and SL-RSRP impossible. Regarding power control, it is up to RAN1.

Based on the above analysis, we suggest just sending RAN2 the information that measurement accuracy requirements for SL-RSRP and SD-RSRP are the same, and not concluding on whether direct comparison of SD-RSRP and SL-RSRP is possible.

Proposal 1: Just send RAN2 the information that measurement accuracy requirements for SL-RSRP and SD-RSRP are the same and not conclude on whether direct comparison of SD-RSRP and SL-RSRP is possible in RAN4.

- Summary

In this paper, we have some discussion on R18 sidelink relay enhancements. We have the following proposals:

Observation 1: Measurement accuracy requirements for SL-RSRP and SD-RSRP are the same.

Proposal 1: Just send RAN2 the information that measurement accuracy requirements for SL-RSRP and SD-RSRP are the same and not conclude on whether direct comparison of SD-RSRP and SL-RSRP is possible in RAN4.

4 Reference

- R2-2302234, LS on Comparison of SL-RSRP and SD-RSRP measurements, Nokia, RAN2#121, Mar. 2023.