3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304941

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of triple beat rules for MSD analysis

Agenda item:	4.1.1.2

Document for:	Discussion and Approval

Introduction

In RAN4#105 companies presented analysis steps for triple beat analysis that covers inter-band CA in UL with one of the UL bands having (non-)contiguous intra-band carrier aggregation as well [1]. In this document we would like to discuss is the analysis conditions should be different in aspects of category and analysis steps.

Triple beat analysis

In the WF [1] further descriptions of how triple beat rules may be captured there is the assumption of the analysis aspect that “ULCAMBW is the non-contiguous 2 tone allocation spacing” which is the reason the analysis defines f1, f2 and f3 making the UL a three-tone state of the UE. A non-contiguous 2 tone allocation is also configurable in LTE as the non-contiguous resource allocation, which could be applied even in single carrier operation. For NR this configuration is only supported in a ULCA case – not single channel configuration of the uplink.

The analysis of [1] may serve as well for the case of non-contiguous UL intra-band CA with inter-band ULCA and we propose to look at this combination rather than the special case of UL resource allocation separation in intra-band contiguous ULCA. Both UL configurations have the same impact as they separate to uplink spectrum. In NC IB ULCA it is CC’s that are separate in C IB ULCA it may be the PRBs that are separated, and it may be much less distance and spectrum allocation that the NC case.

We present in Table 1 an overview of the different ULCA combinations of FDD and TDD along with special cases of simultaneous RX/TX or not. From the table we show the analysis variants of an inter-band ULCA with non-contiguous UL intra-band CA (treat contiguous UL intra-band CA with non-contiguous resource allocation similarly)

Table 1: Overview of analysis for any FDD and TDD combination with NC ULCA

|  | FDD-FDD | FDD-TDD NC ULCA in FDD | FDD-TDD NC ULCA in TDD | TDD-TDD |
| --- | --- | --- | --- | --- |
| Simultaneous RX/TX | MSD w. Triple beat analysis for both FDD RX bands | 1) MSD w. NC ULCA analysis into TDD band 2) MSD w. triple beat analysis only for FDD RX band | MSD w. triple beat analysis only for FDD RX band | MSD w. NC ULCA analysis into other TDD RX band |
| Non-simultaneous RX/TX |  | 1) MSD w. NC ULCA analysis into TDD band 2) MSD w. triple beat analysis only for FDD RX band | MSD w. triple beat analysis only for FDD RX band |  |

What the table shows is that FDD-TDD cases will require or must be accompanied with an NC ULCA analysis when the NC ULCA is within the FDD band. We have presented the type of analysis for NC ULCA in [2].

For the collision check we do not see the component carrier spacing in a non-contiguous UL intra-band CA considered so far in the frequency range check calculations. We encourage that both the minimum and the maximum component carrier separation is included in the frequency range check like the non-contiguous resource allocation of [1] as we have shown to be important in [2].

For the collision check we do not see the component carrier spacing in a non-contiguous UL intra-band CA considered so far in the frequency range check calculations.

We encourage that both the minimum and the maximum component carrier separation is included in the frequency range check as we have shown to be important in [2].

Triple beat with (non-) contiguous intra-band ULCA

Same as the approach for intra-band ULCA an analysis that includes the aspects of least and largest channel/PRB separation has been made to identify the products that define the ranges for which the IMD analysis must be based. Murata and Skyworks have looked into the rules for triple beat in [1] and made the observations to look only at FDD-FDD and FDD-TDD, since non-simultaneous RX/TX TDD-TDD has no RX during TX and simultaneous RX/TX TDD-TDD will become similar to the intra-band ULCA case, as captured in Table 1. Additionally, FDD-TDD is only relevant with the impact on the FDD RX, since the TDD RX case falls into regular NC ULCA analysis [2].

Only the 1st order triple beat is considered since this is already a 3rd order intermodulation product. It is not expected that 2nd order harmonics of the UL signals will cause any impact due to triple beat. This is because the maximum output power of the fundamentals in triple beat already have been backed off to keep compliancy with the power class, which in turn would reduce the harmonic response even further than the back-off of the fundamental TX.

Assuming that the maximum frequency separation may be contained inside the frequency band with additional margin it is necessary to consider both lowest and highest channel allocation including the allocation of the second component carrier.  This is shown in Figure 1 in the form of the use fUxL and fUxH.

Figure 1: Overview of carrier bandwidth for triple beat IMD analysis

a = channel bandwidth of 1st CC in aggressor TX band

b = channel bandwidth of 2nd CC in aggressor TX band

c = minimum CC separation + ULCC1 CBW

d = maximum CC separation + ULCC1 CBW

m = channel bandwidth of ULCC in higher band

n = channel bandwidth of ULCC in lower band

fU1L = minimum frequency in TX aggressor band of ULCC1

fU2L = minimum frequency separation in TX aggressor band of ULCC2 (fU2L= fU1L+c)

fU3L = maximum frequency separation in TX aggressor band of ULCC2(fU3L= fU1L+d)

fU1H = maximum frequency in TX aggressor band of ULCC1

fU2H = minimum frequency separation in TX aggressor band of ULCC2(fU2H= fU1H-c)

fU3H = maximum frequency separation in TX aggressor band of ULCC2(fU3H= fU1H-d)

fSCCL_L = minimum frequency in singleCC lower band

fSCCL_H = maximum frequency in singleCC lower band

fSCCH_L = minimum frequency in singleCC higher band

fSCCH_H = maximum frequency in singleCC higher band

Analyzing the first order triple beat products of FDD-TDD with FDD below TDD, seeking insight into the FDD RX victim band we see from the analysis below that the channel/PRB separation determines how the triple beat extends across the FDD RX band based on the separation between the CC’s/PRBs within the TDD band. An exemplary case of CA_n28A-n77(2A) is presented in Figure 2.

The channel/PRB separation determines how the triple beat extends across the FDD RX band based on the separation between the CC’s/PRBs

Depending on the FDD duplexing topology (UL-DL or DL-UL) it will be the highest (fUL3 or fUH3) separation within the TDD band that forces the triple-beat product into the FDD RX band (in yellow in Figure 2). Notice how low- separation (fU2L - fU1L - fSCCL_H) products remain inside the UL band until the separation increases to the level that the product starts to enter the FDD RX band from the duplex gap direction. In Figure 2 only some products are shown, this is because the differences fU3L - fU1L equals fU3H - fU1H and so on. Figure 2 also captures the high side difference product range (the two TDD UL components added) subtracted the FDD TX (in blue in Figure 2) while the summation product ranges using the purple and green colors is the range located at the highest frequency range.

Figure 2: Lower side FDD-TDD analysis CA_n28A-n77(2A)

Using the same intervals to determine the triple beat analysis outcome for higher side FDD-TDD we get the following outcome using the exemplary case CA_n7A-n40(2A):

Figure 3: Higher side FDD-TDD analysis CA_n7A-n40(2A)

The first range (lower side product) below the 2UL band (n40) adds the 2UL and subtracts the third UL (n7) (fSCCH). The next two ranges are produced by subtracting the 2UL band (n40) and respectively adding or subtracting the third UL (fSCCH). One range is determined by the lowest allocation in the single UL band, fSCCH_L (Upside 1), followed by a range determined by the highest allocation in the single UL band fSCCH_H (Upside 2). The upper range is again the sum products of all UL.

For the Upper side FDD-TDD it is the Upside 1 and Upside 2 ranges that cause the self-interference. Depending on the duplexing topology (UL-DL, DL-UL) it is either the lowest or highest frequency allocation of the single UL band (FDD in this case) that will cause the triple-beat product to fall inside the FDD RX band.

For the case of CA_n7A-n40(2A) similar to CA_n28A-n77(2A), both having (F(DL) > F(UL)) it is the highest FDD channel that will determine the triple beat range that hits the FDD RX.

The FDD-FDD case will combine the outcome of the analysis in Figure 2 and Figure 3, meaning that either of the FDD RX band may get hit.

The analysis of the ranges is shown in Table 2 for CA_n28A_n77(2A) with 10MHz channel BW,10MHz minimum separation and 600MHz maximum separation.

Table 2: Triple-beat products for the exemplary case CA_n28A-n77(2A)

| CC location | fSCCL | fSCCH | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 703 | 748 | 3300 | 3320 | 3900 | 4200 | 4180 | 3600 |
| 1st order 3-beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL3 -fUL1 + fSCCL ¦ | I fUL3 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ | I fUH1 -fUH3 - fSCCL ¦ | I fUH1 -fUH3 + fSCCL ¦ | I fUH1 -fUH3 - fSCCH ¦ | I fUH1 -fUH3 + fSCCH ¦ |
|  | I fUL2 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL2 -fUL1 + fSCCH ¦ | I fUH1 -fUH2 - fSCCL ¦ | I fUH1 -fUH2 + fSCCL ¦ | I fUH1 -fUH2 - fSCCH ¦ | I fUH1 -fUH2 + fSCCH ¦ |
|  | I fUL3 +fUL1 - fSCCL ¦ | I fUL3 +fUL1 + fSCCL ¦ | I fUL3 +fUL1 - fSCCH ¦ | I fUL3 +fUL1 + fSCCH ¦ | I fUH1 +fUH3 - fSCCL ¦ | I fUH1 +fUH3 + fSCCL ¦ | I fUH1 +fUH3 - fSCCH ¦ | I fUH1 +fUH3 + fSCCH ¦ |
|  | I fUL2 +fUL1 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUL2 +fUL1 - fSCCH ¦ | I fUL2 +fUL1 + fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUH1 +fUH2 + fSCCL ¦ | I fUH1 +fUH2 - fSCCH ¦ | I fUH1 +fUH2 + fSCCH ¦ |
| Ranges | 103 | 1303 | 148 | 1348 | 103 | 1303 | 148 | 1348 |
|  | 683 | 723 | 728 | 768 | 683 | 723 | 728 | 768 |
|  | 6497 | 7903 | 6452 | 7948 | 7097 | 8503 | 7052 | 8548 |
|  | 5917 | 7323 | 5872 | 7368 | 7677 | 9083 | 7632 | 9128 |

The grey cells are not relevant since these products match (the former mentioned “fUL3 - fUL1 equals fUH3 - fUH1” part). As for the rest of the products there are intervals as shown in Figure 2 also represented through colored cells. All white cells are not needed since the products they produce are inside the colored ranges. They can therefore be skipped.

Similar range analysis is shown in Table 3 for CA_n7A_n40(2A) with 10MHz channel BW, 10MHz minimum separation and 80MHz maximum separation.

Table 3: Triple-beat products for the exemplary case CA_n7A-n40(2A)

| CC location | fSCCL | fSCCH | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2500 | 2570 | 2300 | 2320 | 2380 | 2400 | 2380 | 2320 |
| 1st order 3-beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL3 -fUL1 + fSCCL ¦ | I fUL3 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ | I fUH1 -fUH3 - fSCCL ¦ | I fUH1 -fUH3 + fSCCL ¦ | I fUH1 -fUH3 - fSCCH ¦ | I fUH1 -fUH3 + fSCCH ¦ |
|  | I fUL2 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL2 -fUL1 + fSCCH ¦ | I fUH1 -fUH2 - fSCCL ¦ | I fUH1 -fUH2 + fSCCL ¦ | I fUH1 -fUH2 - fSCCH ¦ | I fUH1 -fUH2 + fSCCH ¦ |
|  | I fUL3 +fUL1 - fSCCL ¦ | I fUL3 +fUL1 + fSCCL ¦ | I fUL3 +fUL1 - fSCCH ¦ | I fUL3 +fUL1 + fSCCH ¦ | I fUH1 +fUH3 - fSCCL ¦ | I fUH1 +fUH3 + fSCCL ¦ | I fUH1 +fUH3 - fSCCH ¦ | I fUH1 +fUH3 + fSCCH ¦ |
|  | I fUL2 +fUL1 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUL2 +fUL1 - fSCCH ¦ | I fUL2 +fUL1 + fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUH1 +fUH2 + fSCCL ¦ | I fUH1 +fUH2 - fSCCH ¦ | I fUH1 +fUH2 + fSCCH ¦ |
| Ranges | 2420 | 2580 | 2490 | 2650 | 2420 | 2580 | 2490 | 2650 |
|  | 2480 | 2520 | 2550 | 2590 | 2480 | 2520 | 2550 | 2590 |
|  | 2180 | 7180 | 2110 | 7250 | 2220 | 7220 | 2150 | 7290 |
|  | 2120 | 7120 | 2050 | 7190 | 2280 | 7280 | 2210 | 7350 |

The numbers in the colored cells of Table 2 and Table 3 match those shown in Figure 2 and Figure 3 respectively.

A simplified version focusing on the ranges that define the range limits (the colored cells only) A similar table as shown in Table 4 (for CA_n7A_n40(2A)) must be used for triple beat analysis.

Table 4: Proposed triple beat analysis table for CA_n7A_n40(2A)

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2300 | 2320 | 2380 | 2400 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 2500 | 2570 | 2380 | 2320 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 2420 | 2520 | 2550 | 2650 |  | 80 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 2050 | 2280 | 7120 | 7350 |  |  |

A similar table as shown in Table 4 (for CA_n7A_n40(2A)) must be used for triple beat analysis.

The guidance for triple beat analysis shall be captured in TR 38.846 as given in the provided TP in section 4

Returning to the former to exemplary cases and finishing the frequency range checks we get the following outcome.

For CA_n28A-n77(2A) only the FDD RX is relevant and here we see an issue in the frequency range check.

| CC location | fU1L |  | fU2L | fU3L |  | fU1H |  |  | Channel BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 |  | 3340 | 3880 |  | 4200 |  |  | 20 |  |
| CC location | fSCCL |  | fSCCH | fU2H |  | fU3H |  |  | Min channel separation |  |
| Frequency | 703 |  | 748 | 4160 |  | 3620 |  |  | 20 |  |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  | Max channel separation |  |
| Ranges | 123 |  | 743 | 708 |  | 1328 |  |  | 580 |  |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |  |  |
| Ranges | 5892 |  | 7657 | 7343 |  | 9108 |  |  |  |  |
|  |  | RX n28 |  |  |  |  |  |  |  |
| fx_low |  |  |  |  | fx_high |  |  |  |  |
| 758 |  |  |  |  | 803 |  |  |  |  |
| I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ |  |  | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  |
| Clear |  |  |  |  | Issue |  |  |  |  |
| I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ |  |  | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Clear |  |  |  |  | Clear |  |  |  |  |

For CA_n7A-n40(2A) only the FDD RX is relevant and here we see an issue in the frequency range check.

| CC location | fU1L |  | fU2L | fU3L |  | fU1H |  |  | Channel BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2300 |  | 2340 | 2380 |  | 2400 |  |  | 20 |  |
| CC location | fSCCL |  | fSCCH | fU2H |  | fU3H |  |  | Min channel separation |  |
| Frequency | 2500 |  | 2570 | 2360 |  | 2320 |  |  | 20 |  |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  | Max channel separation |  |
| Ranges | 2420 |  | 2540 | 2530 |  | 2650 |  |  | 80 |  |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |  |  |
| Ranges | 2070 |  | 2260 | 7140 |  | 7330 |  |  |  |  |
|  |  |  | RX n7 |  |  |  |  |  |  |  |
| fx_low |  |  |  |  |  | fx_high |  |  |  |  |
| 2620 |  |  |  |  |  | 2690 |  |  |  |  |
| I fUL3 -fUL1 - fSCCL ¦ |  |  | I fUL2 -fUL1 + fSCCL ¦ |  |  | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  |
| Clear |  |  |  |  |  | Issue |  |  |  |  |
| I fUL2 +fUL1 - fSCCH ¦ |  |  | I fUH1 +fUH2 - fSCCL ¦ |  |  | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Clear |  |  |  |  |  | Clear |  |  |  |  |

We would like to return to the rules captured in [1] about same or adjacent band group as at this points and opens up for further agreement. We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

Table 5 : Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

The restrictions on when the triple beat analysis shall be done as summarized also in the beginning of this document in Table 1, may be unclear also when taking the above discussion frequency ranges into account. It is therefor proposed it capture guidance on this in the TR 38.846.

We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

TP to TR 38.846

The following presents a TP to 38.846 to capture the guidance for triple beat analysis.

**************** Start of TP ***************

## 6.5	Guidelines on Co-Existence analysis

6.5.x	Triple Beat Analysis

For triple beat analysis the uplink interband carrier aggregation with one of the bands configured for intra-band carrier aggregation a similar table as the table below must serve for frequency range checks based on minimum and maximum separation within the intra-band transmission. fU is the frequencies within the intra-band transmission and fSCC is the frequencies within the inter-band.

Table 6.5.x-1: Triple Beat Analysis Table (an example for CA_n7A_n40(2A) case)

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2300 | 2320 | 2380 | 2400 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 2500 | 2570 | 2380 | 2320 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 2420 | 2520 | 2550 | 2650 |  | 80 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 2050 | 2280 | 7120 | 7350 |  |  |

Overlapping ranges is determined from the downlink frequency range(s) of the FDD RX band(s) towards the ranges of the table.

**************** End of TP ***************

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: For the collision check we do not see the component carrier spacing in a non-contiguous UL intra-band CA considered so far in the frequency range check calculations.

Proposal 1: We encourage that both the minimum and the maximum component carrier separation is included in the frequency range check as we have shown to be important in [2].

Observation 2: The channel/PRB separation determines how the triple beat extends across the FDD RX band based on the separation between the CC’s/PRBs

Proposal 2: A similar table as shown in Table 4 (for CA_n7A_n40(2A)) must be used for triple beat analysis.

Proposal 3: The guidance for triple beat analysis shall be captured in TR 38.846 as given in the provided TP in section 4

Proposal 4: We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

- References

[1] R4-2220556 WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA, WG4 Meeting #105

[2] R4-2300413 Discussion of non-contiguous intra-band uplink analysis, WG4 Meeting #106