3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304944

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.846 to add guidance on Co-existence studies for Uplink Intra-Band Non-Contiguous CA

Agenda item:	5.1.2

Document for:	Approval

Introduction

In RAN4#106 proposal 1, 2 and 3 in [1] were agreed. With this TP we implement the agreed proposal also to the band combination guidance TR. This is the resulting TP.

TP to TR 38.846

******************************* Start of TP to TR 38.486 *******************************

- 6.5		Guidelines on Co-Existence analysis

6.5.x		Uplink Intra-Band CA with two UL transmissions

When adding a band combination including two uplink transmission in one UL Intra-Band Carrier Aggregation this study is needed for both non-contiguous and contiguous intra-band uplink CA.

If any issues are identified via the calculations presented in Table 6.5.x-1 additional REFSENS requirements may be needed.

Table 6.5.x-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data |  |  |  |  |  | - |
| CC location | fU1L=min freq | fU2L=fUL1+min separation | fU3L=fUL1+max separation | fU1H=max freq | fU2H=fU1H-min separation | fU3H=fU1H-max separation |
| Frequency |  |  |  |  |  |  |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges |  |  |  |  |  |  |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges |  |  |  |  |  |  |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |

******************************* END of TP to TR 38.486 *******************************

- References

[1] R4-2300413, IMD analysis of NC intra-band CA in uplink, Nokia