3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304943

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.718-02-01 Addition of UE co-existence studies for 2 Uplink in one Intra-Band CA

Agenda item:	4.10.1

Document for:	Approval

Introduction

In RAN4#106 proposal 1, 2 and 3 in [1] were agreed. With this TP we implement the agreed proposal to the relevant TR via a TP.

TP for TR 38.718.02-01

**************** Start of TP ***************

## 5.x	CA_nX-nY

*************************** Unchanged CLAUSES Omitted ***************************

### 5.x.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

*************************** Unchanged CLAUSES Omitted ***************************

#### 5.x.2.2	UE co-existence studies

Table 5.x.2.2-1 lists Band nX + Band nY 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band nX and Band nY UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | – |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | – |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  |  | – |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on Table 5.x.2.2-1, nth order IMD may also fall into Rx frequencies of bands nX or band nY.

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_nX-nY | E-UTRA Band ..... | FDL_low | - | FDL_high |  |  | x |
|  | Frequency range |  | - |  |  |  |  |
| NOTE x: ..... Editor's note: The NOTE order must keep consistent with the Table 6.5A.3.2.3-1 in TS38.101-1. |  |  |  |  |  |  |  |

#### 5.x.2.2.1	UE co-existence studies for 2 Uplink in one Intra-Band CA

Editor's note: This study is only needed in case of 2 uplink transmission in one UL Intra-Band Carrier Aggregation. Applicable to both non-contiguous and contiguous intra-band uplink CA.

Table 5.x.2.2.1-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data |  |  |  |  |  | - |
| CC location | fU1L=min freq | fU2L=fUL1+min separation | fU3L=fUL1+max separation | fU1H=max freq | fU2H=fU1H-min separation | fU3H=fU1H-max separation |
| Frequency |  |  |  |  |  |  |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges |  |  |  |  | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges |  |  |  |  |  |  |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges |  |  |  |  |  |  |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |

#### 5.x.2.3	REFSENS requirements

Editor's note: Text will be added if IMD due to 2 bands UL issues are identified.

**************** End of TP ***************

- References

[1] R4-2300413, IMD analysis of NC intra-band CA in uplink, Nokia