3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304946

# Electronic meeting, 17th – 26th April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DRAFT | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 draftCR to 38 101-1 for Clarification of UL Tx Switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For uplink Tx Switching, the TS38.101-1 defines the location of the switching period for the case where the UE is scheduled or configured to transmit, but there is a switching-induced interruption preventing the transmission to take place If the gNB does not schedule or configure the UE to transmit for a sufficiently large time duration between the end of the transmision on the switch-from carrier and the start of the transmission on the switch-to-carrier the specification is unclear if one or the other transmission is anyway impacted. RAN4 received an LS from RAN1 (R4-2304008) indicating that in such a case the switching period is absorbed by the gap the gNB had provided between the two uplink transmissions: - If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period, - The time of no UL transmission allocated absorbs the switching period - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period. - The setting of uplinkTxSwitchingPeriodLocation has no impact. - Defer the discussion on whether/how to define the exact location of the switching period indicated by the UE capability in time domain to RAN4 - From RAN1 point of view, for Rel-16, the implication is to the time domain location of potential interruption of downlink reception if reported by the UE for the band combination - Defer the potential RAN1 spec change until RAN4 has had the time to react to the RAN1 LS to RAN4. - Send an LS to RAN4 requesting RAN4 to, in this regard, clarify TS38.101-1 subclauses 6.3A.3.3.2 and 6.3C.3.1 for CA, and SUL based UL Tx Switching, and to TS38.101-3 subclause 6.3B.4.1 for EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that if there is sufficient time of no scheduled or configure uplink transmission on the switch-from carrier before the start of the transmission on the switch-to carrier, the switching period is absorbed by that period of no-transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification can be understood to always cut either the end of the transmission on the switch-from carrier or the start of the transmission on the switch-to carrier even if there was a scheduled gap between the transmissions that could absorb the switching period. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.2, 6.3C.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR |  |  |  |  |  |  |  |  |

##### 6.3A.3.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this sub-clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power when the capability uplinkTxSwitching-PowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from carrier 1 to carrier 2, if there is no uplink transmission scheduled or configured on carrier 1 for at least the duration of the switching period (X µs) before the start of the transmission on carrier 2, the switching period is fully contained in the time period between the end of the transmission on carrier 1 and the start of the transmission on carrier 2.

When switching from carrier 2 to carrier 1, if there is no uplink transmission scheduled or configured on carrier 2 for at least the duration of the switching period (X µs) before the start of the transmission on carrier 1, the switching period is fully contained in the time period between the end of the transmission on carrier 2 and the start of the transmission on carrier 1.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

[unchanged subclauses not shown]

#### 6.3C.3.1	Time mask for switching between two uplink carriers

The switching time mask specified in this sub-clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod is present, is only applicable for uplink switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [10], where NR SUL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3C.3.1-1a and Figure 6.3C.3.1-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from carrier 1 to carrier 2, if there is no uplink transmission scheduled or configured on carrier 1 for at least the duration of the switching period (X µs)  before the start of the transmission on carrier 2, the switching period is fully contained in the time period between the end of the transmission on carrier 1 and the start of the transmission on carrier 2.

When switching from carrier 2 to carrier 1, if there is no uplink transmission scheduled or configured on carrier 2 for at least the duration of the switching period (X µs) before the start of the transmission on carrier 1, the switching period is fully contained in the time period between the end of the transmission on carrier 2 and the start of the transmission on carrier 1.

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].