3GPP TSG-RAN WG4 Meeting #106bis	R4-2304996

Electronic Meeting, 17 – 26 Apr, 2023

Source:	ZTE Corporation

Title:	Discussion on L1-RSRP measurement requirements for simultaneous DL reception from different directions

Agenda Item:	5.8.3.2

Document for:	Approval

- Introduction

In RAN plenary 98 meeting, a revised WID[1] for Rel-18 was approved to further clarify the scope of the  simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. The main update is to remove the sub bullet related with L3 measurement delay and put the L3 measurement related aspects into the NOTEs. The objective in the revised WID is as follows:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTEs: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited. - The work on L3 measurement related aspects for scheduling/measurement restriction requirements is not precluded. - Further check in RAN #100 whether to include other L3 measurement related aspects and objectives subject to RAN4 progress.

In last RAN4 meeting, RAN4 has spread extensive discussion around the L1-RSRP measurement and some agreements were achieved in [2].

<Agreement >: #### Issue 1-2-7: Necessity of group-based beam reporting for simultaneous reception - Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases. #### Issue 1-1-6: Simultaneous L3 measurements and L1 measurements - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx. - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS. #### Baseline L1 measurement requirements for potential multi-Rx enhancement - For cell specific RLM: section 8.1 - For cell specific link recovery: section 8.5 - For TRP specific link recovery: section 8.18 - For L1-RSRP measurement: section 9.5.

In this document, we provide our further analysis focus on L1-RSRP measurement around simultaneous multi-panel reception.

- Discussion

Regarding to L1 measurement enhancement under multi-panel Rx, it has been approved that there is not any room to reduce the sample number in 104bis meeting. So we can explore from the following aspects:

- The relation with the group based report

- The reduction of scaling factor for Rx beam sweeping

- Scheduling restriction

- Measurement restriction

- The last two aspects would be discussed in another document from us. So we would discuss the first two aspects.

2.1 Group based report

- The group based report was supported by RAN1 from R15. Further more, in R16 and R17, some further enhancements were introduced. The details in [3] are shown as follows:

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Capability[Set]Index' or 'ssb-Index-RSRP-Capability[Set]Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters. - if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-SINR', 'ssb-Index-SINR', 'cri-SINR-Capability[Set]Index' or 'ssb-Index-SINR-Capability[Set]Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting. - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE.

- Within the above details, we can see three UE capabilities are referred: groupBeamReporting, groupSINR-reporting-r16 and mTRP-GroupBasedL1-RSRP-r17. The exact specification on them as as follows:

| groupBeamReporting Indicates whether UE supports RSRP reporting for the group of two reference signals. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| groupSINR-reporting-r16 Indicates whether UE supports group based L1-SINR reporting. UE indicates support of this feature shall indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| mTRP-GroupBasedL1-RSRP-r17 Indicates the support of group based L1-RSRP reporting enhancements. This feature also includes following parameters: - maxNumBeamGroups-r17 indicates the maximum number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets. - maxNumRS-WithinSlot-r17 indicates the maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs. - maxNumRS-AcrossSlot-r17 indicates the maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. maxNumRS-WithinSlot-r17 and maxNumRS-AcrossSlot-r17 are also counted in maxTotalResourcesForOneFreqRange-r16 and maxTotalResourcesForAcrossFreqRanges-r16. | Band | No | N/A | N/A |

In R15, the L1-RSRP group-based report is supported. If the UE is capable of groupBeamReporting and the NW enable the groupBasedBeamReporting, the UE can report a pair of SSBRI or CRI to support mTRP transmission.

In R16, the L1-SINR group based report is supported. The mechanism is similar as L1-RSRP group-based report in 15. The corresponding UE capability is groupSINR-reporting-r16.

In R17, some further enhancement on L1-RSRP group-based report is introduced. NW can configure two CSI-RS resource set to UE, so that each CSI-RS resource set corresponds to each TRP. Further more, the number of SSBRI or CRI pair reported by UE is extended from one to nrofReportedRSgroup. The corresponding UE capability is mTRP-GroupBasedL1-RSRP-r17. For L1-SINR report, not any enhancement achieved in R17.

Observation 1: L1-RSRP group-based report is supported from R15 and is further enhanced in R17. L1-SINR group-based report is supported from R16. Three corresponding UE capabilities: groupBeamReporting, groupSINR-reporting-r16 and mTRP-GroupBasedL1-RSRP-r17 were identified in R15, R16 and R17 respectively.

Observation 2: The three corresponding UE capabilities are all optional type.

In last RAN4 meeting, it has been approved that R17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception, such simultaneous reception includes Data/Data, RS/RS and Data/RS cases. With such prerequisite, NW can identify which beam pairs can be used for simultaneous reception then when NW configures the RS or schedules the data, the available beam pairs can be considered. So before receiving the group-based report, the NW can not schedule the data which would be simultaneously received with other data or RS.

Observation 3: Before receiving the group-based report, the NW can not schedule the data which would be simultaneously received with other data or RS.

Regarding to the RS configuration, for the SSB, SP CSI-RS and AP CSI-RS, the beam can be relatively flexible determined, so no impact. While for the P CSI-RS, since the qcl-InfoPeriodicCSI-RS is configured at the CSI-RS resource configuration, which can only be updated when adding or releasing an NZP-CSI-RS-Resource. If without any adding or releasing operation, the NW has to choose the suitable CSI-RS to match other CSI-RS or data to simultaneously transmit.

Another point should be noticed is that based on RAN1 specification “group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting”, it can be inferred that within the group reported beam pair, only same RS type are supported, i.e. a beam pair of {SSB, SSB} or {CSI-RS, CSI-RS} is allowed. While the mixture of different RS types is not allowed. Therefore, given that the group-based report is the prerequisite, only the same type of RSs can be simultaneously received by UE. For the case of intra-cell mTRP, a single PCI shared by the multiple TRP, so for the simultaneous RS reception, only one type of combination is possible, i.e. CSI-RS and CSI-RS. If we further consider the case of inter-cell mTRP, the combination of SSB and SSB is also possible.

Observation 4: Since RAN1 has specified that only same type of RSs is allowed to report as a beam pair, for the case of intra-cell mTRP, in terms of simultaneous RS reception, only the combination of CSI-RS and CSI-RS is allowed. If the case of inter-cell mTRP can be further considered, the combination of SSB and SSB is also possible.

2.2 The reduction of scaling factor N for Rx beam sweeping

During RRM measurement and L1-RSRP measurement, for SSB based, the UE always need to perform Rx beam sweeping to acquire the Rx beam tracing. The different point is that for RRM measurement, coarse beam is assumed since of the lack of prior information. While for L1-RSRP measurement, it is only oriented to the serving cell, so fine beam is assumed during the SSB based L1-RSRP measurement.

For the UE configured with multiple antenna panels, since of some prior information acquired by the UE, so for Rx beam sweeping of L1-RSRP measurement, maybe one antenna panel has been down-selected through the previous RRM measurement. Under such assumption, the Rx beam sweeping of L1-RSRP measurement is only performed on a single antenna panel. Another UE implementation is that still all antenna panels are referred to during Rx beam sweeping of L1-RSRP measurement.

- Assumption 1: L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement

- Assumption 2: L1-RSRP measurement is performed by multiple panels

Observation 5: For the case of a single panel has been down-selected by previous RRM measurement(named as Assumption 1, the Rx beam sweeping during L1-RSRP measurement is performed only by a single panel. While for other case(named as Assumption 2, the Rx beam sweeping during L1-RSRP measurement is performed by multiple panels.

Combined with the group-based report, once the UE has reported the beam pair(s) to NW, which means the UE can receive such beam pair(s) simultaneously. Under such assumption, it can be assumed that for the UE reported SSB/CSI-RS or other SSB/CSI-RS QCL-typeDed with the reported SSB/CSI-RS within a group pair, so as to realize simultaneous reception with another beam direction, the UE would receive such SSB/CSI-RS only based on a single panel rather than multiple panels.

Nevertheless, the RSRP reported in the group pair maybe very low since of the UE movement or rotation, RAN1 did not introduce any fallback mechanism to support the group-based report switch to non group-based report when not any beam pair can work with good performance. So it is subject to the NW implementation. Once receiving a group report with very low RSRP, the NW would not transmit multiple RS from different TRPs in parallel, so that the UE can perform beam training through multiple panels. So the assumption of performing the L1-RSRP measurement by a single panel, which may need some additional side condition.

Observation 6: From RAN1 perspective, once the group-based report configured for the UE, no matter the measurement result is good or not, the UE always reports some group pairs. Not any fallback mechanism allowed to support the group-based report switching to non group-based report when not any beam pair can work with good performance.

Proposal 1: Under the prerequisite of group-based report, some additional side condition is needed to guarantee the UE performing the SSB/CSI-RS based L1-RSRP measurement by a single panel. Such side condition can be some RSRP threshold for the group report.

Under Assumption 1 furthermore the additional side condition can meet, it is convenient to realize simultaneous two L1-RSRP measurement procedures for two TRPs. As shown in the below figure 1. In this case, UE can perform two L1-RSRP measurement for the two TRPs in parallel. while for the UE not capable of simultaneous multi-panel Rx, it can only perform the two L1-RSRP measurement sequentially, as shown in below Figure 2.

- Figure 1. Perform L1-RSRP measurement for two TRPs in parallel

- Figure 2. Perform L1-RSRP measurement for two TRPs sequentially

The total measurement period can be reduced since of the relaxation on measurement restriction. However regarding to the beam sweeping factor, we can not see direct enhancement.

Proposal 2: Under Assumption 1, i.e. L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement, no room to reduce the scaling factor of Rx beam sweeping.

Under Assumption 2, as shown in Figure 3, it is obvious that the fully beam sweeping needs multiple panels’ participation, for the UE capable of simultaneous multi-panel Rx, UE can perform partial beam sweeping by each panel at the same time as below. So the beam sweeping factor can be reduced by a half, i.e. from N to N/2. Further more, one condition is that each panel can cover half of the fully beam directions. If the beam coverage of multiple panels are different, e.g. one panel covers 2 beam directions and another panel covers 6 beam directions, as shown in figure 4, under such situation, the beam sweeping factor can not relax from N to N/2, but only relax from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

- Figure 3. Each L1 measurement needs beam sweeping including multiple panels

- Figure 4. Each panel covers different number of beam directions

Proposal 3: Under Assumption 2, i.e. L1-RSRP measurement is performed by multiple panels, UE can perform partial beam sweeping by each panel at the same. So the beam sweeping factor can be reduced.

- For the case of each panel can cover half of the fully beam directions, the beam sweeping factor can be reduced by a half, i.e. from N to N/2.

- For other cases, the beam sweeping factor can be reduced from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

- Conclusion

In this contribution, we have the following observations and proposals around L1-RSRP measurement for simultaneous DL reception from different directions:

Observation 1: L1-RSRP group-based report is supported from R15 and is further enhanced in R17. L1-SINR group-based report is supported from R16. Three corresponding UE capabilities: groupBeamReporting, groupSINR-reporting-r16 and mTRP-GroupBasedL1-RSRP-r17 were identified in R15, R16 and R17 respectively.

Observation 2: The three corresponding UE capabilities are all optional type.

Observation 3: Before receiving the group-based report, the NW can not schedule the data which would be simultaneously received with other data or RS.

Observation 4: Since RAN1 has specified that only same type of RSs is allowed to report as a beam pair, for the case of intra-cell mTRP, in terms of simultaneous RS reception, only the combination of CSI-RS and CSI-RS is allowed. If the case of inter-cell mTRP can be further considered, the combination of SSB and SSB is also possible.

Observation 5: For the case of a single panel has been down-selected by previous RRM measurement(named as Assumption 1), the Rx beam sweeping during L1-RSRP measurement is performed only by a single panel. While for other case(named as Assumption 2), the Rx beam sweeping during L1-RSRP measurement is performed by multiple panels.

Observation 6: From RAN1 perspective, once the group-based report configured for the UE, no matter the measurement result is good or not, the UE always reports some group pairs. Not any fallback mechanism allowed to support the group-based report switching to non group-based report when not any beam pair can work with good performance.

Proposal 1: Under the prerequisite of group-based report, some additional side condition is needed to guarantee the UE performing the SSB/CSI-RS based L1-RSRP measurement by a single panel. Such side condition can be some RSRP threshold for the group report.

Proposal 2: Under Assumption 1, i.e. L1-RSRP measurement is performed only by a single panel down-selected by RRM measurement, no room to reduce the scaling factor of Rx beam sweeping.

Proposal 3: Under Assumption 2, i.e. L1-RSRP measurement is performed by multiple panels, UE can perform partial beam sweeping by each panel at the same. So the beam sweeping factor can be reduced.

- For the case of each panel can cover half of the fully beam directions, the beam sweeping factor can be reduced by a half, i.e. from N to N/2.

- For other cases, the beam sweeping factor can be reduced from N to maximum[N1, N2], here N1and N2 refer to the number of beams covered by each panel respectively.

- Reference

- RP-223527 “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

- R4-2303302 “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo.

- TS38.214-h40