# 3GPP TSG-RAN WG4 Meeting #106bis 	R4-2305034

# Electronic Meeting, 17 - 26th April, 2023

# Source:	ZTE Corporation

# Title:	Discussion on Network B requirements

# Agenda item:	5.26.2.4

# Document for:	Approval

Introduction

- At RAN 95 meeting the revised WI “Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR” [1] was approved.  In the last meeting, we had the discussion on Network B requirements, and the outcomes were captured in [2].

Discussion

In #98e meeting, we discussed whether to remove NW B requirements in R18 or not [3]. Majority companies would like to remove Network B requirement in R18, however some companies do not agree. There is no consensus on it. Rapporteur thinks that we can leave it as RAN4 now. The outcomes of discussion on NW B requirements in the last meeting as below:

Issue 4-1-1: Whether to define network B requirements - Proposals - Option 1: Deprioritize NW B requirement at R18. (Xiaomi ZTE) - Option 2: No measurement requirements in network B will be defined by RAN4 at R18 (Apple oppo Huawei MTK Qualcomm) - Option 3: Define Network B requirements when UE is allocated MUSIM gaps (Nokia CMCC, Ericsson) - Option 3-1: Define NW B requirements only under the following conditions: (vivo, Ericsson) - Only define NW B measurement/cell reselection requirements at idle mode (Apple oppo) - No more stringent requirements when measurements are performed based on MUSIM gaps, or maximum one measurement per DRX cycle. - Focus on the simplest scenario where MUSIM gap is not collided with other gaps or not dropped or not shared (Huawei) - Re-use the existing cell reselection requirements for IDLE as baseline with DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (Huawei) - Agreements - Define NW B measurement/cell reselection requirements in IDLE mode only - Prioritize the scenario where - MUSIM gap is not colliding with other NW A gaps and not dropped - NW A configures MUSIM gaps requested by UE - FFS whether and how to define test cases for these requirements Recommended WF - Continue discussion other conditions when defining NW B requirements

From a network perspective, any network may be NW B. We unanimously agreed in the last meeting that its requirements need to be defined, but there are related limitations, as stated in the agreements above. Based on the above limitations, it can be seen that defining NWB's requirements is a relatively conventional scenario without gap collisions or discards, and this is only to ensure mobility performance.

Figure 1 the connection between the NW A and NW B

As the figure shown that the connection between the NW A and NW B, when the UE’s SIM A is in RRC connected state in NW A while the UE’s SIM B is in RRC Idle or RRC Inactive in NW B, UE will send UAI to the network requesting the gap information allocated by the network. Upon receiving the gap configured by network A, UE will go to network B to perform corresponding operations (such as paging, SI reading, measurement, etc.). During this period, UE and network A will remain connected. However, when the UE is in NW B’s Idle mode, UE shall do the measurement in order to avoiding the change of cell or channels, if UE doesn’t perform cell re-selection right now, UE maybe still in the poor serving cell and the paging for cell re-selection will be missed. Based on above, it is necessary for us to define the requirements for NW B to guarantee the better serving cell switching. Because of the defining requirements’ limitations, the legacy requirements shall be as the baseline or starting point instead of using it directly:

Figure 2 DRX cycle for MUSIM

The periodicity of  the MUSIM gaps can reach a certain value which is lager than the DRX cycle, so the NW B’s requirements shall be defined as max(DRX cycle, MGRP) and this can guarantee the effective requirements.

Observation 1: The periodicity of  the MUSIM gaps can reach a certain value which is lager than the DRX cycle, so the NW B’s requirements shall be defined as max(DRX cycle, MGRP) and this can guarantee the effective requirements.
Proposal 1: The legacy requirements for Idle/Inactive can be the baseline or starting point when defining the requirements for NW B.

Conclusion

In this paper we provided our views on NW B requirements related to MUSIM gaps.

Observation 1: The periodicity of  the MUSIM gaps can reach 5.12s which is lager than the DRX cycle, so the NW B’s requirements shall be defined as max(DRX cycle, MGRP) and this can guarantee the effective requirements.
Proposal 1: The legacy requirements for Idle/Inactive can be the baseline or starting point when defining the requirements for NW B.

- References

- RP-221018, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation

- R4-2303248, WF on NR Dual Tx/Rx Multi-SIM,Vivo

- [98e-21-R18-MUSIM] vivo RP-223461

- R4-2301979, Discussion on NW B requirements with MUSIM gaps, Huawei

- R4-2302166, Discussion on NW B’s requirements, Ericsson