3GPP TSG RAN WG4 Meeting #106-bis-e	R4-2305044

Electronic Meeting, April 17 – April 26, 2023

Agenda item:			5.8.3.5

Source:	vivo

Title:	On TCI state switch delay for multi-Rx

Document for:	Discussion

- Introduction

In the RAN4#106 meeting, there were discussions on dual TCI states switching for FR2 multi-Rx chain DL reception. The following agreements on general aspects relevant to dual TCI states switching were achieved and captured in the WF [1].

- TCI State Switching - Agreements in online session - Group Based Reporting and s/mDCI - Group based reporting applies to both sDCI and mDCI - Investigate and if needed specify requirements for the following TCI state switching scenarios - single TCI to dual TCI - dual TCI to dual TCI - dual TCI to single TCI - TCI Switch triggering method: - MAC-CE based dual TCI state switch for PDCCH reception - DCI based dual TCI state switch for PDCSH reception - FFS RRC triggered TCI state configuration.

There were also many other issues of TCI state switching, which were captured in the topic summary [2]. In this contribution, we further provide our views on TCI state switching requirements for FR2 multi-Rx chain DL reception.

- Discussion

- 2.1 General aspects for dual TCI

Though the reference signals for the dual TCI states may be configured on the same OFDM symbol, it should not be the typical case. Before group-based beam reporting, NW has no information which pair of reference signals can be received simultaneously. It is not possible for NW to configure appropriate beam pair on the same resource. After group-based beam reporting, NW knows the beam pair that UE can receive simultaneously. However, it doesn’t mean NW will configure the reference signals for the beam pair on the same resources. It needs RRC reconfiguration to reconfigure the reference signals and beam pair(s) could change from time to time. Therefore, it is more reasonable to define dual TCI states switching requirements without considering simultaneous reception as baseline.

Proposal 1: The baseline requirements for dual TCI states switching is not based on simultaneous reception of TCI state reference signals.

From enhancement of multi-Rx UE point of view, it is also fine to specify dual TCI state switching with simultaneous reception of TCI state reference signals additionally.

Proposal 2: Enhanced requirements for dual TCI states switching based on simultaneous reception of TCI state reference signals can also be considered additionally.

- 2.2 TCI state switch delay requirements for PDCCH

In the last meeting, following TCI state switching triggering mechanism were agreed.

- TCI Switch triggering method: - MAC-CE based dual TCI state switch for PDCCH reception - DCI based dual TCI state switch for PDCSH reception - FFS RRC triggered TCI state configuration.

In legacy NR requirements, RRC triggered TCI state switch/configuration is for PDCCH reception. The requirements for RRC based TCI state switch delay apply when only 1 TCI state is configured in RRC TCI state list. In our understanding, if multiple TCI states are configured in RRC command, it needs further triggering by MAC CE command. Thus, typically RRC command cannot trigger dual TCI state switching.

Proposal 3: RRC triggered TCI state configuration is only considered after feasibility is confirmed.

MAC-CE based dual TCI states switching for PDCCH

For PDCCH with single-DCI, two cases need to be considered.

- For SFN PDCCH, two TCI states should be activated and indicated by one single MAC CE. Then, the UE can receive from different directions based on the configured dual TCI states. SFN PDCCH is for single-DCI scheduling.

- For PDCCH non-SFN (PDCCH repetition), the TCI states are configured for different CORESETs independently

SFN PDCCH cannot be used for supporting 4-layer MIMO. The dual TCI states switch requirements for SFN PDCCH can be considered not relating to 4-layer MIMO in FR2. The requirements are missing in current specification. Thus, it is necessary to define the corresponding requirements to make the feature more useful though it is not 4-layer MIMO relevant.

PDCCH repetition can only be supported for UE supporting mTRP-PDCCH-TwoQCL-TypeD-r17. With this UE capability, there could be the case that UE can receive time-domain overlapping CORESETs with two QCL type-D. PDCCH repetition may also be used for 4-layer MIMO, e.g., by scheduling SDM PDSCHs.

For PDCCH with multi-DCI, two TCI states are configured for PDCCH, and each TCI state is activated by one MAC CE. Multi-DCI PDCCH is also typical use case for supporting 4-layer MIMO in FR2.

Observation 1: MAC-CE based Dual TCI states switching requirements for PDCCH repetition and PDCCH with multi-DCI is relevant to 4-layer MIMO. MAC-CE based Dual TCI states switching requirements for SFN PDCCH is NOT relevant to 4-layer MIMO.

Proposal 4: For MAC CE based dual TCI states switch, requirements are specified for single-DCI of SFN PDCCH, PDCCH repetition and multi-DCI.

It was agreed in the last meeting following cases are considered as dual TCI states switching to discuss the requirements

- Single DCI to Dual TCI

- Dual TCI to Dual TCI

- FFS: Dual TCI to Single TCI

In our view, the TCI state switch delay requirements could be irrelevant of source TCI state unless one of the target TCI is exactly the same as source TCI state in which case optimization may be considered. It is important to define the baseline requirements with new dual TCI states.

Therefore, requirements for MAC-CE based TCI state switch delay are defined for the following cases.

- Case 1: Dual TCI states based one MAC CE

- Case 2: Dual TCI states based two MAC CEs, and one MAC CE for each TCI state

- Case 3: Single TCI state switched from dual TCI states

Case 1 is for SFN PDCCH. Case 2 is for single-DCI PDCCH repetition and multi-DCI. Case 3 is for fall back from multi-TRP/panel to single TRP/panel.

For case 3, legacy MAC-CE based TCI state switch delay requirements could be re-used largely. One special case could be considered is that the target single TCI state is the same as one of the source dual TCI states, which would lead to different TCI state switch delay requirements from legacy one as optimization could be made.

Proposal 5: Requirements for MAC-CE based TCI state switch delay are defined for the following cases.

-	Case 1: Dual TCI states switch based one MAC CE

-	Case 2: Dual TCI states based two MAC CEs, and one MAC CE for each TCI state

-	Case 3: Single TCI state switched from dual TCI states

The legacy MAC-CE based TCI state switch delay requirements are specified for both known and unknown cases. For known cases, one SSB is needed for timing/frequency tracking if the target TCI state is not within the active TCI state list.

For MAC-CE based dual TCI states switch delay requirements, we need to decide whether both known and unknown cases should be considered. At least, the requirements for known cases should be specified. It needs further consideration if requirements should be specified for unknown case.

Since the target dual TCI states is for simultaneous PDCCH/PDSCH reception, it should be based on group-based beam reporting. Therefore, both the dual TCI states should be either known or unknown. It should be typical use case that both the dual TCI states are known. For unknown case, it may also be considered for the use case that dual TCI states are triggered by MAC-CE later than, e.g., 1280ms, after group-based beam reporting. In this case, beam pair information is still known to NW. NW may still configure the UE to use this beam pair for simultaneous PDCCH/PDSCH reception.

Proposal 6: Requirements for MAC-CE based dual TCI states switch delay for PDCCH reception are defined for known + known case. Unknown + unknown case may also be considered.

It was agreed that for each of the two TCI states, the TCI state switch is assumed to be independent. the definition/scope of “independency” is FFS. In our view, the independent TCI sate switch could only be based on that T/F tracking are independent for the dual TCI states and UE is capable of multi-Rx by using of at least two panels (it could still be one antenna module based on RF conclusion). Depending on assumption of receiving time difference, UE may track timing of only one of the TCI states if receive timing difference is within CP. Otherwise, independent timing tracking for both TCI state switch is needed.

Proposal 7: For UE with multi-Rx chain, it should be able to track timing/frequency independently for each TCI state when dual TCI states are activated either based on T/F tracking for one TCI state or based on independent T/F tracking.

- 2.3 TCI state switch delay requirements for PDSCH

TCI states switching for PDSCH is triggered by DCI. For PDSCH with single-DCI, two TCI states are indicated by a TCI code point. Same threshold is used for the dual TCI states. The dual TCI state switch delay requirements could reuse legacy requirements as baseline.

For PDSCH with multiple-DCI, two TCI states are indicated by two TCI code points in two DCIs. It is considered as two independent DCI based TCI state switch. Legacy requirements could be reused as baseline. Further clarification on applicable conditions may be considered.

Regarding dual TCI states switch for PDSCH, the TCI state switch delay requirements should be specified when both TCI states are known as in legacy requirements. Since unified TCI framework is not considered in the WI, the threshold of time offset between the reception of the PDCCH and the corresponding PDSCH is timeDurationForQCL.

Proposal 8: Requirements for DCI based dual TCI states switch delay for PDSCH reception are defined for known case only.

Proposal 9: Requirements for DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

- 2.4 Known/unknown condition

In general, the known/unknown condition can be defined based on legacy known/unknown conditions for TCI state switch when possible. Additional conditions specific to dual TCI states can be further considered. In our view, the L1-RSRP measurement reporting should be group-based beam reporting that which beam pair(s) UE can receive simultaneously is reported and one reported beam pair is configured as target dual TCI states. Therefore, following option would be reasonable for definition of known/unknown condition for dual TCI states switch.

Proposal 10: Known/unknown conditions for dual TCI states switch can be defined as follows.

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least one group-based L1-RSRP report configured with groupBasedBeamReporting or groupBasedBeamReporting-r17 that the target dual TCI states are reported within one group before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

- 2.5 TCI state list update requirements

As in legacy active TCI state list update requirements, we think requirements for addition of a new TCI state to the active TCI state list is needed even for dual TCI states. Legacy requirements can be reused in this case.

For enhanced TCI states activation/deactivation for PDSCH, addition of dual TCI states may be considered. The T/F tracking for the dual TCI states could still be based on single SSB. In this case the legacy requirements can also be used as baseline. If the T/F tracking for the dual TCI states is based on different SSBs, it is FFS how the requirements are specified.

Proposal 11: For active TCI state list update, requirements for addition of dual TCI states should be specified.

Proposal 12: For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs.

- Summary

In this contribution, we provided views on dual TCI states switching requirements for NR FR2 multi-Rx chain DL reception. Following proposals are present.

Proposal 1: The baseline requirements for dual TCI states switching is not based on simultaneous reception of TCI state reference signals.

Proposal 2: Enhanced requirements for dual TCI states switching based on simultaneous reception of TCI state reference signals can also be considered additionally.

Proposal 3: RRC triggered TCI state configuration is only considered after feasibility is confirmed.

Proposal 4: For MAC CE based dual TCI states switch, requirements are specified for single-DCI of SFN PDCCH, PDCCH repetition and multi-DCI.

Proposal 5: Requirements for MAC-CE based TCI state switch delay are defined for the following cases.

-	Case 1: Dual TCI states based one MAC CE

-	Case 2: Dual TCI states based two MAC CEs, and one MAC CE for each TCI state

-	Case 3: Single TCI state switched from dual TCI states

Proposal 6: Requirements for MAC-CE based dual TCI states switch delay for PDCCH reception are defined for known + known. Unknown + unknown case may also be considered.

Proposal 7: For UE with multi-Rx chain, it should be able to track timing/frequency independently for each TCI state when dual TCI states are activated either based on T/F tracking for one TCI state or based on independent T/F tracking.

Proposal 8: Requirements for DCI based dual TCI states switch delay for PDSCH reception are defined for known case only.

Proposal 9: Requirements for DCI based dual TCI states switch delay for PDSCH reception are to reuse legacy requirements as baseline.

Proposal 10: Known/unknown conditions for dual TCI states switch can be defined as follows.

The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least one group-based L1-RSRP report configured with groupBasedBeamReporting or groupBasedBeamReporting-r17 that the target dual TCI states are reported within one group before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

Proposal 11: For active TCI state list update, requirements for addition of dual TCI states should be specified.

Proposal 12: For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs.

Observation 1: MAC-CE based Dual TCI states switching requirements for PDCCH repetition and PDCCH with multi-DCI is relevant to 4-layer MIMO. MAC-CE based Dual TCI states switching requirements for SFN PDCCH is NOT relevant to 4-layer MIMO.

- References

- R4-2303317	WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2), Qualcomm

- R4-2302768	Topic summary for [106][210] FR2_multiRx_part2, Moderator (Qualcomm)