3GPP TSG RAN WG4 Meeting #106-bis-e	R4-2305048

Electronic Meeting, April 17 – April 26, 2023

Agenda item:			5.11.2.4

Source:	vivo

Title:	On RRM impact of option B-1-2 for BWP operation without restriction

Document for:	Discussion

- Introduction

In the RAN#99 meeting, a new WID to complete the specification support for BWP operation without restriction in NR [1]. The work item includes following objectives:

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

In this contribution, we provide our views on RRM impact of option B-1-2 for BWP operation without restriction.

- Discussion

In the RAN#97 meeting, RAN asks RAN4 to do a high-level analysis of the options in RAN4’s answer to Q2 in RP-221911 and report it to RAN#98 for RAN decision. RAN4 had done the high-level analysis in the two RAN4 meetings in Q4 2022. In the RAN4#105 meeting, RAN4 reached agreements on high-level analysis of down-selected options. The conclusion was reported to RAN plenary meeting in the RAN4 LS [2].

The high-level analysis on RRM requirements impact was summarized in Table 1 in LS report to RAN plenary [2]. For option B-1-2, it was summarized as in Table 1 below.

Table 1: High-level analysis of candidate options on RRM requirements impact/workload in RAN4

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option B-1-1) | - Existing SSB based RLM/BFD/BM measurement requirements will apply. - The applicability rule of existing requirements is to be updated. - Depending on RF BW and BB BW (FFT BW) assumption, at least intra-frequency measurement with gap need to be revisited | Low |
| Option B-1-2) | - Same as Option B-1-1). Additionally, - Interruption requirements need to be developed additionally to allow UE for switching. | Low or Medium |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium RRM requirements impact.

- 2.1 RLM/BFD/BM Requirements for option B-1-2

Same analysis and conclusion for option B-1-1 in section 2.1 as in [3].

Proposal 1: It is clarified in the spec that existing RLM/BFD/CBD/BM(L1-RSRP) requirements for non-RedCap UE are applicable when SSB is outside active DL BWP if the UE supports option B-1-2 for BWP operation without restriction.

- 2.2. Timing requirements for option B-1-2

Same analysis and conclusion for option B-1-1 in section 2.2 as in [3].

Proposal 2: It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

- 2.3 L3 measurement requirements for option B-1-2

Same analysis and conclusion for option B-1-1 in section 2.3 as in [3].

Proposal 3: RAN4 to study and conclude if intra-frequency measurement without gap based on SSB outside active BWP is a must for UE supporting option B-1-2.

Furthermore, if intra-frequency measurement based on SSB outside active BWP is performed without gap, it may have impact on interruption requirements as it needs more RF retuning.

- 2.4 Interruption requirements for option B-1-2

It was agreed in the RAN#99 meeting that the interruption related requirements for UE supporting option B-1-2 will be decided and specified in RAN4.

In general, there are three alternatives to define interruption requirements in RRM specification.

Alt 1: Interruption requirements are specified with interruption length and starting point

Alt 2: Interruption requirements are specified with interruption length and number of interruptions during certain period.

Alt 3: Interruption requirements are specified with interruption length during certain procedure.

Alt 4: Interruption requirements are specified with probability of missed ACK/NACK.

For UE supporting option B-1-2, Alt 4 is the most appropriate approach to define interruption requirements.

Proposal 4: Interruption requirements for UE supporting option B-1-2 are specified with probability of missed ACK/NACK.

When UE is performing RLM/BFD/BM(L1-RSRP) measurements based on SSB outside active BWP, it may cause interruptions for each measurement depending on UE implementation. However, this may have big impact on system throughput. To achieve balance between system performance and UE power consumption, it is proposed that allowed probability of missed ACK/NACK for ALL RLM/BFM/BM(L1-RSRP) measurements based on SSB outside active BWP is 1%.

Proposal 5: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP) measurements based on SSB outside active BWP.

- Summary

In this contribution, we provided our views on RRM impact of option B-1-2 for BWP operation without restriction. Based on above analysis, following proposals are present.

Proposal 1: It is clarified in the spec that existing RLM/BFD/CBD/BM(L1-RSRP) requirements for non-RedCap UE are applicable when SSB is outside active DL BWP if the UE supports option B-1-2 for BWP operation without restriction.

Proposal 2: It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

Proposal 3: RAN4 to study and conclude if intra-frequency measurement without gap based on SSB outside active BWP is a must for UE supporting option B-1-2.

Proposal 4: Interruption requirements for UE supporting option B-1-2 are specified with probability of missed ACK/NACK.

Proposal 5: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP) measurements based on SSB outside active BWP.

- References

- RP-230805	New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, RAN#99

- R4-2220437	LS on BWP operation without bandwidth restriction, vivo, RAN4, RAN4#105

- R4-230x On RRM impact of option B-1-1 for BWP operation without restriction, vivo