3GPP TSG-RAN WG4 Meeting #106bis-e	                  R4-2305074

Online, April 17 – April 26, 2023

Source: 	vivo

Title: 	Discussion and reply LS on interference modelling for duplex evolution

Agenda Item:	5.20.2

Document for:	Approval

Introduction

An incoming LS [1] was received from RAN1 on interference modelling and several issues need to be confirmed by RAN4. In this contribution, we provided our views for the response to the LS.

Discussion

In the following discussion, our replies on interference modelling for duplex evolution are marked in red.

In RAN1#112, RAN1 made the following agreements.

Agreement-1

Agree the updated TR for RAN1 in R1-2300997 in principle.

In RAN4#106 meeting, TR skeleton of RAN4 part for TR 38.858 was also discussed and decided. Companies are encouraged to prepare TPs for interested topics in further meetings.

Agreement-2

Inform RAN4 of the updated RAN1 part of the TR. And include the following conclusion in the LS to RAN4.

Conclusion

Regarding the feasibility analysis of SBFD, RAN1 focus on feasibility analysis from performance perspective, specification perspective and impact on legacy operation perspective. The study on implementation feasibility is up to RAN4.

In RAN4#106 meeting, TR skeleton of RAN4 part for TR 38.858 was discussed and decided. Besides the implementation part, impact on RF requirements, adjacent channel co-existence evaluation and regulatory aspects will be in charge by RAN 4. The Sections for RAN4 part in TR 38.858 are listed in the following table. Companies are encouraged to prepare TPs for interested topics in further RAN4 meetings.

| No. | Section for TR 38.858 |
| --- | --- |
| 1 | 10.1 Background for analysis |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects |
| 5 | 10.5 Feasibility of FR2 BS aspects |
| 6 | 10.6. FR1 Feasibility of UE aspects |
| 7 | 10.7 FR2 Feasibility of UE aspects |
| 8 | 10.8 Summary |
| 9 | 11.1 Impact on BS RF requirements |
| 10 | 11.2 Impact on UE RF requirements |
| 11 | 12 Adjacent channel co-existence evaluation results |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum |
| 13 | Annex <D>: Adjacent channel co-existence evaluation |

Agreement-3

For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows.

Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB

Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj

- Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj.

- Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale).

- NDLRBadj is the total number of DL RBs in adjacent channel.

- Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel.

- αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector.

  - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

  - FFS the concrete value of αadj, co-site-x

  - ACLRBS and ACSBS are in linear scale.

Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

In the second part of reply LS [2], RAN4 provided the spatial isolation and the values 10*log10ACLRBS and  10*log10ACSBS for co-site gNB-gNB adjacent-channel CLI modelling. RAN1 can calculate the value of αadj, co-site-x

based on RAN4’s reply.

  - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4:

- FR1: 62-93dB with 75dB being typical values.

- FR2: 75-98dB with 88dB being typical values.

  - The ACLR/ACS values for FR1 and FR2 are shown in the table below.

| Range | ACLR [dB] | ACS [dB] |
| --- | --- | --- |
| FR-1 | 45 | 46 |
| FR-2 | 28 | 24 |

Agreement-4

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB

where

- IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

- PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

- CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

- NULRB is the total number of UL RBs in the UL subband

- ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

- 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein,

  - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

  - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1

  - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

  - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

    - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

    - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

  - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

  - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

Generally, the above agreement is inline with RAN4’s understanding.

In RAN4, we discussed UE co-channel Rx model and no final consensus was reached. For Sub-band/In-channel selectivity, that is ICSUE in RAN1’s model, the range 20~33 dB was proposed for FR1 and 20~34dB for FR2-1. RAN1 can refer to this range if no further RAN4 reply was received.

Working assumption:

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity, where,

- Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

  - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

  - NR is the number of Rx chains and NT is the number of Tx chains

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

  - yn=y0n,y1n,…,yRnT,

    - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

    - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

- Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB

  - Pblocker=1NR*m∈Used UL RBsHCLImWsm2

  - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

  - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

    - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

  - NULRB is the total number of UL RBs in the UL subbands,

  - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

Generally, the above agreement is inline with RAN4’s understanding.

In RAN4, we discussed UE co-channel Rx model and no final consensus was reached. For Sub-band/In-channel selectivity, that is ICSUE in RAN1’s model, the range 20~33 dB was proposed for FR1 and 20~34dB for FR2-1. RAN1 can refer to this range if no further RAN4 reply was received.

Conclusion

This contribution discusses interference modelling issues for duplex evolution in the LS [1]. Our proposed reply is as in the discussion part.

- References

[1] R1-2302087, LS on interference modelling for duplex evolution, CMCC, RAN1#112.

[2] R4-2302885, Reply LS to RAN1 on interference modelling, CATT, RAN4#106.