3GPP TSG-RAN WG4 # 106-bis-e	R4-2305120

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.19.2

Source:	MediaTek Inc.

Title:	Discussion on Application Layer Data Throughput

Document for:	Discussion

# Introduction

In RAN4#106, the Way Forward document was approved [1]. Also, in RAN4#106 simulation results with and without HARQ retransmissions were already collected in [2]. We provide discussion with proposals for the Rel-18 work in this contribution.

# Discussion

# 2.1 Background

In the following Table 1-1 some main parameters of baseline simulation assumptions of the WF document [1] are listed.

Table 1-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation.

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
| Maximum number of HARQ transmission |  |  | 4 | 4 | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |  |  |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| The Outer Loop Link Adaptation (OLLA) |  |  | Disabled (Follow CQI) |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

# 2.2 Remaining open issues

In this chapter we discuss remaining open items.

Issue 1-3-1: Phy Layer TP test metric

- Average SNR of impairments results to achieve T% of maximum throughput + X dB margin

  - Use Gspan = [2.5] dB to check if the results are aligned

  - Use X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM

X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM

  - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

- Discuss based on the updated simulation results in the next meeting

  - Whether X dB margin is applied to alignment results or impairment results

  - Whether the proposed X dB values are agreeable or not

We see these initial proposals as a good starting point that can be adjusted when simulation results from all interested companies are available.

Proposal #1: Initial proposals of Gspan = 2.5dB and X = 0.5, 0.5, 0.8 and 0.8 for modulations QPSK, 16-QAM, 64-QPSK and 256-QAM respectively is good starting point that can be adjusted in the meeting when simulation results from all interested companies are available.

Issue 1-3-2: Update in ATP simulation alignment results

- Companies are encouraged to bring evaluation results with and without HARQ retransmission

We have provided simulation results in the previous RAN4#106 meeting in separate document [3] that are already included in simulation result collection document [2]. Our simulation assumptions for the previous meeting are still valid and we don’t need to update our previous results.

Issue 1-3-3: Test point T (%) selection

- Test SNR selection criteria

  - Option 1

- Cover both low and higher modulation order/layer

  - Option 2

- For 2Rx: Choose one in rank 1 and one in rank 2

- For 4Rx: Choose both T points in rank 2 region, one in the medium SNR away from rank transition region, and one close to 20 dB (peak SNR).

        - Option 2a: Set of SNR with no/frequent rank transitions

    - Option 2b:

        - For 4Rx: Choose 1 SNR point in high SNR region.

  - Option 3

- Choose the SNRdominant RI transition where major of simulation results shows median RI change

- For 2Rx, add mid-point in [0 ~ SNRdominant RI transition] range

- For 4Rx, add mid-point in [SNRdominant RI transition ~ 20] range

- Test points based on the SNR selection criteria

  - Option 1:

- For FR1 2Rx, T% = (10% or 15%) and (40% or larger)

- For FR1 4Rx. T% = (10% or 15% or 20%) and (45% or larger)

- For FR2 2Rx, T% = (10% or 15% or 20%) and (40% or larger)

  - Option 2:

- For FR1 2Rx, T% = 10% and 40%

- For FR1 4Rx, T% = 15% and 60%

- For FR2 2Rx, T% = 10% and 40%

  - Option 3:

- For FR1 2Rx, T% = 10% and 35%

- For FR1 4Rx, T% = 20% and 55%

- For FR2 2Rx, T% = 10% and 35%

  - Option 4:

- For FR1, T% = 10% and 40%

- For FR2, T% = 10% and 35%

  - Option 5:

- Trimming to T (%) with 5% granularity based on Option 3 for SNR selection

  - For FR1 2Rx, T% = 15% and 30%

  - For FR1 4Rx, T% = 15% and 40%

  - For FR2 2Rx, T% = 20% and 35%

- Discuss in the next meeting with the following aspect based on the updated simulation results

  - SNR options considering uniqueness of test SNR coverage

  - Tentative agreement on T (%) based on simulation results for alignment considering Gspan and margin

  - Confirm T (%) based on simulation results with impairment. It does not preclude the possibility of adjustment with [+- 5% steps] from alignment perspective.

In general, we would slightly prefer Option 2, but we are open to adjust actual values based on final simulation results from all interested companies.

- For FR1 2Rx, T% = 10% and 40%

- For FR1 4Rx. T% = 15% and 60%

- For FR2 2Rx, T% = 10% and 40%

Proposal #2: We slightly prefer Option 2 of Test point T (%) selection from existing options, but we are open to adjust actual values based on final simulation results from all interested companies.

# Conclusion

In this paper we provided the view on the application layer data throughput. The following observations and proposals are made:

Proposal #1: Initial proposals of Gspan = 2.5dB and X = 0.5, 0.5, 0.8 and 0.8 for modulations QPSK, 16-QAM, 64-QPSK and 256-QAM respectively is good starting point that can be adjusted in the meeting when simulation results from all interested companies are available.

Proposal #2: We slightly prefer Option 2 of Test point T (%) selection from existing options, but we are open to adjust actual values based on final simulation results from all interested companies.

# Reference

- R4-2302930, “Way forward on physical layer throughput requirements”, Intel

- R4-2300484, “Summary of simulation results for NR UE absolute physical layer throughput requirements with link adaptation”, Intel

- R4-2301864, “Simulation results for Application Layer Data Throughput”, MediaTek