3GPP TSG-RAN WG4 Meeting # 106bis-e	                               R4-2305142

# Online, April 17 – April 26, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_2UL CA_n3A-n8A-n79A

Agenda Item:	4.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n3A-n8A-n79A in TR38.718-03-01

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2),v0.4.0

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n3-n8-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3-n8-n79 | n3, n8, n79 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n8A-n79A | CA_n3A-n8A CA_n3A-n79A CA_n8A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values have already been included in the spec.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n8A, CA_n3A-n79A and CA_n8A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD3 and IMD4 caused by n3+n8 may fall into the its own band n79 Rx, where the IMDx frequency range are:

- 4300~4485 MHz for IMD3 caused by UL CA_n3-n8

- 4350 ~4530MHz and 4215 ~ 4475 MHz for IMD4 caused by UL CA_n3-n8

IMD3 and IMD4 caused by n3+n79 may fall into the its own band n8 Rx, where the IMDx frequency range are:

- 830~1580MHz for IMD3 caused by UL CA_n3-n79

- 130~955 MHz for IMD4 caused by UL CA_n3-n79

IMD4 caused by n8+n79 may fall into the its own band n3 Rx, where the IMDx frequency range is 1655~2360 MHz

      - 5.x.2.2	REFSENS requirements

In terms of the co-existence studies, the impacted n79 Rx frequency range are from 4300~4485 MHz for IMD3 and  4350 ~4530MHz and 4215 ~ 4475 MHz for IMD4 caused by UL CA_n3-n8. However, in China, the n79 frequency range is restricted to 4800-5000MHz. Therefore, considering this frequency range, n79 will not be impacted by IMD3 and IMD4 caused by 2UL n3 and n8. In the other words, there is no need to define the IMD3 and IMD4 MSD for DL_CA_n3A-n8A-n79_UL_CA_n3A-n8A.

For other MSD requirements, it is proposed to re-use the MSD values of DC_3-8_n79. Therefore, the MSD requirement are defined in table 5.x.2.2-1:

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n8-n79 | n3 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n8 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | TDD | IMD3x, IMD4y |
|  | n3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | 15.3 | FDD | IMD34 |
|  | n3 | 1755 | 5 | 25 | 1850 | 8.8 | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | IMD4 |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE x: There is no IMD3 product in band n79 downlink for n79 operating in 4800 – 5000 MHz frequency range. NOTE y: There is no IMD4 product in band n79 downlink for n79 operating in 4800 – 5000 MHz frequency range. |  |  |  |  |  |  |  |  |

----- End of TP -----