3GPP TSG-RAN WG4 Meeting # 106bis-e	                               R4-2305143

# Online, April 17 – April 26, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-02-01_removing the intra-band UL CA and high power limits information

Agenda Item:	4.10.2

Document for:	Approval

- Introduction

For the maximum output power for inter-band CA_n5-n77, CA_n66-n77 and CA_n71-n77, there are contents related to intra-band non-contiguous CA and high power limits included. However, due to this TR is for PC3 inter-band UL CA, and the TR aims to capture the PC3 inter-band UL CA specific requirements analysis, while intra-band non-contiguous CA and high power limits are the separated topics, they are beyond the TR’s scope.

Meanwhile, the template for the maximum output power for inter-band CA can be found in sub-clause 5.x.2.1-1 in TR38.718-02-01. In the other words, the maximum output power for inter-band CA_n5-n77, CA_n66-n77and CA_n71-n77 are not aligned with the templates.

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +x/-y |

Therefore, to avoid unnecessary misunderstanding, especially for the high power limits, it is necessary to remove the contents which are beyond the scope of this TR.

This contribution provides a text proposal to remove the intra-band UL CA and high power limits information in the TR38.718-02-01.

- Reference

- [1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.4.0

- Text Proposal

----- Start of TP -----

    - 5.14.2		Specific for 2 bands UL CA

      - 5.14.2.1	Maximum output power for inter-band CA

Table 5.14.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n77A | 23 | +2/-3 |

<Next changes>

    - 5.17.2		Specific for 2 bands UL CA

      - 5.17.2.1	Maximum output power for inter-band CA

Table 5.17.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n66A-n77A | 23 | +2/-3 |

<Next changes>

    - 5.18.2		Specific for 2 bands UL CA

      - 5.18.2.1	Maximum output power for inter-band CA

Table 5.18.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n77A | 23 | +2/-3 |

----- End of TP -----