3GPP TSG-RAN WG4 Meeting #106bis-e                	   R4-2305156

Online, April 17 – April 26, 2023

Agenda Item:	7.1.1

Source:	MediaTek Inc.

Title:	Discussion on LS on support of per FR PRS gap

Document for:	Discussion

# Introduction

In [1], RAN2 sent an LS to RAN4, asking for feedback on whether to supports PRS to be associated with per FR measurement gap under concurrent gaps. The content of LS is captured as below for information.

1. Overall Description: In Rel-17, it was allowed to configure the perFR gap for PRS measurements. In the TS 38.306, the UE capability is also specified to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2 for PRS measurement. However, in the current version TS 38.331, the gapAssociationPRS IE can only be configured for per UE measurement gap. During RAN2 discussion on the above issue, there is no consensus whether the PRS measurement can be associated with per FR measurement gap when concurrent gaps are configured, as companies have different understanding whether RAN4 supports so. Therefore RAN2 kindly requests RAN4 to provide feedback whether RAN4 supports PRS to be associated with per FR measurement gap in case of concurrent gaps. 2. Actions: To RAN2 group. ACTION: RAN2 kindly requests RAN4 to provide feedback whether RAN4 supports PRS to be associated with per FR measurement gap in case of concurrent gaps.

In last meeting, a WF [2] was agreed to check the potential impact in TS38.133 when the PRS measurement is associated with a per-FR measurement gap that is configured as part of concurrent measurement gaps:

# 2 Agreement RAN4 should further study the potential impact in TS38.133 when the PRS measurement is associated with a per-FR measurement gap that is configured as part of concurrent measurement gaps: - Gap combination configurations for per-FR PRS measurement in Concurrent measurement gaps - Concurrent measurement gaps collision rule - CSSF - Positioning measurement requirement using per-FR gaps within concurrent measurement gaps The analysis of impact to other aspects may be considered if identified by RAN4 #106-bis meeting.

In this paper, we provide our analysis on the potential spec impact.

# Discussion

The gap combination configuration for concurrent gap is specified in Table 9.1.8-1 of TS38.133, as below.

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2 | 1 | 0 |
| 1 | 1 | 2 | 0 |
| 2 | 0 | 0 | 2 |
| 3Note 1 | 1 | 0 | 1 |
| 4Note 1 | 0 | 1 | 1 |
| 5Note 1 | 1 | 1 | 1 |
| 6 | 2 | 0 | 0 |
| 7 | 0 | 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. |  |  |  |

For a particular combo ID, if network wants to configure per-FR POS gap, this requires UE to support a new capability independentGapConfigPRS-r17. In our view, this needs to be clarified in TS38.133 to avoid misunderstanding.

Proposal 1: Clarify in Table 9.1.8-1 of TS38.133 that if network wants to configure per-FR POS gap, this requires UE to support a new capability independentGapConfigPRS-r17.

# Conclusion

In the contribution, we discuss potential impact in TS38.133 when the PRS measurement is associated with a per-FR measurement gap that is configured as part of concurrent measurement gaps. We have the following proposal.

Proposal 1: Clarify in Table 9.1.8-1 of TS38.133 that if network wants to configure per-FR POS gap, this requires UE to support a new capability independentGapConfigPRS-r17.

# Reference

[1] R4-2300018, “LS on support of per FR PRS gap”, RAN2

[2] R4-2303312, “WF on per-FR Pos gap”, Ericsson