3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2305161

Online, April 17 – April 26, 2023

Agenda Item: 5.8.3.1

Source: MediaTek Inc.

Title: Discussion on general aspects for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “general issues”.

# Discussion

In the following sections, below topics are discussed sequentially.

- Basic assumption

- Scenario

- Signalling and UE capability

- L3 measurement

## 2.1 Basic assumption

In this topic, following four issues are further studied.

- RRM impact of the UE behaviour using a single antenna module

- Beam management

- UE architecture

### 2.1.1 RRM impact of the UE behaviour using a single antenna module

The corresponding discussion [2] for this issue is provided below.

#### Issue 1-1-5: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel - Proposals - Option 1(Apple): - RRM discussion on how a UE supports spatial MIMO can refer to RF agreement on “antenna module” and “panel”. - Option 2(Samsung): - The definition of “Panel” terminology in RRM discussion shall align with that in RF discussion. - Spatial MIMO (either spatial diversity or spatial multiplexing) to receive two independent signals from the same or nearly the same direction is not the scope of this work item. - Option 3(MediaTek): - In R18 multi-Rx chains WI, RRM session deprioritize the case when two signals are received by one active UE Rx antenna module at a time, before any conclusion is reached in RF session. - Option 3(vivo): - spatial MIMO (either spatial diversity or spatial multiplexing) by using one antenna module to receive two independent signals should not be excluded. No RRM requirements impact is identified for the case. - Option 4(ZTE): - Regarding to the spatial MIMO (either spatial diversity or spatial multiplexing) by using a single panel to receive two independent signals from the same of nearly same direction, we do not need to identify any explicit conclusion about this in RRM session, so no need further discussion. - Option 5(OPPO): - It is not necessary to explicitly preclude spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction from the WI. - Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to receive two independent signals from the same or nearly the same direction is up to RF conclusion. No RRM requirements impact is identified. - Option 6(Huawei): - The issues related RF implementation are not duplicate discussed in RRM session. - Option 7(Ericsson): - How Spatial MIMO (either spatial diversity or spatial multiplexing) is used is up to NW and UE implementation. - It is not necessary to explicitly preclude spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction from the WI. - Recommended WF - Needs further discussion.

To us, in R18 multi-Rx chains WI, RAN4 should only define the requirement for the case when two signals with two different QCL Type D RSs are received by using two active UE Rx antenna modules. For the case when UE uses one antenna module to receive the signals, the legacy requirements will be applied by default (E.g., one beam is applied at a time and sharing factor should be considered). And how to identify the case is two active antenna modules or one active antenna module will depend on the applicability condition which will be concluded in RF session and up to UE implementation.

Proposal 1: In R18 multi-Rx chains WI, RRM session deprioritize the case when two signals are received by one active UE Rx antenna module at a time, before any conclusion is reached in RF session.

### 2.1.2 Beam management

In previous meeting, some companies thought the principle of independent beam management (IBM) can be reused for R18 multi-Rx chains WI. The corresponding discussion [2] is provided below.

#### Issue 1-2-3: Beam management related - Proposals - Option 1(LG): - Do not consider IBM principles or requirements in this WI - Option 2(Nokia): - RAN4 to define requirements assuming independent beam management across multiple Rx chains on the same carrier. - Option 3(ZTE): - Under the architecture of 2 RF chains+2 Antenna panels, each RF chain can determine the phase shifter independently, so the independent phase shift of H/V polarization can be assumed for each panel. Multi-Rx UE shall be capable of independent beam management between multiple Rx chains on a Component Carrier (CC). - Recommended WF - Needs further discussion.

As other companies’ comment in previous meeting, IBM is for inter band CA scenario. However, the scenario considered in R18 multi-Rx chains WI is single component carrier. IBM basically supports simultaneous UL/DL with independent RF chains even if they are in the same panel. This is already a very fundamental different to the multi-RX RF architecture. So, based on this observation, we do not think the same logic as IBM can be reused for R18 multi-Rx chains WI.

Proposal 2: Not to use the principle of independent beam management to define the requirement for R18 multi-Rx chains WI.

### 2.1.3 UE architecture

The corresponding discussion [2] is provided below.

#### Issue 1-2-2: UE architectures - Proposals - Option 1(MediaTek): - Multi-RX chain architecture should be discussed in RF session. - Option 2(Nokia): - Multi Rx architecture to assume that each Rx chain can process at an independent FFT window, and independent time tracking per Rx chain. - Multi Rx architecture to assume that each Rx chain can perform independent demodulation and independent RRM measurements. - Option 3(vivo): - It is not necessary to have a general conclusion on multi-Rx chain architecture in RRM session. - Recommended WF - Needs further discussion.

Some companies suggest to have an explicit UE architecture assumption in RRM session to define RRM requirements. However, to our understanding, it should be a RF issue. So, our suggested proposal is as below.

Proposal 3: Multi-Rx chains architecture should be discussed in RF session.

## 2.2 Scenario

In this topic, following four issues are further studied.

- Multiple component carriers

- Inter-cell mTRP operation

- Signal type

- Detectable condition

### 2.2.1 Multiple component carriers

RAN4 has discussed this issue for several meetings, according to the discussion [2] as below, RAN4 agreed to focus on a single CC operation and FFS CA operation in the last meeting.

#### Issue 1-1-2: Single (component) carrier for defining RRM requirements Agreements in the last meeting: RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier. - FFS whether UE can be configured with multiple component carriers, including intra-band CCs and/or inter-band CCs, but multi-Rx chain is enabled on only one of the component carriers. - Proposals - Option 1(Apple): - RAN4 should focus on single-carrier mode at this stage. When all the requirements are completed, RAN4 can discuss DC/CA cases - Option 2(Samsung): - RAN4 shall allow the case that UE can be configured with multiple component carriers, but multi-Rx chain is enabled only on one of the CCs. - Option 3(MediaTek): - In R18 multi-Rx chains WI, to deprioritize the case when UE is configured with multiple component carriers until the requirement of single component carrier is complete - Option 4(vivo): - Multi-Rx can also be enabled on a single FR2 component carrier in FR1+FR2 CA/DC deployment. - Option 5(Ericsson): - RAN4 shall focus on defining requirements for the single-carrier case where the UE is not configured with CA. - Recommended WF - Needs further discussion.

To us, there is a lot of issues to be discussed so far, e.g., L1-RSRP measurement, dual TCI state switch and some basic assumptions. So, we suggest deprioritizing the case when UE is configured with multiple component carriers, but multi-Rx chains are enabled on only one of the component carriers until the requirement of single component carrier is complete.

In terms of intra-band CCs and/or inter-band CCs, this also impacts on RF architecture. To enable one of intra-band CCs may need a different RF capability due to the CCs of multi-RX may share the same RF chain with the other CCs in the same band. This is a kind of extend the WI scope of Multi-Rx requirement and lead more RF impact. As there’re lots of works need to be specified for multi-RX, we suggest deprioritizing CA until basic requirement in single CC are done.

Proposal 4: In R18 multi-Rx chains WI, deprioritize the case when UE is configured with multiple component carriers until the requirement of single component carrier is complete or RF has some conclusion for this issue.

### 2.2.2 Detectable condition

As below, there is one open issue regarding detectable condition in the last meeting.

#### Issue 1-3-3: Detectable condition of RS signals - Proposals - Option 1(MediaTek): - For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period. - Recommended WF - Needs further discussion.

UE may take the other RS (E.g., SSB) which is in the same TCI chain as the tracking RS to perform UE Rx beam selection. Therefore, it would be more reliable if all RSs in the same TCI chain remain detectable during the entire measurement period. Furthermore, as same QCL Type-D means that the 2 RS are transmitted from the same Tx spatial filter, we tend to believe that the detectability should be the same for all RS in the same TCI chain. So, the following proposal is suggested.

Proposal 5: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period.

### 2.2.3 Power saving related requirements

The corresponding discussion [2] is provided below.

#### Issue 1-2-5: Whether and how to define power saving related requirements - Proposals - Option 1(Apple): - RAN4 agrees to allow a UE capable of multi-RX reception to inform the network that it does not support two-Aloe reception, so the network knows the UE does not turn on or off this capability arbitrarily. FFS how this is achieved. - Option 2(Intel): - RAN4 to discuss whether to consider panel activation issues, e.g., UE initiated or NW initiated, panel status and panel activation delay, etc. - Option 3(LG): - No need to specific power saving RRM requirements - But RAN4 to discuss whether and how to inform to network for UE power saving behavior - The UE power saving behavior is that the UE capable of multi-Rx reception turns on (multi-Rx chain) or off (single Rx chain) the multi-Rx chain arbitrarily. - Option 4(Nokia): - RAN4 to study if use of single Rx chain or multiple Rx chain can be adapted over time. - A UE capable of multi-Rx reception and the network should be aware of when the UE is using 2 Rx chains for reducing measurement times or reducing scheduling restrictions or when it is using a single Rx chain. - RAN4 to discuss new signaling for UE indication of use of 2 Rx chains. - Option 5(vivo): - No power saving specific requirements are considered in the WI. - Option 6(Ericsson): - No new power saving requirements specific for multi-rx operation are considered in the WI. - Extend OverheatingAssistance mechanism to also cover multi-RX chain operation and further discuss the details for multi-RX chain ON/OFF mechanism based on OverheatingAssistance. - Option 7: - RAN4 to discuss whether UE shall always be prepared for simultaneous reception or UE is allowed to choose the best Rx beam for single TRP reception (e.g. mDCI). - Recommended WF - Needs further discussion.

As above to discuss whether UE shall always be prepared for Multi-Rx operation is not the scope of power saving requirement. There are many approaches for UE to tell network whether UE now has to fall back to single panel, e.g., through RI report, non-group based L1-RSRP report or OverheatingAssistance. Particularly, we can reuse legacy OverheatingAssistance mechanism to indicate NW to cancel the multi-RX operation by remove group-based configuration. E.g., If UE encounter overheating problem when perform simultaneous DL reception, it can notify NW about the reduced number of DL MIMO layer from 4 to 2 via OverheatingAssistance message. Then NW could stop simultaneous DL transmission further and even remove group-based configuration.

Proposal 6: No new mechanism is needed for UE to fallback from multi-Rx to single Rx.

## 2.3 signalling and UE capability

In this topic, following three issues are further studied.

- Capability simultaneousReceptionDiffTypeD

- Capability simultaneousRxDataSSB-DiffNumerology

### 2.3.1 Capability simultaneousReceptionDiffTypeD

There is one open issue regarding UE capability simultaneousReceptionDiffTypeD in the last meeting. The corresponding requirement is provided as below for reference.

Content extracted from TS 38.306 [3]

| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11]. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

To our understanding, this UE capability is for indicating support of two PDSCH with two different QCL Type D RSs. In previous meeting, a lot of proposals and discussion were provided, e.g., whether to reuse legacy signaling in R18, or introduce new capabilities for different RS types. However, we think the UE capability is highly depending on the basic assumption, e.g., which RS/scenario is considered. In addition, we believe that this WI will consider longer measurement period when UE receives simultaneous RS and RS and scheduling restriction when UE receive simultaneous RS and data. So, we think no need introduce new capability for multi-RX operation when consider RRM requirement.

Proposal 7: Given simultaneousReceptionDiffTypeD-r16, there is no need to specify new capability simultaneous reception for multi-RX operation for RRM requirement.

### 2.3.2 Capability simultaneousRxDataSSB-DiffNumerology

There is another one open issue regarding UE capability simultaneousRxDataSSB-DiffNumerology in the last meeting. The corresponding requirement is provided as below for reference.

Content extracted from TS 38.306 [3]

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

In legacy RRM requirement, this UE capability is used only for FR1 when UE considers measurement/scheduling restriction. In FR2, we will additionally consider the rough/fine beam difference when receiving SSB and data simultaneously. The scheduling restriction is applied regardless of same or different SCSs. As this WI [4] is target at FR2 UE, we think no need to specify new capability for multi-RX chain in FR2 when consider different RS/scenario.

Proposal 8: No need to specify new mix-numerology capability for multi-RX chain in FR2 when consider simultaneous SSB and data reception.

In addition to the simultaneousRxDataSSB-DiffNumerology, the discussion of simultaneousRxDataSSB-DiffNumerology-Inter-r16 was also triggered in the last meeting. The corresponding requirement is provided as below for reference.

Content extracted from TS 38.306 [3]

| simultaneousRxDataSSB-DiffNumerology-Inter-r16 Indicates whether the UE supports concurrent SSB based inter-frequency measurement without measurement gap on neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. UE indicates support of this indicates support of interFrequencyMeas-NoGap-r16. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range where the SSB and PDCCH/PDSCH are received. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

According to above requirement, this UE capability is used for inter-frequency, which belongs to L3 measurement. Therefore, we believe the discussion is not in the scope of this WI.

Proposal 9: No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI.

### 2.3.3 Capability for supporting RTD > CP

The corresponding discussion [2] is provided below.

#### Issue 1-4-6: UE capability for supporting RTD>CP - Proposals - Option 1(Nokia): - RAN4 to reuse UE capability to be defined as part of NR_MIMO_evo_DL_UL for UE capable of supporting RTD>CP as part of the multi-Rx work. - Recommended WF - Needs further discussion.

From summary report [5]NR_MIMO_evo_DL_UL, RAN1 sent LS (R1-2205593) to RAN4 in RAN1#109 meeting. In RAN4#104-bis-e meeting, the reply LS (R4-2217279) is approved and sent to RAN1.

In the reply LS:

RAN4 thanks RAN1 for the LS on maximum uplink timing difference between the two TAs for multi-DCI multi-TRP with two TAs. After RAN4 further discussion, following values are agreed as MTTD values. For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs. For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.

Below agreement is on WF [6] R18 NR MIMO RRM requirements. For the UE do not support RTD > CP, MRTD = CP.

<Agreement > Issue 1-1-2: How to specify new MRTD requirements for UE not supporting RTD>CP? Agreement: - For UE not supporting RTD>CP MRTD = CP

Based on above discussion, we know RTD > CP could be as an optional UE capability. However, we want to emphasize that the WI objective of MIMO evo is different to this Multi-RX WI.

In Multi-RX, we focus on DL 4 layer MIMO from two direction of different QCL type D from multiple TRP.

In MIMO evo, we don’t mention if UE should support UL 4 layer MIMO, at least for PC3. In addition, the scenarios/assumption are different between Multi-RX and MIMO evo. E.g., MIMO evo may support both intra-cell and inter-cell mTRP but Multi-RX only support intra-cell mTRP in R18. Therefore, we prefer not to discuss RTD > CP for this WI in R18, as below proposal.

Proposal 10: Not to discuss RTD > CP for multi-RX WI in R18.

## 2.4 L3 measurement

According to the latest WID [4] as below, our understanding is RAN4 should focus on non L3 measurement related requirement until any update in RAN #100.

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTEs: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited. - The work on L3 measurement related aspects for scheduling/measurement restriction requirements is not precluded. - Further check in RAN #100 whether to include other L3 measurement related aspects and objectives subject to RAN4 progress.

We think L1 measurement related requirement is the high priority task for multi-Rx WI. So, L3 related measurement requirement could be discussed until L1 measurement requirement is done. The following proposal is suggested.

Proposal 11: To deprioritize L3 measurement related requirement until L1 measurement requirement is done.

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: In R18 multi-Rx chains WI, RRM session deprioritize the case when two signals are received by one active UE Rx antenna module at a time, before any conclusion is reached in RF session.

Proposal 2: Not to use the principle of independent beam management to define the requirement for R18 multi-Rx chains WI.

Proposal 3: Multi-Rx chains architecture should be discussed in RF session.

Proposal 4: In R18 multi-Rx chains WI, deprioritize the case when UE is configured with multiple component carriers until the requirement of single component carrier is complete or RF has some conclusion for this issue.

Proposal 5: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period.

Proposal 6: No new mechanism is needed for UE to fallback from multi-Rx to single Rx.

Proposal 7: Given simultaneousReceptionDiffTypeD-r16, there is no need to specify new capability simultaneous reception for multi-RX operation for RRM requirement.

Proposal 8: No need to specify new mix-numerology capability for multi-RX chain in FR2 when consider simultaneous SSB and data reception.

Proposal 9: No need to discuss simultaneousRxDataSSB-DiffNumerology-Inter-r16 in R18 multi-Rx chains WI.

Proposal 10: Not to discuss RTD > CP for multi-RX WI in R18.

Proposal 11: To deprioritize L3 measurement related requirement until L1 measurement requirement is done.

# Reference

- R4-2303302, WF on NR FR2 multi-Rx chain DL reception – Part 1, Vivo, RAN4 #106

- R4-2302767, Topic summary for [106][209] FR2_multiRx_part1, Vivo, RAN4 #106

- TS 38.306, NR; User Equipment (UE) radio access capabilities, 3GPP

- RP-223527, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated, RAN #98e

- R4-2302786, Topic summary for [106][228] NR_MIMO_evo_DL_UL

- R4-2303258, WF on R18 NR MIMO RRM requirements