3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2305166

Online, April 17 – April 26, 2023

Agenda Item: 5.8.3.6

Source: MediaTek Inc.

Title: Discussion on receive timing difference for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “receive timing difference”.

# Discussion

In previous meeting, a lot of discussions and proposals for timing difference were provided. Finally, MRTD < CP is agreed as a baseline to support multi-RX. And whether to support > CP is still FFS.

## 2.1 Define RRM requirement for MRTD handling

For MRTD < CP, this is agreed from last meeting as the following agreement [1].

MRTD Handling - Agreements in online session: - Define RRM requirements for MRTD < CP - FFS whether to support MRTD > CP and the discussion can take place after RAN4 #107 - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier

For the handling MRTD > CP, some companies proposed as below discussion [2].

#### Issue 3-2: Handling MRTD > CP - Proposals - Option 1: Do not discuss in this WI - Option 2: Introduce an optional capability - Option 3: Wait for RAN1 discussion on Rel-18 MIMO evolution until details are clear - Recommended WF - TBA There are different opinions on this issue and it seems difficult to achieve consensus without more discussion on several details

For this issue, we think the key point of whether UE can measure the signals with timing difference larger than on CP is not only related to the number of the UE Rx antenna modules. It also involves the number of FFT and timing estimation/tracking/compensation modules in the baseband. If UE wants to measure the signals by different antenna modules with timing difference larger than one CP, then the number of modules mentioned above should be doubled. Furthermore, 4-layer MIMO is not possible to be supported under large Rx timing difference between 2 TRPs.

observation 1: For timing difference between different antenna modules, the number of the FFT module, timing estimation/tracking/compensation will be doubled.

In addition, RAN4 agreed to focus on intra-cell mTRP in R18 multi-RX. Normally, MRTD > CP will not happen for intra-cell mTRP. We tend to believe MRTD > CP do not need to discuss in Multi-RX WI. The option 2 is likely follow the conclusion of MIMO evo WI, but there’re some different aspect between MIMO evo and Multi-RX discussion as below table:

| R18 WI | Whether support Intra or inter-cell mTRP? | MIMO layer | Whether support sDCI and mDCI? |
| --- | --- | --- | --- |
| Multi-RX | Only support intra-cell mTRP | Focus on DL 4 layers MIMO for PC3, and support 2 layers. | Both are supported from RAN4 agreement. |
| MIMO evo | Support both intra-cell and inter-cell mTRP | Support UL 2 or 4 layers MIMO, but not mention if support UL 4 layers MIMO for PC3. The scenarios of MIMO evo is still unclear. | Both are supported from RAN1 agreement. |

observation 2: Normally, MRTD > CP will not happen for intra-cell mTRP and prefer not to follow the conclusion of MIMO evo until they have more clear scenarios or conclusion.

Base on observation 1 and 2, we provide below proposal:

Proposal 1: Do not consider MRTD > CP in this WI until MIMO evo has some conclusion could be considered in the scope of R18 Multi-RX.

# Summary

In this paper, the discussion of FR2 multiple panels is provided. We have the following proposal:

observation 1: For timing difference between different antenna modules, the number of the FFT module, timing estimation/tracking/compensation will be doubled.

observation 2: Normally, MRTD > CP will not happen for intra-cell mTRP and prefer not to follow the conclusion of MIMO evo until they have more clear scenarios or conclusion.

Proposal 1: Do not consider MRTD > CP in this WI until MIMO evo has some conclusion could be considered in the scope of R18 Multi-RX.

# Reference

- R4-2303317, WF on NR FR2 multi-Rx chain DL reception – Part 2, Vivo, RAN4 #106

- R4-2302768, Topic summary for [106][210] FR2_multiRx_part2, Qualcomm, RAN#106