3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305175

Electronic meeting, 17 April – 26 April 2023

Agenda Item:	6.8.7.1

Source:	Ericsson

Title:	Discussion on eMTC/NB-IoT UE demodulation requirements for NTN

Document for:	Discussion

# 1	Introduction

RAN4#106 agreed with the way forward on NB-IoT/eMTC UE demodulation requirements for NTN [1]. This contribution discusses the remaining open issues based on our updated simulation results.

# 2	Discussion

## 2.1	Cat-M1 for NTN

### 2.1.1	Simulation results

Table 1 shows the common parameters for PDSCH demodulation requirements for NTN, and Table 2 shows the test parameters for each test case and simulation results. Table 3 shows the FRCs agreed in [1].

Table 1	Common parameters for PDSCH demodulation requirements.

| Parameters | Value |
| --- | --- |
| CBW | FDD 1.4MHz |
| Transmission mode | TM1 |
| Antenna configuration | 1x1 |
| K_offset | 8ms |

Table 2	Test parameters for PDSCH demodulation requirements.

| Test number | MCS | PDSCH repetition | Propagation condition | SNR (dB) to achieve 70% of maximum throughput |
| --- | --- | --- | --- | --- |
| 1 | 16QAM 1/2 | 1 | NTN-TDLC5-30 | 7.2 |
| 2 | QPSK 1/3 | 8 | NTN-TDLA100-200 | -7.4 |
| 3 | QPSK 1/10 | 64 | NTN-TDLA100-10 | -17.7 |

Table 3	FRCs used for Cat-M1 PDSCH simulation.

| MCS | Number of RBs | Channel bits (bits) | TBS (bits) |
| --- | --- | --- | --- |
| 16QAM 1/2 | 3 | 1656 (cf. R.79 FDD: 1584) | 744 |
| QPSK 1/3 | 6 | 1656 (cf. R.80 FDD: 1440) | 504 |
| QPSK 1/10 | 6 | 1656 (cf. R.81 FDD: 1584) | 152 |
| Note: CFI=2, 1 CRS port, no DRMS |  |  |  |

### 2.1.2	DL Scheduling for Cat-M1 PDSCH demodulation requirements

RAN4#106 discussed the detailed DL scheduling pattern for Cat-M1 UE demodulation requirements for NTN [1].

Issue 3-4: DL scheduling for Cat-M1 PDSCH demodulation requirements - Dropping/puncturing MPDCCH/PDSCH when MPDCCH/PDSCH subframe collides with PSS/SSS/PBCH/SI subframe

#### 2.1.2.	Assumption of common control channels/signals

According to RAN1 specification, PSS/SSS is transmitted in SF#0 and SF#5 every radio frame, and PBCH is transmitted in SF#0 every radio frame. Since we assume eNB is operated in the dedicated spectrum for eMTC NTN, eNB does not need to schedule SIB1 in SF#5 every radio frame, but instead eNB needs to schedule SIB1-BR. According to TS36.211 Table 6.4.1-1, SIB1-BR is scheduled in SF#4 every other radio frame in the case the system bandwidth is less than 5MHz. RAN4#106 also discussed other essential system information blocks such as SIB2, SIB3, and probably SIB31. However the schedule of these SIBs are up to eNB implementation and will be scheduled less frequent compared with MIB/SIB1-BR, e.g., every 160ms or more.

Table 4	Summary of common control channels/signals and their scheduling

| Common control channels/signals | Scheduling |
| --- | --- |
| PSS/SSS | SF#0 and SF#5 every 10ms |
| MIB (PBCH) | SF#0 every 10ms |
| SIB1-BR | SF#4 every 20ms Even SFN if mod(cell ID, 2) = 0 Odd SFN if mod(cell ID, 2) = 1 |
| SIB2, SIB3, SIB31, … | Up to network implementation, e.g., 160ms. |

#### 2.1.2.2	DL/UL scheduling for NTN Cat-M1 Test 1

Figure 1 illustrates our DL/UL scheduling proposal for Test 1, considering the agreements in [1]. The difference between our proposal and [1] is our proposal schedules MPDCCH in SF#6 and SF#7, and corresponding PDSCH in SF#8 and SF#9 every 10 subframes. On the other hand, [1] schedules MPDCCH in SF#1 and corresponding PDSCH in SF#3 every 10 subframes. The motivation of our proposal is 1) to schedule two PDSCHs in 10ms and 2) to avoid the conflicts with SIB1-BR in SF#4. This scheduling makes the test time short.

Figure 1	MPDCCH/PDSCH scheduling for Cat-M1 PDSCH demodulation test 1 (no repetitions).

Proposal 1: For Cat-M1 PDSCH test 1, add the following note in FRC:

- The downlink subframes are scheduled at the 8th and 9th subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe).

#### 2.1.2.2	DL/UL scheduling for NTN Cat-M1 Test 2

Figure 2 illustrates our DL/UL scheduling proposal for Test 2, which is basically same as the agreements in [1].

Figure 2	MPDCCH/PDSCH scheduling for Cat-M1 PDSCH demodulation test 2 (8 repetitions).

Proposal 2: For Cat-M1 PDSCH test 2, add the following note in FRC:

- PDSCH subframes are scheduled at the 10th to 17th subframes every period=32 ms. Information bit payload is available at the 10th to 17th subframes with repetition. (Starting from the 0th subframe)

#### 2.1.2.3	DL/UL scheduling for NTN Cat-M1 Test 3

Figure 3 illustrates our DL/UL scheduling proposal for Test 3, which is also the same as the agreements in [1], but MPDCCH transmission start from SF#1, although [1] starts MPDCCH transmission from SF#31. The reason we propose to transmit MPDCCH from SF#1 is that our proposal does not exactly assume other SIBs such as SIB2, SIB3, and SIB31. We understand it is necessary to schedule SIB2/SIB3/SIB31 during the test, however we think it is up to RAN5 how to schedule these SIBs, i.e., what SIBs are scheduled how often, as same as other RAN4 UE demodulation requirements. Therefore we don’t think RAN4 FRC need to consider other SIBs explicitly.

Figure 3	MPDCCH/PDSCH scheduling for Cat-M1 PDSCH demodulation (64 repetitions).

Proposal 3: For Cat-M1 PDSCH test 3, add the following note in FRC:

- PDSCH subframes are scheduled at the 66th to 129th subframes every period=160 ms. Information bit payload is available at the 66th to 129th subframes with repetition. (Starting from the 0th subframe)

## 2.2	Cat-NB1/NB2 for NTN

### 2.2.1	Simulation parameters

Table 5 shows the common parameters for NPDSCH demodulation requirements and Table 6 shows the test parameters for each test case. Table 7 shows the FRCs agreed in [1].

Table 5	Common parameters for NPDSCH demodulation requirements.

| Parameters | Value |
| --- | --- |
| CBW | FDD 200kHz |
| Operation mode | Stand-alone |
| Antenna configuration | 1x1 |
| K_offset | 8ms |

Table 6	Test parameters for NPDSCH demodulation requirements

| Test number | MCS | PDSCH repetition | Propagation condition | Carrier type | SNR (dB) to achieve 70% of maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | QPSK 1/2 | 32 | NTN-TDLC5-200 | Anchor | -4.1 |
| 2 | QPSK 1/3 | 128 | NTN-TDLA100-10 | Non-anchor | -12.7 |

Table 7	FRCs used for Cat-NB1/NB2 NPDSCH simulation.

| MCS | ITBS / ISF | Channel bits (bits) per SF | TBS (bits) |
| --- | --- | --- | --- |
| QPSK 1/2 (R.NB.6 FDD) | 9 / 3 | 320 | 616 |
| QPSK 1/3 (R.NB.6-1 FDD) | 6 / 3 | 320 | 392 |

# 4	Summary

Proposal 1: For Cat-M1 PDSCH test 1, add the following note in FRC:

- The downlink subframes are scheduled at the 8th and 9th subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe).

Proposal 2: For Cat-M1 PDSCH test 2, add the following note in FRC:

- PDSCH subframes are scheduled at the 10th to 17th subframes every period=32 ms. Information bit payload is available at the 10th to 17th subframes with repetition. (Starting from the 0th subframe)

Proposal 3: For Cat-M1 PDSCH test 3, add the following note in FRC:

- PDSCH subframes are scheduled at the 66th to 129th subframes every period=160 ms. Information bit payload is available at the 66th to 129th subframes with repetition. (Starting from the 0th subframe)

# References

- R4-2302932, “WF for UE demodulation requirements of IoT over NTN”, MediaTek.