3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305177

Electronic meeting, 17 April – 26 April 2023

Agenda Item:	6.8.7.2

Source:	Ericsson

Title:	Simulation results of SAN demodulation requirements for IoT-NTN

Document for:	Information

# 1	Introduction

RAN4#106 agreed with the way forward on SAN demodulation requirements for IoT over NTN [1], and agreed with the initial simulation assumption. This contribution provides our initial simulation results according to the simulation assumption in the way forward.

# 2	Doppler shift drift

In our simulation we assumed the fixed Doppler shift drift assuming the largest accumulated offset considering the worst case scenario as shown in Table 1, Table 2, and Table 3, where we assumed 0.27 ppm/ms (Table 4.2-2 LTO-600 in [2]) with the carrier frequency of 2.0GHz (=0.54Hz/ms).

Table 1	Doppler shift offset for eMTC.

| Physical channel | Tx duration (ms) per repetition | Number of repetitions | Applied Doppler shift offset (Hz) |
| --- | --- | --- | --- |
| PUSCH CE Mode A | 1 | 1 | 0.54 |
| PUSCH CE Mode A | 1 | 8 | 4.32 |
| PUSCH CE Mode B | 1 | 256 | 138.24 |
| PUCCH format 1a | 1 | 8 | 4.32 |
| PRACH BF0/BF1 | 1 | 8 | 4.32 |
| PRACH BF2 | 2 | 16 | 17.28 |
| PRACH BF3 | 3 | 16 | 25.92 |

Table 2	Doppler shift offset for NPUSCH formats 1 and 2.

| Physical channel | Tx duration (ms) per RU | Number of RUs | Number of repetitions | Applied Doppler shift offset (Hz) |
| --- | --- | --- | --- | --- |
| NPUSCH format 1, 3.75kHz | 32 | 2 | 16 | 552.96 |
| NPUSCH format 1, 15kHz | 1 | 1 | 16 | 8.64 |
| NPUSCH format 2, 3.75kHz | 8 | 1 | 16 | 69.12 |
| NPUSCH format 2, 15kHz | 2 | 1 | 16 | 17.28 |

Table 3	Doppler shift offset for NPRACH.

| Physical channel | Tx duration per symbol group (ms) | Number of symbol groups (P) | Number of repetitions | Applied Doppler shift offset (Hz) |
| --- | --- | --- | --- | --- |
| NPRACH PF0 | 1.4 | 4 | 8 | 24.192 |
| NPRACH PF0 | 1.4 | 4 | 32 | 96.768 |
| NPRACH PF1 | 1.6 | 4 | 8 | 27.648 |
| NPRACH PF1 | 1.6 | 4 | 32 | 110.592 |

# 3	SAN demodulation requirements for eMTC

## 3.1	PUSCH demodulation requirements

Table 4	Simulation parameters for PUSCH.

| Parameters | CE Mode A | CE Mode B |
| --- | --- | --- |
| Max number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 8 | 256 |
|  | FFS: 1 |  |
| Frequency hopping | OFF | OFF |
| Duplex mode | FDD only | FDD only |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz | NTN-TDLA100-X, X = [5, 200] Hz |
| FRC | TS36.104 A3-2 | TS36.104 A3-1 |
| System BW | 1.4MHz only | 1.4MHz only |
| Doppler shift offset | FFS | FFS |
| Transmit timing offset | FFS | FFS |
| Fraction of maximum throughput | 70% | 70% |

Table 5	Simulation results for PUSCH CE Mode A.

| Rx antenna | Repetitions | Propagation condition | SNR: without frequency offset | SNR: with frequency offset drift |
| --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-5 | 1.8 | 1.8 |
| 1 | 1 | NTN-TDLA100-200 | 2.6 | 2.6 |
| 1 | 1 | NTN-TDLC5-5 | -0.5 | -0.5 |
| 1 | 1 | NTN-TDLC5-30 | -0.4 | -0.4 |
| 1 | 8 | NTN-TDLA100-5 | -6.1 | -6.1 |
| 1 | 8 | NTN-TDLA100-200 | -7.7 | -7.7 |
| 1 | 8 | NTN-TDLC5-5 | -6.1 | -6.2 |
| 1 | 8 | NTN-TDLC5-30 | -5.8 | -5.8 |
| 2 | 1 | NTN-TDLA100-5 | -2.7 | -2.7 |
| 2 | 1 | NTN-TDLA100-200 | -1.9 | -1.9 |
| 2 | 1 | NTN-TDLC5-5 | -4.0 | -4.0 |
| 2 | 1 | NTN-TDLC5-30 | -4.1 | -4.1 |
| 2 | 8 | NTN-TDLA100-5 | -10.5 | -10.5 |
| 2 | 8 | NTN-TDLA100-200 | -10.9 | -11.0 |
| 2 | 8 | NTN-TDLC5-5 | -9.2 | -9.2 |
| 2 | 8 | NTN-TDLC5-30 | -9.1 | -9.1 |

Table 6	Simulation results for PUSCH CE Mode B.

| Rx antenna | Repetitions | Propagation condition | SNR: without frequency offset | SNR: with frequency offset drift |
| --- | --- | --- | --- | --- |
| 1 | 256 | NTN-TDLA100-5 | -15.1 | -15.1 |
| 1 | 256 | NTN-TDLA100-200 | -16.2 | -16.1 |
| 2 | 256 | NTN-TDLA100-5 | -17.4 | -17.4 |
| 2 | 256 | NTN-TDLA100-200 | -17.9 | -17.9 |

## 3.2	PUCCH demodulation requirements

### 3.2.1	ACK missed detection requirements for single user PUCCH format 1a

Test metric:

- The ACK missed detection probability shall not exceed 1%.

Table 7	Simulation parameters for PUCCH format 1a.

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz |
| System BW | 1.4MHz |

Table 8	Simulation results for PUCCH format 1a.

| Rx antenna | Repetitions | Propagation condition | SNR: without frequency offset | SNR: with frequency offset drift |
| --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | -5.4 | -5.4 |
| 1 | 8 | NTN-TDLA100-200 | -12.2 | -12.2 |
| 1 | 8 | NTN-TDLC5-5 | -10.2 | -10.1 |
| 1 | 8 | NTN-TDLC5-30 | -12.2 | -12.1 |
| 2 | 8 | NTN-TDLA100-5 | -11.1 | -11.2 |
| 2 | 8 | NTN-TDLA100-200 | -15.5 | -15.5 |
| 2 | 8 | NTN-TDLC5-5 | -15.3 | -15.3 |
| 2 | 8 | NTN-TDLC5-30 | -16.0 | -16.0 |

## 3.3	PRACH demodulation requirements

### 3.3.1	PRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 9	Simulation parameters for PRACH preamble detection requirements.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz | 270Hz + X (X: See Sub-topic 2-2 Additional Doppler shift offset) | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

Table 10	Simulation results for PRACH preamble detection requirements.

| Rx antenna | Repetitions | Propagation condition | Burst format | SNR: with fix frequency offset: 270 Hz | SNR: with frequency offset drift: 270+X Hz |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | BF0 | -13.3 |  |
| 1 | 8 | NTN-TDLA100-5 | BF1 | -13.3 |  |
| 1 | 8 | NTN-TDLA100-200 | BF0 | -19.1 |  |
| 1 | 8 | NTN-TDLA100-200 | BF1 | -19.2 |  |
| 1 | 8 | NTN-TDLC5-5 | BF0 | -18.8 |  |
| 1 | 8 | NTN-TDLC5-5 | BF1 | -18.7 |  |
| 1 | 8 | NTN-TDLC5-30 | BF0 | -20.7 |  |
| 1 | 8 | NTN-TDLC5-30 | BF1 | -20.7 |  |
| 1 | 16 | NTN-TDLA100-5 | BF2 | -19.6 |  |
| 1 | 16 | NTN-TDLA100-5 | BF3 | -19.6 |  |
| 1 | 16 | NTN-TDLA100-200 | BF2 | -24.7 |  |
| 1 | 16 | NTN-TDLA100-200 | BF3 | -24.7 |  |
| 1 | 16 | NTN-TDLC5-5 | BF2 | -23.6 |  |
| 1 | 16 | NTN-TDLC5-5 | BF3 | -23.7 |  |
| 1 | 16 | NTN-TDLC5-30 | BF2 | -25.1 |  |
| 1 | 16 | NTN-TDLC5-30 | BF3 | -25.2 |  |
| 2 | 8 | NTN-TDLA100-5 | BF0 | -18.8 |  |
| 2 | 8 | NTN-TDLA100-5 | BF1 | -18.9 |  |
| 2 | 8 | NTN-TDLA100-200 | BF0 | -22.3 |  |
| 2 | 8 | NTN-TDLA100-200 | BF1 | -22.4 |  |
| 2 | 8 | NTN-TDLC5-5 | BF0 | -22.6 |  |
| 2 | 8 | NTN-TDLC5-5 | BF1 | -22.5 |  |
| 2 | 8 | NTN-TDLC5-30 | BF0 | -23.5 |  |
| 2 | 8 | NTN-TDLC5-30 | BF1 | -23.6 |  |
| 2 | 16 | NTN-TDLA100-5 | BF2 | -24.0 |  |
| 2 | 16 | NTN-TDLA100-5 | BF3 | -24.0 |  |
| 2 | 16 | NTN-TDLA100-200 | BF2 | -27.1 |  |
| 2 | 16 | NTN-TDLA100-200 | BF3 | -27.0 |  |
| 2 | 16 | NTN-TDLC5-5 | BF2 | -26.9 |  |
| 2 | 16 | NTN-TDLC5-5 | BF3 | -26.9 |  |
| 2 | 16 | NTN-TDLC5-30 | BF2 | -27.5 |  |
| 2 | 16 | NTN-TDLC5-30 | BF3 | -27.6 |  |

# 4	SAN demodulation requirements for NB-IoT

## 4.1	NPUSCH format 1 demodulation requirements

Table 11	Simulation parameters for NPUSCH format 1.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-1 | 16 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-5 | 16 | 70% |

Table 12	Simulation results for NPUSCH format 1.

| Rx antenna | Subcarrier spacing | Number of tones | Repetitions | Propagation condition | SNR: without frequency offset | SNR: with frequency offset drift |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 1 | 16 | NTN-TDLA100-1 | -7.6 | -7.3 |
| 1 | 3.75kHz | 1 | 16 | NTN-TDLA100-200 | -5.4 | -5.5 |
| 1 | 3.75kHz | 1 | 16 | NTN-TDLC5-1 | -9.5 | -9.5 |
| 1 | 3.75kHz | 1 | 16 | NTN-TDLC5-30 | -10.1 | -10.1 |
| 1 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -7.5 | -7.5 |
| 1 | 15kHz | 12 | 16 | NTN-TDLA100-200 | -2.3 | -2.3 |
| 1 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -10.0 | -10.0 |
| 1 | 15kHz | 12 | 16 | NTN-TDLC5-30 | -9.9 | -9.9 |
| 2 | 3.75kHz | 1 | 16 | NTN-TDLA100-1 | -10.4 | -10.4 |
| 2 | 3.75kHz | 1 | 16 | NTN-TDLA100-200 | -7.6 | -7.6 |
| 2 | 3.75kHz | 1 | 16 | NTN-TDLC5-1 | -11.8 | -11.8 |
| 2 | 3.75kHz | 1 | 16 | NTN-TDLC5-30 | -12.1 | -12.0 |
| 2 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -11.7 | -11.7 |
| 2 | 15kHz | 12 | 16 | NTN-TDLA100-200 | -6.3 | -6.3 |
| 2 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -13.1 | -13.1 |
| 2 | 15kHz | 12 | 16 | NTN-TDLC5-30 | -12.9 | -12.9 |

## 4.2	NPUSCH format 2 demodulation requirements

### 4.2.1	ACK missed detection requirements

Test metric:

- The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

- The ACK missed detection probability shall not exceed 1%.

Table 13	Simulation parameters for NPUCCH format 2.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 15KHz | 16 |

Table 14	Simulation results for NPUSCH format 2.

| Rx antenna | Subcarrier spacing | Repetitions | Propagation condition | SNR: without frequency offset | SNR: with frequency offset drift |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 16 | NTN-TDLA100-1 |  |  |
| 1 | 3.75kHz | 16 | NTN-TDLA100-200 |  |  |
| 1 | 3.75kHz | 16 | NTN-TDLC5-1 |  |  |
| 1 | 3.75kHz | 16 | NTN-TDLC5-30 |  |  |
| 1 | 15kHz | 16 | NTN-TDLA100-1 | 3.8 | 3.8 |
| 1 | 15kHz | 16 | NTN-TDLA100-200 | -3.8 | -3.8 |
| 1 | 15kHz | 16 | NTN-TDLC5-1 | -5.9 | -5.5 |
| 1 | 15kHz | 16 | NTN-TDLC5-30 | -8.9 | -9.2 |
| 2 | 3.75kHz | 16 | NTN-TDLA100-1 |  |  |
| 2 | 3.75kHz | 16 | NTN-TDLA100-200 |  |  |
| 2 | 3.75kHz | 16 | NTN-TDLC5-1 |  |  |
| 2 | 3.75kHz | 16 | NTN-TDLC5-30 |  |  |
| 2 | 15kHz | 16 | NTN-TDLA100-1 | -5.1 | -5.4 |
| 2 | 15kHz | 16 | NTN-TDLA100-200 | -7.0 | -7.0 |
| 2 | 15kHz | 16 | NTN-TDLC5-1 | -10.9 | -10.7 |
| 2 | 15kHz | 16 | NTN-TDLC5-30 | -11.9 | -12.0 |

## 4.3	NPRACH demodulation requirements

### 4.3.1	NPRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 15	Simulation parameters for NPRACH missed detection.

| Number of Tx | Number of Rx | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |
| 1 | 1 and 2 | 32 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |

Table 16	Simulation results for NPRACH missed detection.

| Rx antenna | Repetitions | Propagation condition | Preamble format | SNR: with fix frequency offset: 200Hz | SNR: with frequency offset drift: 200+X |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-1 | PF0 | 10.5 |  |
| 1 | 8 | NTN-TDLA100-1 | PF1 | 10.5 |  |
| 1 | 8 | NTN-TDLA100-200 | PF0 | does not achieve 99% |  |
| 1 | 8 | NTN-TDLA100-200 | PF1 | does not achieve 99% |  |
| 1 | 8 | NTN-TDLC5-1 | PF0 | 0.1 |  |
| 1 | 8 | NTN-TDLC5-1 | PF1 | 0.3 |  |
| 1 | 8 | NTN-TDLC5-30 | PF0 | -3.0 |  |
| 1 | 8 | NTN-TDLC5-30 | PF1 | -2.6 |  |
| 1 | 32 | NTN-TDLA100-1 | PF0 | 1.8 |  |
| 1 | 32 | NTN-TDLA100-1 | PF1 | 1.5 |  |
| 1 | 32 | NTN-TDLA100-200 | PF0 | does not achieve 99% |  |
| 1 | 32 | NTN-TDLA100-200 | PF1 | does not achieve 99% |  |
| 1 | 32 | NTN-TDLC5-1 | PF0 | -5.1 |  |
| 1 | 32 | NTN-TDLC5-1 | PF1 | -5.2 |  |
| 1 | 32 | NTN-TDLC5-30 | PF0 | -9.0 |  |
| 1 | 32 | NTN-TDLC5-30 | PF1 | -8.7 |  |
| 2 | 8 | NTN-TDLA100-1 | PF0 | 0.7 |  |
| 2 | 8 | NTN-TDLA100-1 | PF1 | 1.1 |  |
| 2 | 8 | NTN-TDLA100-200 | PF0 | does not achieve 99% |  |
| 2 | 8 | NTN-TDLA100-200 | PF1 | does not achieve 99% |  |
| 2 | 8 | NTN-TDLC5-1 | PF0 | -4.9 |  |
| 2 | 8 | NTN-TDLC5-1 | PF1 | -4.5 |  |
| 2 | 8 | NTN-TDLC5-30 | PF0 | -6.8 |  |
| 2 | 8 | NTN-TDLC5-30 | PF1 | -6.7 |  |
| 2 | 32 | NTN-TDLA100-1 | PF0 | -4.6 |  |
| 2 | 32 | NTN-TDLA100-1 | PF1 | -4.7 |  |
| 2 | 32 | NTN-TDLA100-200 | PF0 | does not achieve 99% |  |
| 2 | 32 | NTN-TDLA100-200 | PF1 | does not achieve 99% |  |
| 2 | 32 | NTN-TDLC5-1 | PF0 | -9.6 |  |
| 2 | 32 | NTN-TDLC5-1 | PF1 | -9.6 |  |
| 2 | 32 | NTN-TDLC5-30 | PF0 | -11.7 |  |
| 2 | 32 | NTN-TDLC5-30 | PF1 | -11.6 |  |

# References

- R4-2302933, “WF for SAN demodulation requirements for IoT over NTN”, Ericsson.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.1.0, 3GPP