3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2305203

Online, April 17th – 26th, 2023

Source:	Samsung

Title:	Discussion on implementation feasibility study of SBFD: FR1 BS Aspects

Agenda item:			5.20.2.2.1

Document for:	Discussion

# 1. Introduction

Rel-18 Study Item is approved on Study on evolution of NR duplex operation with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation. According to latest SID in [1], in this RAN1 led SI tasks for RAN4 scope are explicitly stated as below:

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In RAN4#104-bis-e, WF on SBFD feasibility study and RF impacts from BS aspects was agreed [4], which includes the further WF based upon the agreement on August meeting [2][3]. In RAN4#105, further agreements and way forwards have been achieved and captured in [6] and reply LS to RAN1 for interference modelling and subband configuration has been approved in [8].  In RAN4#106, WFs on SBFD BS feasibility study on the self-interference and co-channel inter-sector interference are approved in [9] and [10] respectively, and one reply LS to RAN1 on interference modelling has been approved in [11]. Accordingly, in this contribution, we would like to further provide our views on the feasibility and RF impact of SBFD from FR1 BS aspects.

# 2 SBFD Self-Interference Modeling

Based upon the RAN4 discussion over RAN4#104-e and RAN4#104-bis-e, the RSIC capability is broken down into four aspects: (1) spatial isolation; (2) frequency isolation; (3) beam nulling/isolation and (4) digital IC. And based upon the inputs from companies, the ranges for values of (1)-(4) are summarized in table 1 of reply LS [2]; however, the detailed ranges are the supersets of results provided from source companies which require further feasibility analysis. During RAN4#105, a more detailed RSIC analysis framework has been approved [6], and further agreements are achieved in the approved WF [9] and response LS to RAN1 [11].

## 2.1 Further Discussion on RSIC Capability for FR1 BS

Based on our hardware PoC and further analysis of the component RSIC capability, the FR1 RSIC budget calculation is further provided in Table 1 which is based on the agreed RSIC analysis framework.

Observation 1: Samsung’s input for RSIC budget calculation for FR1 BS is provided in Table-1.

Table 1: FR1 RSIC budget calculation Summary

| FR1 |  |  |  | Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 38 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD utilized | DPD utilized |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -81 dBm Note: provided by ①-②-③-⑨ dBm | -92 dBm Note: provided by ①-②-③-⑨ dBm | -106 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | 25 dBc | 25 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 0dBc | 0dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA | TBA | TBA |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61 dBm Note: provided by ①-③-④-⑤dBm | -72 dBm Note: provided by ①-③-④-⑤dBm | -86 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc | 25 dBc | 20 dBc |
|  |  | Frequency isolation techniques |  | Filtering | Filtering | N/A |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | -20dBm | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm | -176dBm | -218dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | N/A | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm Note: provided by ①-③-④-⑤-⑥dBm | -97 dBm Note: provided by ①-③-④-⑤-⑥dBm | -106 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | 20 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 150.0 dBc | 148.8 dBc | 147.0 dBc |
| Noise floor ⑩dBm |  |  |  | -95dBm/20MHz | -90dBm/20MHz | -87dBm/20MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -101 dBm | -96 dBm | -93dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 150 dBc | 134 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | DUD(40-20-40MHz) | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB | 5 PRB | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 20MHz | 20MHz |
| Others |  |  |  |  |  |  |

Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR1 BS.

# 3 Co-channel Inter-Subband gNB-gNB CLI Modeling

## 3.1 Existing RAN4 Agreement

Based on the approved WF [6] as below, there are two key issues to be further discussed for co-channel co-site inter-sector gNB-gNB CLI evaluation: (1) the analysis framework, and (2) the achievable antenna isolation in the case of co-site inter-sector gNB-gNB CLI.

Agreement: - FFS the analysis framework co-channel co-site inter-sector gNB-gNB CLI - FFS the RSIC analysis framework can be reused or not. - For co-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for: - The achievable coupling loss in the case of co-site inter-sector gNB-gNB - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss). - Information about the following aspects can be provided: - Operating band - BS class - Inter-sector distance - Details about isolation structure - Other site considerations

And based on approved WF [10], the following agreements are agreed:

Agreement Inter-sector isolation value range - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4: - FR1: 62-93dB with 75dB being typical values. - FR2: 75-98dB with 88dB being typical values. - Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. Further improvement over the spatial isolation is FFS. - In forthcoming meetings values for macro and other BS classes should be proposed Agreement Evaluation of inter-sector interference impacts - For co-channel co-site inter-sector inter-gNB CLI, RAN4 to reuse the self-interference analysis framework with revisited mitigation capabilities if found necessary: - FFS how much desense because of co-site inter-sector CLI - FFS 1dB desense (in additional to the self-interference) can be used as starting point for further study. - FFS the desense value contains the interference from both neighboring sectors or from only one neighboring sector. - The following analysis framework has been provided as an example. It is not agreed, but is provided as a reference for further discussion: FR1 (or FR2-1); Company-A. BS class; Wide Area BS. BS TX Power dBPTX = ① dBm; xxx dBm. Component capability and parameters; Frequency isolation at TX; Frequency isolation capability dBαSI-frequency, TX = ② dBc; xxx dBc. Frequency isolation techniques used; e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX. Spatial isolation; Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc; xxx dBc. Co-channel Co-site Inter-sector Spatial isolation techniques used; e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation. TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc; xxx dBc. DL EIRP impact due to beam nulling in TX sub-band. Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1). RF IC and other tech. (before LNA); RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc; xxx dBc. RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc; xxx dBc. RF IC techniques and other tech. (before LNA); e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA). Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA; xxx dBc. Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1). Blocker Suppression at RX; Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc; xxx dBc. Frequency isolation techniques; e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX. RX IMD; Rx IIP3 capability (dBm). Rx IM3 contribution (dBm). Other RX; Any other RX impacts if significant (e.g. ADC noise, phase noise etc.). Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2); xxx dBm. RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc; xxx dBc. RX sensitivity degradation caused by RX beam nulling; xxx dBc. Digital IC dBαSI-digtial = ⑦ dBc; xxx dBc. Overall RSIC capability dBRSIC (Note 1); xxx dBc. Noise floor ⑩dBm; xxx dBm/CBW. Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB); xxx dBm. Required RSIC budget (①-⑪dBc); xxx dBc. SBFD configuration. Guardband assumption (if exist). bandwidth over which suppression is achieved. Others. Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics..

## 3.2 Further Discussion on FR1

### 3.2.1 Discussion on analysis framework

Based on the agreement from RAN4#106, for co-channel co-site inter-sector inter-gNB CLI, RAN4 agree to reuse the self-interference analysis framework with revisited mitigation capabilities, and the level of desense which can be regarded as the acceptable interference level is still FFS.

It should be noted that for the inter-sector interference, the implementation feasibility study could involve more practical issues, e.g., the installation placement of sector antennas etc. From that perspective, to conclude the co-channel co-site inter-sector CLI will cause the desense above or below 1dB will be highly dependent on the detailed deployment scenario. Furthermore, since in both RAN1/4 evaluation, the co-channel co-site inter-sector interference has been evaluated and a rough 1dB (or XdB) desense to estimate its impact is not that necessary. We would like to propose that company can report XdB desense (relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, which contains the interference either from both co-site neighboring sectors or from one single sector.

Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors.

- There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

### 3.2.2 Co-channel co-site inter-sector gNB-gNB CLI

By using the detailed analysis framework provided in our paper [12], we provided the analysis for co-channel co-site inter-sector gNB-gNB CLI

Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR1 BS is provided in Table-2, in which the interference from one co-site sector can be suppressed to the level lower than noise floor by 5dB.

Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | [110] dBc if additional [10dB] contributed by BF considered |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 75dB for typical spatial isolation, additional 25dB by installing EM conjugated structure between sectors, and additional [10dB] contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case, but technical possible if spatial isolation is lower |
| Overall interference suppression capability dB(Note 1) |  |  |  | 148.8 dBc |
| Noise floor ⑩dBm |  |  |  | -95 dBm/CBW |
| Residual Interference budget with 1 dB desense target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

### 3.2.3 Antennal Isolation for Co-channel Co-site Inter-sector gNB-gNB CLI

Based on the RF simulation provided in the last meeting, the antenna isolation (with the achievable coupling loss) has been evaluated for the following scenario:

- 3 sector scenario is under consideration:

  - The angle between every two sectors’ boresight directions is 120 degree;

  - Sector antenna panel’s width is 180mm;

  - Between two sectors’s antenna panel:

    - The center to center distance is: 150mm;

    - The nearest distance between edge to edge is: 60mm;

  - Three antenna elements are used to form an antenn port.

  - 3.5GHz operating frequency with 100MHz bandwidth.

Figure-1: (Left figure) 3-sector scenario for co-channel co-site inter-sector antenna isolation study; 
(Right figure) top view for the 2-sector scenario.

Based on the observations given in [10], the RF simulation has shown the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI can be in the range of 62-93dB, depending on different antenna pair and co/cross-polarization. Accordingly, the following agreement has been reached in RAN4#106:

Agreement Inter-sector isolation value range - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4: - FR1: 62-93dB with 75dB being typical values. - FR2: 75-98dB with 88dB being typical values. - Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. Further improvement over the spatial isolation is FFS. - In forthcoming meetings values for macro and other BS classes should be proposed

Furthermore, as we proposed in the last meeting, the above spatial isolation values (typical value for 75dB) have not yet reflected EM conjugated structure we used in our testbed having two panels within a sector. In our testbed to evaluate self-interference within a sector, the EM conjugated structure can improve around 20~30dB additionally, i.e., without the EM conjugated structure, we observed 50~60dB antenna isolation value. We can anticipate the similar improvement if the EM conjugated structure is installed between two-sector antennas. Hence, the EM conjugated structure, we expect the achievable antenna isolation shall be improved by around 25dB.

Observation 3: Installing EM conjugated structure between sectors can provide additional inter-sector spatial isolation at the level of 25dB.

Furthermore, considering the distinct beamforming directions for different sectors, additional isolation from the suppression given by beamforming sidelobe can be added into spatial isolation. RAN4 can further study the value of this contribution from the suppression given by beamforming sidelobe, and we see [10]dB is feasible for FR1 BS implementation.

Observation 4: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR1 BS implementation.

As we provided in previous meeting, since the common DU can be used for co-site sectors, it is still possible to have digital IC to be used, which can provide additional interference suppression capability.

Observation 5: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference.

Moreover, the effect of the co-site inter-antenna interference has also been studied in WIMAX Forum [13], where they checked antenna isolation capability between two antennas, in which two antennas can be positioned horizontally or vertically and have different tilting angles. They concluded that

•	More than 2m horizontal spacing is required for the isolation to exceed 55dB

•	Positive rotation of boresight angle direction can improve the isolation by more than 10dB

•	Electrical tilt improves the isolation by 20dB at 4° downward

•	Vertical separation provides at least 70dB of isolation even in the case of 0m separation distance

•	Rotation of boresight angle direction can improve the isolation by only 10dB in 180° of boresight angle

•	Simultaneous electrical down-tilt of both antennas improves the isolation by more than 7dB at 4° downward

This result reveals that antenna isolation of two-sector antennas is able to increase by larger horizontal distance, or vertical antenna arrangement, or different boresight angle directions, or different electrical tilts or combination thereof.

Observation 6: There are many other deployment-relevant methods to further improve inter-sector antenna isolation, including:

- Larger horizontal distance between co-site sectors;

- Vertical antenna arrangement for co-site sectors;

- Different boresight angle directions, or different electrical tilts or combination thereof for co-site sectors.

Proposal 3: RAN4 shall confirm RAN1 that additional spatial isolation can be achieved for co-site inter-sector co-channel CLI, with typical value at least 35dB.

For other type of BSs, i.e., medium range and local area, we see no necessity to discuss the co-site inter-sector antenna isolation because the deployment is most likely not based on sector for these BS types.

Proposal 4: There is limited necessity to further discuss the co-site inter-sector antenna isolation for BS types other than marco BS.

# 4 Adjacent-channel gNB-gNB CLI Modelling

Based on the approved WF [R4-2214377] and WF [R4-2217464], the following RAN4 agreement is given:

Agreement on feasibility and how to model co-site gNB-gNB CLI modelling (R4-2214377) - Proposal: as no path loss model applicable this modelling could be different compared with inter-site gNB-gNB CLI modelling with below alternatives: - Alternative 1: ACLR and ACS based with potential other solution from SBFD capable gNB to cancel co-cite adjacent channel interference(i.e. ACLR from the SBFD gNB towards the victim or ACS impact from the aggressor towards the SBFD gNB) - A non-SBFD aggressor or victim in the adjacent channel should be assumed to have ACLR or ACS according to the RAN4 specifications - Note: RAN4 will further study the possibility of improved performance/requirements compared to existing referred requirements list above. - Alternative 2: similar modelling as for self-interference(RSI) can be applied but may with different parameters especially on antenna isolation and required overall isolation if both gNBs with SBFD capability - And digital IC is not feasible if gNBs belong to different operators for this case Agreement (R4-2217464) Adjacent-channel co-site inter-sector gNB-gNB CLI modelling - For adjacent-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for: - The achievable coupling loss in the case of co-site inter-sector gNB-gNB - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss). - Practical issues to achieve antenna isolation can be considered: e.g. increasing sector separation, mounting isolating materials on the site and the physical characteristics of such materials (size, weight etc.) - Clarification on the value discussed here: - the adjacent-channel co-site inter-sector gNB-gNB CLI discussed here is for the sum contributions from all co-site gNBs.

Based on the above RAN4 agreement, we can generally confirm the following method to derive αadj,co-site-x, which is aligned with current RAN4 co-existence study.

Proposal 5: For SBFD feasibility study, the interference suppression capability of co-site inter-sector adjacent-channel inter-subband CLI between the aggressor sector x and the victim sector can be modeled by

10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS

in which

  - ACLRBS and ACSBS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation.

  - spatial isolationdB is the corresponding spatial isolation between the aggressor sector x and the victim sector.

For the spatial isolation for co-site inter-sector adjacent-channel CLI modeling, based on the analysis in our accompanying paper on the interference modelling and reply LS under AI 5.20.2, we conclude the following proposal. In short, we expect the spatial isolation shall be larger for adjacent-channel CLI compared with co-channel case, which can be translated to 20-30dB higher spatial isolation to be added to the spatial isolation for co-channel inter-sector CLI.

Proposal 6: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

    - FR1: 87-118dB with 100dB being typical value.

For adjacent-channel inter-sector CLI, we also see the possibility to use EM conjugated structure to provide further spatial isolation in addition to the values given above. By arranging sector antennas for two adjacent channels to be deployed vertically, additional spatial isolation e.g., 25dB can be provided if the EM conjugated structure is installed in the middle.

Observation 7: For co-site inter-sector adjacent-channel CLI, additional spatial isolation can be provided by inserting the EM conjugated structure.

Furthermore, the possibility of applying RF interference cancellation is also possible for co-site inter-sector adjacent-channel CLI, as demonstrated in [R4-2300061], which is similar to self-interference RF cancellation. Based on the results provided therein, even with the isolation of 77.28dB, which we believe can be much improved if RF interference cancellation is applicable.

Observation 8: For co-site inter-sector adjacent-channel CLI, RF interference cancellation can provide additional isolation.

# 5. Conclusion

In this contribution, we provided our viewpoints on the feasibility and RF impact of SBFD from FR1 BS aspects, accordingly the following observations and proposals are obtained:

SBFD Self-Interference Modelling

Observation 1: Samsung’s input for RSIC budget calculation for FR1 BS is provided in Table-1.

Table 1: FR1 RSIC budget calculation Summary

| FR1 |  |  |  | Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 38 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD utilized | DPD utilized |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -81 dBm Note: provided by ①-②-③-⑨ dBm | -92 dBm Note: provided by ①-②-③-⑨ dBm | -106 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 25 dBc | 25 dBc | 25 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 0dBc | 0dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation | subband filtering or RF interference cancellation |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | TBA | TBA | TBA |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61 dBm Note: provided by ①-③-④-⑤dBm | -72 dBm Note: provided by ①-③-④-⑤dBm | -86 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc | 25 dBc | 20 dBc |
|  |  | Frequency isolation techniques |  | Filtering | Filtering | N/A |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm | -20dBm | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm | -176dBm | -218dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | N/A | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm Note: provided by ①-③-④-⑤-⑥dBm | -97 dBm Note: provided by ①-③-④-⑤-⑥dBm | -106 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 5 dBc | 5 dBc | 5 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Limited, ~0dB | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | 20 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 150.0 dBc | 148.8 dBc | 147.0 dBc |
| Noise floor ⑩dBm |  |  |  | -95dBm/20MHz | -90dBm/20MHz | -87dBm/20MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -101 dBm | -96 dBm | -93dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 150 dBc | 134 dBc | 117 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | DUD(40-20-40MHz) | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB | 5 PRB | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 20MHz | 20MHz |
| Others |  |  |  |  |  |  |

Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR1 BS.

Co-channel Inter-Subband gNB-gNB CLI Modelling

Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors.

- There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR1 BS is provided in Table-1, in which the interference from one co-site sector can be suppressed to the level lower than noise floor by 5dB.

- Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | [110] dBc if additional [10dB] contributed by BF considered |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 75dB for typical spatial isolation, additional 25dB by installing EM conjugated structure between sectors, and additional [10dB] contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -61dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 40 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -143dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -101 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case, but technical possible if spatial isolation is lower |
| Overall interference suppression capability dB(Note 1) |  |  |  | 148.8 dBc |
| Noise floor ⑩dBm |  |  |  | -95 dBm/CBW |
| Residual Interference budget with 1 dB desense target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | 20MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

Observation 3: Installing EM conjugated structure between sectors can provide additional inter-sector spatial isolation at the level of 25dB.

Observation 4: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR1 BS implementation.

Observation 5: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference.

Observation 6: There are many other deployment-relevant methods to further improve inter-sector antenna isolation, including:

- Larger horizontal distance between co-site sectors;

- Vertical antenna arrangement for co-site sectors;

- Different boresight angle directions, or different electrical tilts or combination thereof for co-site sectors.

Proposal 3: RAN4 shall confirm RAN1 that additional spatial isolation can be achieved for co-site inter-sector co-channel CLI, with typical value at least 35dB.

Proposal 4: There is limited necessity to further discuss the co-site inter-sector antenna isolation for BS types other than marco BS.

Adjacent-channel gNB-gNB CLI Modelling

Proposal 5: For SBFD feasibility study, the interference suppression capability of co-site inter-sector adjacent-channel inter-subband CLI between the aggressor sector x and the victim sector can be modeled by

10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS

in which

  - ACLRBS and ACSBS: RAN4 agree to apply gNB ACLR and gNB ACS minimum requirement according to the RAN4 specification as the baseline for SBFD evaluation.

  - spatial isolationdB is the corresponding spatial isolation between the aggressor sector x and the victim sector.

Proposal 6: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

    - FR1: 87-118dB with 100dB being typical value.

Observation 7: For co-site inter-sector adjacent-channel CLI, additional spatial isolation can be provided by inserting the EM conjugated structure.

Observation 8: For co-site inter-sector adjacent-channel CLI, RF interference cancellation can provide additional isolation.

# 6. Reference

[1] RP-222110, “Revised SID: Study on evolution of NR duplex operation”, CMCC, RAN#97.

[2] R4-2214376, “Reply LS on interference modelling for duplex evolution”, RAN4, Samsung, CMCC, Qualcomm, RAN4#104-e.

[3] R4-2214377, “WF on feasibility study from RF perspective”, Samsung, RAN4#104-e.

[4] R4-2217464, “WF on SBFD feasibility study and RF impact: BS aspect”, Samsung, RAN4#104-Bis-e.

[5] R4-2217513, “WF on SBFD feasibility study and RF impact: UE aspect”, Qualcomm, RAN4#104-Bis-e.

[6] R4-2220244, “WF for the feasibility from BS aspect”, Samsung, RAN4#105.

[7] R4-2220245, “WF for the feasibility from UE aspect”, Qualcomm, RAN4#105.

[8] R4-2220243, “LS response to RAN1 for interference modelling and Sub-band configuration”, RAN4, Samsung, RAN4#105.

[9] R4-2302883, “WF for Self-interference analysis from BS aspect for SBFD operation”, Samsung, RAN4#106.

[10] R4-2302922, “WF for Co-channel inter-sector interference analysis from BS aspect for SBFD operation”, Ericsson, RAN4#106.

[11] R4-2302885, “Reply LS on interference modelling”, RAN4, CATT, RAN4#106.

[12] R4-2302968, “Discussion on feasibility and RF impact of SBFD: BS Aspects”, Samsung, RAN4#106.

[13] https://files.wimaxforum.org/Document/Download/Managing_TDD-FDD_Interference_between_Co-Sited_Base_Stations_Deployed_in_Adjacent_Frequency_Blocks