3GPP TSG-RAN WG4 Meeting # 106bis					        	   R4-2305234

# Online, April 17 – April 26, 2023

Agenda Item:	5.9.2.3

Source: 	OPPO

Title: 	Discussion on other enhancements for FR2 Scell activation delay reduction

Document for:	Approval

1	Introduction

A WF on Rel-18 eFeRRM[1] was approved in RAN4#106 meeting [2]. In this contribution, we further discuss other potential enhancements for FR2 SCell activation delay reduction.

Table 1. Summary for candidate solutions for FR2 unknown SCell activation enhancement

| Enhancement part | Issue index in topic summary (R4-2302769) | Enhancement solution | RAN4 decision on whether to continue discuss this enhancement solution in R18? |
| --- | --- | --- | --- |
| Others | Issue 3-1-1 | SCell without SSB for inter-band FR2 unknown SCell activation | Deprioritized |
|  | Issue 3-1-2/ Issue 3-1-3/Issue 3-1-4 | SCell without SSB for inter-band FR1 unknown SCell activation | Deprioritized |
|  | Issue 3-1-5 | known/unknown condition enhancement for FR2 SCell activation | Deprioritized |
|  | Issue 3-1-6 | Scheduling enhancement for FR2 unknown SCell activation | Yes, continue discussion |
|  | Issue 3-1-7 | CBRA for FR2 unknown SCell activation | Deprioritized |

2	Discussion

Issue 3-1-6: Scheduling enhancement for FR2 unknown SCell activation

- Proposals

  - Option 1 (Apple): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

  - Option 2 (Nokia):

    - The potential to enable earlier data transmission within the activation period shall be studied.

    - The UE shall be scheduled by the network immediately after L1-RSRP reporting.

    - The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

In our understanding, if network wants to schedule this UE before valid CQI reporting, it’s possible and allowed, but whether or not this UE will response or react correctly to this scheduling is up to UE. For example, when the UE has DL timing and beam/TCI information, UE is able to be scheduled and such early scheduling is not precluded. As long as UE is ready on timing and beam, UE will receive such scheduled data to increase the throughput and reduce latency. Thus, it is up to network implementation and no enhancement is needed in the spec.

Proposal 1: No scheduling enhancement is needed for FR2 unknown SCell activation, which is up to network implementation.

3	Conclusion

Proposal 1: No scheduling enhancement is needed for FR2 unknown SCell activation, which is up to network implementation.

4	Reference

[1] R4-2203228, WF on R18 eFeRRM, Apple