3GPP TSG-RAN WG4 Meeting # 106bis					        	R4-2305235

# Online, April 17 – April 26, 2023

Agenda Item:	5.9.3

Source: 	OPPO

Title: 	Discussion on remaining issues of RRM requirements for FR1+FR1 NR-DC

Document for:	Approval

1	Introduction

A WF on Rel-18 R18 eFeRRM was approved in RAN4#106 meeting [1]. In this contribution, we discuss the leftover issues of RRM requirements for FR1-FR1 NR-DC.

| Potential RRM requirements for FR1+FR1 NR-DC | Impacted section/clause in TS 38.133 | Volunteer of draft CR |
| --- | --- | --- |
| Applicability rule | 3.6.2.4 | QC |
| PSCell addition/release delay and conditional PSCell addition delay | 8.2.4.1, 8.9.2 | vivo |
| Scheduling availability | 8.1.7, 8.5.7, 9.2.5 8.5.8, 9.5.6, | QC, OPPO, MTK, Nokia |
| CSSF | 9.1.5.1.4, 9.1.5.2.4 | OPPO |
| HO with PSCell | 6.1.5.1, 6.1.5.4 | Xiaomi |
| SCG activation/deactivation | 8.17.2 | Ericsson |

2	Discussion

SCG activation/deactivation

Issue 1-4-1: Tsearch for RACH-less SCG activation Agreement: - For RACH-less based SCG activation for FR1+FR1 NR-DC, - [if TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell] - FFS on requirements for additional scenarios including FR1 PSCell is unknown

For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥-2 dB, then Tsearch = 3* Trs ms.

For RACH-less SCG activation, we are open to further discuss if any progress of SCG activation was achieved.

TS 38.331 (clause 5.3.5): 3> else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message: 4> if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or 4> if lower layers indicate that a Random Access procedure is needed for SCG activation: 5> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; (=> RACH-based procedure) 4> else the procedure ends; (=> RACH-less procedure) 3> else the procedure ends;

Based on RACH-based and RACH-less procedure in TS 38.331 (clause 5.3.5), if UE conduct RACH-less procedure when SCG was deactived, two conditions should be fulfilled that bfd-and-RLM should be configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message, and lower layers does not indicate that a Random Access procedure is needed for SCG activation. Otherwise, UE initiate the Random Access procedure on the PSCell. Therefore, for RACH-less based PSCell activation, the conditions bfd-and-RLM was configured to true must be satisfied. There is no need to further clarify these in the requirements of RAN4 spec.

Proposal 1：For RACH-less based PSCell activation, the condition that bfd-and-RLM was configured to true can be removed.

In FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

However, for the classification of known/unknown PSCell and TCI state defined in RAN4, we think the condition should be reused for FR1+FR1 NR-DC. The side conditions of known/unknown PSCell and TCI state are different. If the target cell is a known FR1 PScell and TCI state is known, cell search can be skipped and Tsearch = 0 ms. For other cases, there are no requirements if FR1 PSCell is unknown and/or TCI state is unknown.

According to current definition of known/unknown TCI state, if UE detected beam failure, then TCI state is unknown.

If the UE is configured to perform BFD while the SCG is deactivated - The TCI state is known if the following conditions are met: - During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated, - UE has not detected beam failure - Otherwise, the TCI state is unknown.

We propose that the requirements for RACH-less based PSCell activation can be defined as:

For RACH-less based PSCell activation in FR1+FR1 NR-DC scenario, Tsearch = 0 ms if the target cell is a known FR1 PScell. There are no requirements if FR1 PSCell is unknown and/or TCI state is unknown..

Proposal 2: For RACH-less based PSCell activation in FR1+FR1 NR-DC scenario, Tsearch = 0 ms if the target cell is a known FR1 PScell and TCI state is known. For other cases, there are no requirements if FR1 PSCell is unknown or TCI state is unknown.

3	Conclusion

In this paper, we propose the following observations and proposals for RRM requirements of FR1+FR1 NR-DC.

Proposal 1：For RACH-less based PSCell activation, the condition that bfd-and-RLM was configured to true can be removed.

Proposal 2: For RACH-less based PSCell activation in FR1+FR1 NR-DC scenario, Tsearch = 0 ms if the target cell is a known FR1 PScell and TCI state is known. For other cases, there are no requirements if FR1 PSCell is unknown or TCI state is unknown.

4	Reference

[1] R4-2303304, WF on RRM Core requirements for FR1-FR1 NR-DC, OPPO