3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305246

# Online, April 17 – April 26, 2023

Agenda Item:	5.31.3.2

Source: 	OPPO

Title: 	Discussion on SL-U RRM requirements

Document for:	Approval

1	Introduction

During RAN-P #98-e meeting, the WID: NR sidelink evolution [1] was revised and RAN4 is supposed to work the corresponding requirements. Last RAN4 meeting had a general discussion on the scope [2] and this contribution will give our considerations on one of the objectives, SL-U.

2	Discussion

<Way forward >: RAN4 to continue discussion whether and how to define the following requirements considering SL-U operation - UE transmit timing - Initiation/Cease of SLSS transmission - Selection/Reselection of V2X synchronization reference source - L1 SL-RSRP measurement - Congestion control measurements - Interruption - Scheduling availability

As shown above, RAN4 will continue study the RRM impacts due to LBT mechanism on unlicensed band, and we will analysis them case by case.

In current TS 38.133, four timing error requirements are specified considering different synchronization reference sources. When GNSS, NR cell or E-UTRAN cell is used as synchronization reference source, the synchronization signals are from GNSS or Uu-link, which will not be impacted by SL-U. When SyncRef UE is used as synchronization reference source, the S-SSB transmitted from SyncRef UE may be postponed or dropped due to LBT failure. In NR-U, the SSB transmission from reference cell may be postponed due to LBT failure, but the same UE transmit timing requirement is applicable to both licensed carrier and unlicensed carrier subject to CCA as shown below. Then for SL-U, the legacy SL timing error requirements could also be reused as long as the S-SSB from SyncRef UE is available and detected.

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

Proposal-1: UE transmit timing error requirements for NR SL could be reused for SL-U.

For initiation/cease of SLSS transmission with SyncRef UE as synchronization reference, UE is required to evaluate and measure the PSBCH-RSRP within Tevaluate,SLSS, which is expected to be extended in case of LBT failure when the selected SyncRef UE transmitting S-SSB.

Similarly, during selection/reselection of V2X synchronization reference source, UE needs to detect or identify new SyncRef UE within Tdetect,SyncRefUE, and perform PSBCH-RSRP measurements for 3 identified SyncRef UEs within Tmeasure,PSBCH-RSRP. These two delay requirements should also be extended.

Proposal-2: Discuss how to update the following core requirements due to LBT failure:

- Tevaluate,SLSS for initiation/cease of SLSS transmission with SyncRef UE as synchronization reference

- Tdetect,SyncRefUE and Tmeasure,PSBCH-RSRP for selection/reselection of V2X synchronization reference source

The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2. The L1 SL-RSRP measurement period corresponds to one slot and the measurement shall meet the L1 SL-RSRP measurement accuracy requirement in Clause 10. After resource (re-)selection procedure, re-evaluation is performed on the reserved resources by L1 SL-RSRP measurements before transmission of SCI with reservation when the conditions specified in TS 38.214[26] are satisfied.

L1-RSRP is used to determine whether a resource is available during resource reselection and pre-emption procedure. And the corresponding measurement period is introduced, for example one slot for resource reselection procedure as shown above. However, the measurement period should not be extended since in our understanding, L1-RSRP is not specific for some certain reference signal or channel from a certain UE, but to derive the total power level. Even if transmission from one Tx UE is blocked due to LBT failure, the Rx UE could continue L1-RSRP measurement and the corresponding delay shall not be extended.

Observation-1: UE could continue L1-RSRP measurement regardless of LBT failures of other Tx UEs.

Proposal-3: L1-RSRP measurement period for NR SL could be reused for SL-U.

## 12.6 Congestion Control measurements The UE shall be capable of estimating the channel busy ratio for one or more transmission pools indicated by higher layers in TS 38.331[2], based on SL-RSSI measurements provided by the physical layer. When no sidelink transmissions occur, the UE physical layer shall perform a single-shot SL-RSSI measurement for each sub-channel included in all the slots configured as transmission pools. The SL-RSSI measurement performed according to this clause shall meet the SL-RSSI measurement accuracy requirements defined in Clause 10. The UE shall perform channel busy ratio (CBR) measurement based on SL-RSSI measurements as described in TS 38.215 [4].

For UEs that support NR Uu and sidelink operation, the UEs shall not send event C1 triggered measurement reports during T1 and shall send event C1 triggered measurement reports during T2. For UEs that support sidelink operation only, the UE channel occupancy ratio shall be larger than 0.001 during T1, and the UE channel occupancy ratio shall be smaller than 0.001 curing T2.

In legacy SL operation, congestion control mechanism is used for UE to estimate the channel busy ratio for one or more transmission pools. In SL-U, LBT mechanism is applied but the logic is different from congestion control. LBT is shot-term evaluation performed right before the transmission to make sure the transmission resource is not occupied; while congestion control is long-term evaluation to select a better resource. Therefore, congestion control could be decoupled with LBT in our view. Currently, the SL-RSSI measurement for congestion control is single-shot measurement and this could be reused for SL-U. Besides, the test case for congestion control may need to be revised. As shown above, the channel occupancy ratio (e.g. 0.001) may be relaxed due to LBT failure.

Observation-2: The logic of congestion control can be reused that UE estimates the channel busy ratio for one or more transmission pools based on SL-RSSI measurements.

Proposal-4: Update the test case for congestion control considering LBT failure in SL-U.

<Agreement >: Other issues due to LBT - FFS: Conditions when SLSS occasion can be considered as available or unavailable - FFS: UE behavior when experiencing large number of missing SLSS due to LBT failures

In the requirements of clause 9.2A, the term SMTC occasion not available at the UE refers to when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but NSSB candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding period, where: - For the cell detection procedure: NSSB is at least one candidate SSB position (NOTE: the one candidate SSB position for the cell detection shall not be impacted by the set of candidate SSB positions which are already being measured by the UE within the current measurement period of the on-going measurements), and - For other procedures in clause 9.2A: NSSB are the first two successive candidate SSB positions when two or more candidate SSB positions are configured for this SSB index in one discovery burst transmission window, otherwise NSSB is one candidate SSB position; otherwise the SMTC occasion is considered as available at the UE.

NR-U introduced the term “SMTC occasion not available” and interpreted it as “when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but Nssb candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding”. The interpretation of Nssb candidate SSB positions can be different for difference procedure as shown above. It means “at least one candidate SSB position” for the cell detection procedure, and “the first two successive candidate SSB positions” for other procedures in clause 9.2A. In SL-U, whether and how to define discovery burst transmission window and QCL relation are still open. At this stage, we can first reach a general consensus like SLSS occasion can be considered unavailable when N S-SSB positions are not available, and N candidate S-SSB positions should be further discussed for different procedure, including initiation/cease of SLSS transmission, detection and measurement period for Selection/Reselection of V2X synchronization reference source.

Observation-3: For the condition of “SLSS occasion is not available”, the definition of “Nssb candidate SSB positions for the same SSB index within certain transmission window are not available at the UE due to DL CCA failures” can be considered as the baseline for SL-U.

Proposal-5: The condition of “SLSS occasion is not available” can be defined when N S-SSB positions are not available, and FFS N for different procedures, including initiation/cease of SLSS transmission, detection and measurement period for Selection/Reselection of V2X synchronization reference source.

Table 9.2A.5.1-1: Time period for PSS/SSS detection (FR1)

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max( 600ms, ceil((5+LPSS/SSS) x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x (5+LPSS/SSS) x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil((5+LPSS/SSS) x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When DRX is not configured, LPSS/SSS is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When DRX is configured, LPSS/SSS is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_intra_CCA for PSS/SSS detection, where LPSS/SSS< LPSS/SSS,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 3: LPSS/SSS,max =7 for Max(DRX cycle,SMTC period)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,max =5 for 40ms<Max(DRX cycle,SMTC period)≤320ms, LPSS/SSS,max = 3 for DRX cycle>320ms. NOTE 4: Upon exceeding LPSS/SSS,max, the UE is not required to meet the requirements for PSS/SSS detection. |  |

When defining the measurement period for NR-U, the measurement period is extended based on the number of unavailable SMTC occasions, e.g. LPSS/SSS and Lind. Besides, the upper limit of unavailable SMTC occasion, e.g. LPSS/SSS max and Lind, are introduced. When the number of unavailable SMTC occasion exceeds the upper limit, UE is not required to meet the corresponding requirements.

Proposal-6: When the number of missing SLSS due to LBT failure is too large, UE is not required to meet the corresponding requirements.

3	Conclusion

This contribution gave our views on core requirements for SL-U and the following observations and proposals:

Observation-1: UE could continue L1-RSRP measurement regardless of LBT failures of other Tx UEs.

Observation-2: The logic of congestion control can be reused that UE estimates the channel busy ratio for one or more transmission pools based on SL-RSSI measurements.

Observation-3: For the condition of “SLSS occasion is not available”, the definition of “Nssb candidate SSB positions for the same SSB index within certain transmission window are not available at the UE due to DL CCA failures” can be considered as the baseline for SL-U.

Proposal-1: UE transmit timing error requirements for NR SL could be reused for SL-U.

Proposal-2: Discuss how to update the following core requirements due to LBT failure:

- Tevaluate,SLSS for initiation/cease of SLSS transmission with SyncRef UE as synchronization reference

- Tdetect,SyncRefUE and Tmeasure,PSBCH-RSRP for selection/reselection of V2X synchronization reference source

Proposal-3: L1-RSRP measurement period for NR SL could be reused for SL-U.

Proposal-4: Update the test case for congestion control considering LBT failure in SL-U.

Proposal-5: The condition of “SLSS occasion is not available” can be defined when N S-SSB positions are not available, and FFS N for different procedures, including initiation/cease of SLSS transmission, detection and measurement period for Selection/Reselection of V2X synchronization reference source.

Proposal-6: When the number of missing SLSS due to LBT failure is too large, UE is not required to meet the corresponding requirements.

4	Reference

- RP-222806, WID revision: NR sidelink evolution, OPPO.

- R4-2303265, WF on R18 NR SL RRM requirements, LG Electronics