3GPP TSG-RAN WG4 Meeting # 106bis-e			 				    		                 R4- 2305248

Electronic Meeting, April 17th – April 26th, 2023

Agenda item:			5.20.2.1

Source:	Samsung,

Title:	Co-ex study calibration and results sharing for SBFD

Document for:	Discussion

# Introduction

In this document, we would like to submit our calibration data and the updated co-ex study results to the meeting for discussion.

# Calibration data updates

Between #106 and #106bis-e, we have participated the offline calibration work, and produced the calibration data for the interested scenarios of ours. In this paper, we provide our data for meeting to discuss and consider.

## FR1 SBFD Urban Macro UL

Victim: SBFD (DU)

SINR baseline: No system in adjacent channel

SINR with ACI: NR TDD DL in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  | UL power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 59.94 | 88.58 | 112.04 | -5.15 | 11.40 | 13.16 | -5.91 | 10.71 | 13.12 | -21.07 | 7.63 | 23.00 |

## FR1 SBFD Urban Macro DL

Victim: SBFD (DU)

SINR baseline: No system in adjacent channel

SINR with ACI: NR TDD DL in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 59.94 | 88.58 | 112.04 | 7.86 | 24.50 | 47.80 | 7.17 | 23.94 | 47.45 |

## FR1 legacy TDD Urban Macro UL

Victim: NR TDD UL

SINR baseline: NR TDD UL in adjacent channel

SINR with ACI: SBFD (DU) in adjacent channel

Priority: Low

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  | UL power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 60.03 | 88.46 | 112.08 | -7.57 | 12.94 | 14.94 | -9.95 | 10.74 | 14.81 | -13.97 | 14.46 | 23.00 |

## FR1 legacy TDD Urban Macro DL

Victim: NR TDD DL

SINR baseline: NR TDD DL in adjacent channel

SINR with ACI: SBFD (DU) in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 59.92 | 88.59 | 112.03 | 7.23 | 23.90 | 47.42 | 7.15 | 23.93 | 47.38 |

## FR2 SBFD Urban Macro UL

Victim: SBFD (DU)

SINR baseline: No system in adjacent channel

SINR with ACI: NR TDD DL in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  | UL power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 65.89 | 81.48 | 97.22 | 1.48 | 13.08 | 13.23 | 1.47 | 13.08 | 13.23 | -7.09 | 8.50 | 13.40 |

## FR2 SBFD Urban Macro DL

Victim: SBFD (DU)

SINR baseline: No system in adjacent channel

SINR with ACI: NR TDD DL in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 65.89 | 81.48 | 97.22 | 12.00 | 26.78 | 42.85 | 11.41 | 26.49 | 42.63 |

## FR2 legacy TDD Urban Macro UL

Victim: NR TDD UL

SINR baseline: NR TDD UL in adjacent channel

SINR with ACI: SBFD (DU) in adjacent channel

Priority: Low

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  | UL power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 65.91 | 81.59 | 97.33 | -3.45 | 12.27 | 15.00 | -3.33 | 12.14 | 15.00 | -0.08 | 13.40 | 13.40 |

## FR2 legacy TDD Urban Macro DL

Victim: NR TDD DL

SINR baseline: NR TDD DL in adjacent channel

SINR with ACI: SBFD (DU) in adjacent channel

Priority: High

|  | Coupling Loss |  |  | Baseline SINR |  |  | SINR with ACI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Samsung | 65.88 | 81.56 | 97.22 | 11.56 | 26.54 | 42.72 | 11.44 | 26.46 | 42.57 |

# Co-ex study results updates

Following the latest assumptions from R4-2220246 in RAN4#105 meeting, we further udpates the co-ex study results for Urban Macro scenarios.

For those assumptions with multiple options, we listed our assumptions implemented below:

| LoS probability for Macro-to-Macro case: | 1st Priority: Reuse the same model (including LoS) as in TR 38.828 with h_UT equals to 25m |
| --- | --- |
| SBFD antenna configuration | SBFD antenna configuration 1 (1x size) and 2 (2x size) |
| BS antenna model | Baseline: TR 38.828 |
| UE distribution | Baseline: 20% indoor and 80% outdoor for FR1 |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD at gNB side |

Simulation Scenarios:

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Low |
|  | 3 | Indoor -> Indoor | Low |
| FR2 (30GHz) | 4 | Urban Macro -> Urban Macro | High |
|  | 5 | Urban Hotspot -> Urban Hotspot | Low |
|  | 6 | Urban Micro -> Urban Micro | Low |
|  | 7 | Indoor -> Indoor | Low |

Simulation Cases:

| Victim | Aggressor | Figures: Aggressor(left) and Victim(right) | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 | NR TDD DL | High |
| NR TDD UL | SBFD(DUD) | Case 3 | NR TDD UL | Low |
|  | SBFD (DU) | Case 4 | NR TDD UL | Low |
| SBFD (DUD) | NR TDD DL | Case 5 | No system in adjacent channel | High |
| SBFD (DU) | NR TDD DL | Case 6 |  | High |
| SBFD(DUD) | NR TDD UL | Case 7 |  | Low |
| SBFD(DU) | NR TDD UL | Case 8 |  | Low |

## Scenario 1: FR1, Urban Macro -> Urban Macro

### Case 1: SBFD DUD (aggressor) to legacy NR TDD DL (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB):

Table 3.1.1-1: Case 1 SINR and throughput degradation (SBFD gNB antenna config. 1)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD (antenna config. 1) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | -0.07 | 0.79 |
|  | 50% | -0.05 | -0.22 |
|  | 95% | 0.07 | 0 |

Figure 3.1.1-1: Case 1 CDF of SINR and throughput degradation (SBFD gNB antenna config. 1)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.1.1-2: Case 1 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD (antenna config. 2) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | -0.27 | -2.89 |
|  | 50% | -0.21 | -0.86 |
|  | 95% | -0.27 | 0 |

Figure 3.1.1-2: Case 1 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 2: SBFD DU (aggressor) to legacy NR TDD DL (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB):

Table 3.1.2-1: Case 2 SINR and throughput degradation (SBFD gNB antenna config. 1)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DU (antenna config. 1) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | -0.10 | -1.07 |
|  | 50% | -0.09 | -0.39 |
|  | 95% | -0.11 | 0 |

Figure 3.1.2-1: Case 2 CDF of SINR and throughput degradation (SBFD gNB antenna config. 1)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.1.2-2: Case 2 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DU (antenna config. 2) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | 0.02 | 0.19 |
|  | 50% | 0.05 | 0.19 |
|  | 95% | -0.02 | 0 |

Figure 3.1.2-2: Case 2 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 5: Legacy NR TDD DL (aggressor) to SBFD DUD (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB): TBA

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.1.3-2: Case 5 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy SBFD DUD (antenna config. 2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aggressor: TDD DL |  |  |  |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
| Samsung | - | SBFD DL | SBFD UL | SBFD DL | SBFD UL |
|  | 5% | 0.67 | 0.22 | 5.72 | 4.42 |
|  | 50% | 0.60 | 0.26 | 2.45 | 1.99 |
|  | 95% | 0.39 | 0.02 | 0 | 0.13 |

Figure 3.1.3-2: Case 5 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 6: Legacy NR TDD DL (aggressor) to SBFD DU (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB): TBA

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.1.4-2: Case 6 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy SBFD DU (antenna config. 2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aggressor: TDD DL |  |  |  |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
| Samsung | - | SBFD DL | SBFD UL | SBFD DL | SBFD UL |
|  | 5% | 0.68 | 0.22 | 5.82 | 4.33 |
|  | 50% | 0.58 | 0.27 | 2.35 | 2.04 |
|  | 95% | 0.38 | 0.02 | 0 | 0.13 |

Figure 3.1.4-2: Case 6 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

## Scenario 4: FR2, Urban Macro -> Urban Macro

### Case 1: SBFD DUD (aggressor) to legacy NR TDD DL (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB):

Table 3.2.1-1: Case 1 SINR and throughput degradation (SBFD gNB antenna config. 1)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD (antenna config. 1) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | 0.17 | 1.35 |
|  | 50% | 0.14 | 0.51 |
|  | 95% | 0.05 | 0 |

Figure 3.2.1-1: Case 1 CDF of SINR and throughput degradation (SBFD gNB antenna config. 1)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.2.1-2: Case 1 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD (antenna config. 2) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | 0.23 | 1.80 |
|  | 50% | 0.02 | 0.08 |
|  | 95% | 0.02 | 0 |

Figure 3.2.1-2: Case 1 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 2: SBFD DU (aggressor) to legacy NR TDD DL (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB):

Table 3.2.2-1: Case 2 SINR and throughput degradation (SBFD gNB antenna config. 1)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DU (antenna config. 1) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | 0.09 | 0.73 |
|  | 50% | 0.07 | 0.25 |
|  | 95% | 0.08 | 0 |

Figure 3.2.2-1: Case 2 CDF of SINR and throughput degradation (SBFD gNB antenna config. 1)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.2.2-2: Case 2 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DU (antenna config. 2) |  |
|  |  | SINR degradation (dB) | Throughput degradation (%) |
| Samsung | 5% | 0.28 | 2.18 |
|  | 50% | 0.18 | 0.68 |
|  | 95% | 0.11 | 0 |

Figure 3.2.2-2: Case 2 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 5: Legacy NR TDD DL (aggressor) to SBFD DUD (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB): TBA

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.2.3-2: Case 5 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy SBFD DUD (antenna config. 2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aggressor: TDD DL |  |  |  |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
| Samsung | - | SBFD DL | SBFD UL | SBFD DL | SBFD UL |
|  | 5% | 0.53 | 0.00 | 4.08 | 0.02 |
|  | 50% | 0.25 | 0.00 | 0.95 | 0.02 |
|  | 95% | 0.19 | 0.00 | 0 | 0.00 |

Figure 3.2.3-2: Case 5 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

### Case 6: Legacy NR TDD DL (aggressor) to SBFD DU (victim)

When SBFD gNB antenna assumes SBFD gNB antenna configuration 1 (1x size as legacy gNB): TBA

When SBFD gNB antenna assumes SBFD gNB antenna configuration 2 (2x size as legacy gNB):

Table 3.2.4-2: Case 6 SINR and throughput degradation (SBFD gNB antenna config. 2)

| Source | Observation Point | Victim: Legacy SBFD DU (antenna config. 2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Aggressor: TDD DL |  |  |  |
|  |  | SINR degradation (dB) |  | Throughput degradation (%) |  |
| Samsung | - | SBFD DL | SBFD UL | SBFD DL | SBFD UL |
|  | 5% | 0.59 | 0.01 | 4.51 | 0.08 |
|  | 50% | 0.29 | 0.00 | 1.09 | 0.02 |
|  | 95% | 0.21 | 0.00 | 0 | 0.00 |

Figure 3.2.4-2: Case 6 CDF of SINR and throughput degradation (SBFD gNB antenna config. 2)

# Reference

[1] R4-2220246, “WF for co-existence study”, CMCC, Samsung

[2] R4-2302888, “WF for SBFD adjacent channel co-existence study”, CMCC