3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305266

# Online, April 17 – April 26, 2023

Agenda item:	5.9.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2 SCell activation enhancement

Document for:	Discussion

# Introduction

The Rel-18 WI of even further RRM enhancement was approved in [1] and further revised in [2]. One of the objectives is to study the SCell activation delay reduction in FR2. In RAN4#105, some high-level principle was agreed in [3]. In RAN4#l06 meeting, the scope of the solutions was further narrowed down [4]. In this paper, we further provide our views of RRM impact of the objective and provide further analysis on the solutions.

# Discussion

Based on the discussion in last meeting, some solutions are down scoped which was captured in [4]. One of the remaining issues for uncategorized solution is shown as follows:

Issue 3-1-6: Scheduling enhancement for FR2 unknown SCell activation - Proposals - Option 1 (Apple): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed. - Option 2 (Nokia): - The potential to enable earlier data transmission within the activation period shall be studied. - The UE shall be scheduled by the network immediately after L1-RSRP reporting. - The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

Based on discussion in last meeting, the concerns from proponent of option 2 is that whether scheduling is allowed before the SCell activation delay requirements defined in existing spec. The related descriptions are quoted as follows:

TS 38.133 8.3.2 Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

The above description can be ambiguous to some extent. Companies may interpret this time occasion as the time defined in the existing spec, which is the minimum requirements for SCell activation delay. However, from our understanding, it is the actual time UE completes the SCell activation when UE is able to report valid CQI for the SCell.

Observation 1: The existing requirements does not prohibit UE from reporting valid CQI before the delay requirements defined in the specification.

Thus, if UE is able to complete SCell activation earlier than what is required by the requirements, UE will report valid CQI and NW can schedule the UE. Additionally, before UE reports valid CQI, NW is also not forbidden from scheduling though the performance is less guaranteed.

Proposal 1: The early scheduling (CQI reported before the delay requirements) is not prohibited in existing SCell activation requirement. No enhancement is needed.

# Conclusions

Observation 1: The existing requirements does not prohibit UE from reporting valid CQI before the delay requirements defined in the specification.

Proposal 1: The early scheduling (CQI reported before the delay requirements) is not prohibited in existing SCell activation requirement. No enhancement is needed.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2220440, WF on R18 eFeRRM - FR2 SCell activation enhancement

[4] R4-2303228, Way forward on R18 RRM enhancement (part 1)