3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305280

Online, April 17 – April 26, 2023

Title: 	Discussion on	improvement on FR2 SCell/SCG setup/resume

Source: 	Huawei, HiSilicon

Agenda item:	5.25.4

Document for:	Discussion

Introduction

In R18 WI on Further NR Mobility, the Objective #7 on Enhancements improvement on FR2 scell/SCG setup/resume is revised in [RP-223520]. The revised objective is duplicated as below [1].

To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including: - Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and - Definition of corresponding RRM requirements [RAN4]; and - If necessary based on RAN4 outcome, definition of corresponding signalling support [RAN2]. Note 5: RAN4 will coordinate in due course with RAN2 to start the work. Note 6: R4-2220415 serves as baseline for future work in RAN4 Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.

This contribution provides analysis on this revised objective.

Discussion

In last meeting, the below agreement is achieved for the improvement on SCell/SCG setup delay [2].

- Agreement - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure. - RAN4 can continue discussion on the feasibility of doing additional measurement starting from RRC setup/resume, and requirements shall be defined if feasible solution is agreed.

- Reusing existing measurement

In R16, UE can perform EMR (early measurement report) measurement on neighbour cells and serving cell during idle/inactive mode. The EMR results are reported to network upon UE enters to RRC connected mode.

In R18 mobility enhancement, some companies proposed some further enhancements when T331 expires during idle/inactive mode or after UE enters to connected mode, e.g, require UE to continue performing measurement when T331 expires or is stopped in idle/inactive mode and/or in connected mode. As per R16 RAN2’s specification, when T331 expired, whether to continue R16 EMR measurements is up to UE implementation.

5.7.8.3 T331 expiry or stop [TS38.331] The UE shall: - 1> if T331 expires or is stopped: - 2> release the VarMeasIdleConfig. - NOTE: It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

In our understanding, T331 is used for balancing UE power consumption and performance. Continuing measurement regardless of T331 would increase UE power consumption and violate the significance of T331. This kind of enhancements has been ever discussed during R16 EMR, and the final conclusion is that it is up to UE implementation whether to continue idle/inactive measurements after T331 expires. It is not preferred to re-open the discussion.

Proposal 1: For EMR based measurement, it is not preferred to continue measurements after T331 expires/ is stopped (including in idle/inactive and connected mode).

According to the revised objective #7, RAN4 will study how to use the EMR measurements (if any) achieved in idle/inactive mode. In our understanding, ‘Available’ means that measurement results are stored in the memory when UE is required to report measurement results or indicate available measurement indication. In R16 EMR, the “available” concept has already been applied. Taking RRC resume procedure (for Inactive UE) as an example, there are two cases. For the below two existing cases, availability judgement has been executed:

- If the idleModeMeasurementReq is included in the RRCResume message, if the EMR measurement results are available, UE is to report the corresponding results in the RRCResumeComplete message.

- If the idleModeMeasurementReq is not included in the RRCResume message, UE indicates idleMeasAvailable in the RRCResumeComplete message to network (if UE has available measurements).

2> if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport: 3> if the idleModeMeasurementReq is included in the RRCResume message: 4> set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available; 4> set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available; 4> discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers; 3> else: 4> if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or 4> if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport: 5> include the idleMeasAvailable;

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

Observation 1: Availability judgement of IDLE/INACTIVE mode measurement results has already been executed according to existing specification.

Regarding the definition of “validation”, one simple description is:

“The UE has acquired EMR measurement result during the last 5 seconds before transmission of the EMR measurement results or transmission of a measurement indication. ”

Then the possible UE behaviour can be:

-For inactive UE, if the idleModeMeasurementReq is included in the RRCResume message, if the EMR measurement results are available, UE can additionally make a judgement whether these measurement results are acquired with 5 seconds. If the measurement results are available and valid, UE can report the corresponding results in the RRCResumeComplete message.

-For Idle UE and Inactive UE for whom the idleModeMeasurementReq is not included in the RRCResume message, similar principle as above can be followed. When the measurement results are available and valid, UE can indicate an indication to network in the RRCSetupComplete/ RRCResumeComplete message.

As UE is able to make decision whether the current measurement results are valid or not, we don’t observe the necessity of introducing additional indication.

Proposal 2: For the direction of reusing existing measurement results, EMR measurement results are regarded as valid if the UE has acquired the EMR measurement result during the last 5 seconds before transmission of the EMR measurement results or transmission of a measurement indication.

Proposal 3: As UE is able to make decision whether the current measurement results are valid or not, the necessity of introducing additional indication is not observed.

- Enhanced FR2 measurement

This direction is that UE performs enhanced FR2 measurement during RRC setup/resume procedure and connected mode. This is not related with existing EMR measurement and does not consider the existing cell reselection measurement results. One proposal is “network provide explicit information such as target frequency and/or Cell ID, and/or target SSB info for enhanced measurement on FR2. When certain trigger condition is met, UE starts to perform FR2 measurement after MSG1. UE is allowed to continue the enhanced measurement after UE enters connected mode”. We understand that the applicable scenario of this solution is that PCell is on FR1 and UE support per-FR gap capability. The analysis on the idea is analyzed in the following.

To avoid the instant channel impact, several physical measurement samples shall be linear averaged. Using one or less physical measurement samples to present of the cell quality is not preferred, as such measurement accuracy is low and it would degrade the performance robustness.

Observation 1: For enhanced FR2 measurement, to guarantee the measurement accuracy, the measurement samples are not supposed to be reduced.

As we know, UE shall sweep the RX beams to perform FR2 L3 mobility measurement. For FR2 L3 measurement in both idle/inactive mode and connected mode, the RX beam number is 8. We don’t think UE will apply a special implementation to reduce the RX beam sweeping number during the RRC connection setup/resume procedure, as the time duration is very short and afterwards UE will go back to normal implementation.

Observation 2: For enhanced FR2 measurement, not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure.

Thus even if SSB period is considered instead of SMTC period, the enhanced FR2 measurement periodicity is 800ms. To our knowledge, the latency of RACH procedure (control plane latency) from RRC_Idle/RRC_INACTIVE state to RRC_CONNECTED state was ever widespread evaluated in R16 study on self-evaluation towards IMT-2020 submission [3]. The conclusion is achieved in TS37.910 clause 5.7.2.1:

“It is observed that NR fulfils the control plane latency requirement of 20ms in a wide range of configurations. If, in control plane procedure, the latency of step 7 and step 9 can be further reduced, the 10ms target as encouraged by ITU-R can be achieved in some cases.”

Even considering security procedure (typical 20ms), the time duration of RRC setup/resume procedure is dozens of milliseconds.

In our understanding, if there are large volume of data in network side, network can indicate MO during RRC resume procedure for inactive UE, or network can immediately configure the MO upon UE completes RACH procedure and enters to connected mode. Then the question is how much gain would be achieved by allowing UE perform enhanced measurement just dozens of milliseconds in advance. Moreover whether UE can acquire a complete measurement result during dozens of milliseconds is questionable.

Proposal 4: It is not clear how much gain would be achieved by allowing UE perform enhanced measurement just dozens of milliseconds in advance, compared with the case network configure MO upon UE enters to connected mode. Moreover whether UE can acquire a complete measurement result during dozens of milliseconds is questionable.

Conclusions

This contribution provides the consideration on improvement of FR2 SCell/SCG setup/resume in R18 mobility enhancement. The following proposals are provided:

Proposal 1: For EMR based measurement, it is not preferred to continue measurements after T331 expires/ is stopped (including in idle/inactive and connected mode).

Observation 1: Availability judgement of IDLE/INACTIVE mode measurement results has already been executed according to existing specification.

Proposal 2: For the direction of reusing existing measurement results, EMR measurement results are regarded as valid if the UE has acquired the EMR measurement result during the last 5 seconds before transmission of the EMR measurement results or transmission of a measurement indication.

Proposal 3: As UE is able to make decision whether the current measurement results are valid or not, the necessity of introducing additional indication is not observed.

Observation 1: For enhanced FR2 measurement, to guarantee the measurement accuracy, the measurement samples are not supposed to be reduced.

Observation 2: For enhanced FR2 measurement, not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure.

Proposal 4: It is not clear how much gain would be achieved by allowing UE perform enhanced measurement just dozens of milliseconds in advance, compared with the case network configure MO uponUE enters to connected mode. Moreover whether UE can acquire a complete measurement result during dozens of milliseconds is questionable.

Reference

- RP-223520, Revised WID on Further NR mobility enhancements, MediaTek Inc., Apple

- R4-2303309, WF on NR Mobility Enhancements RRM requirements (part 2), Apple