3GPP TSG-RAN WG4 Meeting # 106bis-e										                   R4-2305286

Online, April 17 – April 26, 2023

Title: 					Discussion on RRM impact for Enhanced support of reduced capability NR devices

Source: 	Huawei, HiSilicon

Agenda item:	5.32.4

Document for:	Discussion

Introduction

The revised WID on enhanced support of reduced capability NR devices has been approved in [1]. Techniques for further UE complexity reduction have been studied in the study item documented in TR 38.865. The WID has the following objectives:

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

This contribution provides the preliminary analysis on the possible RRM impacts due to eRedCap.

Discussion

- Enhanced eDRX in RRC_INACTIVE

RedCap UE in inactive mode shall monitor both CN paging and RAN Paging. RAN2 has the below principle of configuring IDLE eDRX and INACTIVE eDRX:

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle.

Therefore it is invalid case that only RRC Inactive eDRX > 10.24 is configured while RRC Idle eDRX > 10.24 is not configured. So the first two open issues captured in [2] don’t need to be discussed.

Serving cell requirements in RRC Inactive state for eDRX > 10.24 - Option 1: Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state - Option 1a: - Maximum INACTIVE (RAN paging) eDRX length = 10485.76s - 1.28s ≤ PTW length ≤ IDLE state PTW length, with 1.28s granularity - Option 2: RAN4 to await progress in RAN2/CT/SA to discuss and specify RRM core requirements for eDRX in RRC_INACTIVE state. Neighbour cell requirements in RRC Inactive state for eDRX > 10.24 Option 1: Idle state eDRX requirements for eDRX cycle > 10.24 sec are used as baseline for Inactive state - Option 1a: - Maximum INACTIVE (RAN paging) eDRX length = 10485.76s - 1.28s ≤ PTW length ≤ IDLE state PTW length, with 1.28s granularity - Option 2: RAN4 to await progress in RAN2/CT/SA to discuss and specify RRM core requirements for eDRX in RRC_INACTIVE state. When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements - Option 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion. - Option 2: Requirement based on T (according to clause 7.1 in TS38.304) needs discussions. When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements - Option 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration which are larger than 10.24s, UE performs intra-frequency/inter-frequency measurement according to a fixed period rather than T(s).

In R17 RedCap WI, eDRX for RRC_INACTIVE is up to 10.24s, and there is no PTW for RRC_INACTIVE. Therefore for R17 RedCap UE with both RRC_IDLE eDRX configuration and RRC_INACTIVE eDRX configuration, the reselection requirements for serving cell are defined based on T which is determined in RAN2, and the intra-frequency and inter-frequency cell reselection requirements are defined based on eDRX_Inactive cycle.

Further enhancement is provided for R18 RedCap UE. The extended DRX for RRC_INACTIVE can be larger than 10.24s, then two PTW (one for RRC-IDLE and the other for RRC_INACTIVE) with different parameters (e.g., length, etc.) can be present at the same time. RAN4 would carry out the study on the definition of RRM measurement requirements for cell reselection

As per some conclusion from SI phase, IDLE eDRX is not less than inactive eDRX. Figure 1 provides one case that there are some PH where RAN paging and CN paging are overlapped, where Idle eDRX cycle length is 40.96s and Inactive eDRX cycle length is 20.48s.

|  | PH#1 | PH#2 | PH#3 | PH#4 | PH#5 | PH#6 | PH#7 | PH#8 | PH#9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Idle eDRX |  |  |  |  |  |  |  |  |  |
| Inactive eDRX |  |  |  |  |  |  |  |  |  |

Figure 1

In last RAN2 meeting, the following agreements are achieved.

Enhanced eDRX cycle and PTW configurations RAN2 confirms the R17 agreements made at RAN2#114 for enhanced INACTIVE eDRX: - - It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length. - - When RAN and CN paging coincide in the same PH, the actually used PTW starting location is the same for RAN and CN paging. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW. PTW length value range of enhanced INACTIVE eDRX is same as IDLE eDRX, i.e. from 1.28s to 40.96s in the step of 1.28s. Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024 Add the configuration of eDRX cycle (>10.24 s) and PTW length for enhanced INACTIVE eDRX in the RRCRelease message

When RAN and CN paging coincide in the same PH, the PTW starting locations are the same. And RAN2 will further discuss how to calculate the PTW starting location and how UE is expected to monitor RAN and CN PTW, e.g. whether UE in RRC_INACTIVE monitors for only RAN PTW or both CN and RAN PTW when they overlap. In other words, in coincide PH, RAN2 would discuss how to decide DRX cycle (which is noted as “T”), e.g., T2-1 for overlapping part of CN and RAN PTW, and T2-2 for the non-overlapping part with PTW, or a unified T2 which is common for both parts.

Figure 2. ‘T’ in RAN and CN paging coincide PH and non-coincide PH

As in R17 RedCap, the RRM requirements of serving cell are specified based on T (paging DRX) which is applied for both inside PTW and outside PTW. When two PTW exist, there are multiple kinds of T: T1 within PTW in non-overlapped PTW, unified T2 or separate T2-1/T2-2 in PH where RAN and CN paging are coincide. Anyway the principle of how to determine T when two PTW are overlapped shall wait for RAN2. Which T UE shall use to perform serving cell measurement needs further consideration.

As in R17 RedCap, the measurement period of intra-frequency/inter-frequency measurement depends on inactive eDRX cycles. The reason why T is not used for neighbour cell measurement is that paging DRX T is different within idle_PTW and outside idle_PTW. Measurements as per changed T would complicate UE implementation. We think the same principle can be reused for two PTWs case for R18 RedCap: defining intra-frequency/inter-frequency measurement requirements according to a fixed period when both IDLE eDRX and Inactive eDRX larger than 10.24s. How to determine the fixed period is FFS.

For both serving cell and intra-frequency/inter-frequency measurements, one thing is noted that UE shall not perform measurements outside PTW(s) where PTW can be idle_PTW or and inactive_PTW, otherwise the power saving gain of introducing PTW would be vanished.

Proposal 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, UE shall NOT perform measurements outside PTW(s) for idle eDRX and inactive eDRX.

Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion.

Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, define intra-frequency/inter-frequency measurement requirements according to a fixed period where the fixed period can be further studied.

- Complexity/cost reduction

There are some progress in RAN1 on UE BB bandwidth reduction. These complexity reduction focus on reduce the UE BB bandwidth for PDSCH and PUSCH to 5MHz. We assume there is no impact on SSB reception and processing.

#110b - For UE BB bandwidth reduction - the scheduling of SIB1/ broadcast OSI (PDSCH) are allowed to be larger than 5 MHz (as in legacy operation). The size of UE post-FFT buffering is FFS. - a UE is not expected to receive an UL grant, either dynamic grant or configured grant, with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop. - For UE Peak data rate reduction - The value X in “The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X” got no consensus yet. - For standalone PR1, the constraint value can be different from the X above. #111 - For UE BB bandwidth reduction - PR3 is agreed in RAN1 to replace BW3, i.e. “a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.” - For broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz. - Broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. - For paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). - A UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop. - For RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. Only for this case, a timeline relaxation for Msg3 transmission can be further discussed. #112 Agreement Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In RAN1 it is agreed that the scheduling of SIB1 is allowed to be larger than 5MHz. For interruption in paging reception, RRC re-establishment or Connection Release with Redirection, the SI reading related time is denoted as TSI-NR, and no concrete value.

Observation 1: For interruption in paging reception, RRC re-establishment or Connection Release with Redirection, the SI reading related time is denoted as TSI-NR, and no concrete value.

Intuitively the CGI reading related requirements may be impacted. However we think as the CGI reading delay based on TSMTC and TRMSI-scheduling, UE has efficient time to decode SIB1 as long as UE received the complete SIB1.

For interruption requirements due to CGI reading, if UE BB bandwidth for PDSCH is reduced, the processing time of decoding SIB1 may be enlarged. Then the interrupted duration on serving cell may be larger accordingly. However as we know CGI reading is for SON/ANR purpose. If network would like to know the CGI of one pico cell, it can request a normal UE with normal BB bandwidth to perform CGI reading. It is not a typical case that network indicate a R18 Redcap UE with reduced BB bandwidth to do such procedure.

Tidentify_CGI = (TMIB + T SIB1) ms Where: TMIB is the time period used to acquire MIB message. TMIB = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB = 25 * TSMTC ms for target cell carrier frequency on FR2. TSIB1 is the time period used to acquire SIB1 message. TSIB1 = 6 * TRMSI-scheduling ms.

Observation 2: It is not typical that network request a R18 RedCap UE with baseband bandwidth reduction to perform CGI reading for the purpose of reportCGI.

Proposal 4: No RRM impacts are foreseen due to UE BB bandwidth reduction.

Conclusions

This contribution provides analysis on the measurement requirements for RedCap UE when eDRX is configured. According to the analysis, we propose the following proposals.

Proposal 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, UE shall NOT perform measurements outside PTW(s) for idle eDRX and inactive eDRX.

Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, measurement requirements on serving cell wait for RAN2 conclusion.

Proposal 3: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, define intra-frequency/inter-frequency measurement requirements according to a fixed period where the fixed period can be further studied.

Observation 1: For interruption in paging reception, RRC re-establishment or Connection Release with Redirection, the SI reading related time is denoted as TSI-NR, and no concrete value.

Observation 2: It is not typical that network request a R18 RedCap UE with baseband bandwidth reduction to perform CGI reading for the purpose of reportCGI.

Proposal 4: No RRM impacts are foreseen due to UE BB bandwidth reduction.

References

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson