3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305303

# Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on the feasibility of FR2 BS aspects

Agenda Item:	5.20.2.2.2

Document for:	Approval

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

In this contribution, we provide some discussion on the open issues on FR2 BS aspects.

- Discussion

2.1 General part for RSIC analysis framework

The  following agreements agreed in last meeting.

- Agreement:

  - RSIC analysis framework table shall be adopted for SBFD BS RF feasibility study to be captured in TR38.858, and subsection for different component capabilities shall be reserved to encourage companies’ inputs.

  - RAN4 target to draw initial common observations based on the collected data till this meeting for self-interference analysis from BS aspect.

  - RAN4 also target to list open issues for the cases which diverge views observed based on the data collected from companies

In general we agree with the above agreements. From the collected data of last meeting, we observe that there are some parameters may need alignment on the common understanding.

Firstly, as discussed for FR1 BS [5], we think the scalling factor should be used in the budget calculation for the asymmetrical SBFD.

Secondly, description in current table on beam nulling /isolation may be ok to FR1 but it is not friendly to FR2. The beam to beam isolation is always evaluated and measured in the test due to ABF or DBF architectures. Hence we propose not to distinguish ”TX” and ”RX” but to consider them together. It is proposed to change them to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR2 ABF or HBF architectures.

Thirdly, as the same reason of FR2 BS architectures, one RX chain is drived by a large number of LNAs, hence it is not a suitable reference point by using ”before LNA”. We propose to remove ”before LNA” in the table and to evaluate the Self-Interference signal in gNB TX subband at ”RX ant” as that for Self-interference leakage in gNB RX subband.

For FR2 our analysis and evaluation is sumarized in Table 2.1-2, where only some terms are updated as proposed above and the values are remaining unchanged as in [6]. The From Talbe 2.1-2, it can be found that the blocking level to RX pannel is weak and should be even lower at each LNA input. Hence the IM3 is not a limited factors. Due to the same reason that the blocking level is relatively weak, the other RX impacts due to blocker in Tx sub-band can also be negligible. Hence we think that the evaluation on Self-Interference leakage in gNB RX subband would be sufficient for FR2.

Proposal 1: It is proposed to change them to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR2 ABF or HBF architectures.

Proposal 2: It is proposes to remove ”before LNA” in the table and to evaluate the Self-Interference signal in gNB TX subband at ”RX ant” as that for Self-interference leakage in gNB RX subband.

Table 2.1-2 RSIC Analysis for FR2

| FR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc | 28 |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc | 85~95 |
|  |  | Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation | spatial separation between TX/RX panel |
|  | Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | -94~-104 |
|  | RF IC and other tech. | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc | N/A |
|  |  | RF IC techniques and other tech. |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques |  | xxx dBc | N/A |
|  | Self-Interference signal in gNB TX subband, measured at dBPSI_TXSBRX ant. |  |  |  | -60 ~ -70 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc | N/A |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |  |
|  |  |  | Rx IM3 contribution (dBm) |  | negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm | negligible |
|  | Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc | - |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc | 129 ~ 139 |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW | -88 dBm/40 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc | 129 |
| SBFD configuration |  |  |  |  | [80,40,80] |
| Guardband assumption (if exist) |  |  |  |  | Existing SU |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |

2.2 Inter-sector interference analysis

For FR2 inter-sector isolation, some measurements based on existing modules are proceed as below. The AAS mounting in the same mast would be worse cases for radiated isolation. The following two cases are the typical site deployments for the worse cases. The left one (Case 1) is up-down installation and the right one (case 2)is 120° installation.

For each case, 60*60=3600 beam combinations are measured. The results are shown in Table 2.2-1 for existing modules and Table 2.2-2 for improved modules. Some surface wave suppression measures can be used to improve the isolation, e.g. reflection and absorption structure. Using the measures, more than 10 dB improvement can be achieved fort the case with poor isolation, as shown in in the measurements evaluation.

Table 2.2-1: Inter-sector isolation for existing modules

| Test cases | Existing modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 96 | 99 | 103 |
| Case 2 | 120°installation | 85 | 90 | 100 |

Table 2.2-2: Inter-sector isolation for improved modules

| Test cases | Improved modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 101 | 103 | 104 |
| Case 2 | 120°installation | 100 | 101 | 102 |

Proposal 3: 100 dB inter-sector isoaltion is achieveable for FR2

The spatial isolation can be improved by BS schedule, e.g.TX/RX beam selection. Hence we think 95% CDF can be used for further evaluation. The inter-sector interference impacts for FR2 is sumarized in Table 2.2-3.

Table 2.2-3: Inter-sector interference analysis

| FR2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dB |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 100 |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Spatial separation |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -99 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |
|  | Self-Interference signal in gNB TX subband, measured at RX ant.dBPSI_TXSB |  |  | -65 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | -- |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 134 dBc |
| Noise floor ⑩dBm |  |  |  | -88 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 129 dBc |
| SBFD configuration |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

- Conclusion

In this contribution, we provide some consideration on FR2 BS aspects.

Proposal 1: It is proposed to change them to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR2 ABF or HBF architectures.

Proposal 2: It is proposes to remove ”before LNA” in the table and to evaluate the Self-Interference signal in gNB TX subband at ”RX ant” as that for Self-interference leakage in gNB RX subband.

Proposal 3: 100 dB inter-sector isoaltion is achieveable for FR2

Our analysis and evaluation is sumarized in Table 2.2-3 for FR2 inter-sector interference analysis.

- Reference

[1] R4-2220244, “WF for the feasibility from BS aspect”, Samsung

[2] R4-2216239, “Evolution of receiver blocking and AGC”, Huawei, HiSilicon

[3] R4-2303755, Topic summary for [106][310] FS_NR_duplex_evo_Part1, Moderator (Samsung)

[4] R4-2302883, “WF for Self-interference analysis from BS aspect for SBFD operation”

[5] R4-2305302, “Discussion on the feasibility of FR1 BS aspects”, Huawei, HiSilicon

[6] R4-2301426, “Discussion on the feasibility from BS aspect”, Huawei, HiSilicon