3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305316

Electronic Meeting, April 17th – April 26th, 2023

Title: 	Discussion on scheduling/measurement restrictions for R18 FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	5.8.3.4

Document for:	Discussion

Introduction

In this contribution, we will provide the discussion on scheduling restrictions and measurement restrictions for FR2 multi-Rx DL reception.

Discussion

Scheduling Restrictions

The scheduling restrictions requirements in TS 38.133 specifies the conditions when data receptions and L1/L3 measurements cannot be performed simultaneously.

For data receptions, the beam information is derived based on the active TCI state and non-beam sweeping operation is assumed for data receptions. However, UE is assumed to perform L3 measurements with beam sweeping operation. If simultaneous of data reception and L3 measurement is supported, which requires that UE activates one antenna panel for data reception and activates another antenna panel for L3 measurement, which cannot be guaranteed by UE. Besides, rough beam is assumed for L3 measurements and fine beam is assumed for data receptions. It is impossible to pair a rough beam and a fine beam for simultaneous receptions. Hence, the scheduling restrictions due to L3 measurements still need to be applied for FR2 multi-Rx DL receptions.

Proposal 1: For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and L3 measurement, and the existing scheduling restrictions requirements due to L3 measurements still need to be applied.

For L1 measurements, the existing scheduling restrictions requirements define that UE is not able to perform data receptions when RS for L1 measurement is not QCL with the TCI state of data.

For SSB or CSI-RS with repetition on, beam sweeping operation is assumed and UE needs to try multiple beam directions on this RS resource. However, there is no beam sweeping operation for data receptions. Simultaneous of data reception and L1 RS with beam sweeping operation is not considered. The scheduling restrictions due to L1 measurements on SSB or CSI-RS with repetition on still need to be applied.

Proposal 2: For R18 multi-Rx DL receptions, the scheduling restrictions on SSB for L1 measurements or and on CSI-RS with repetition=on still need to be applied.

For FR2 multi-Rx DL reception, the two activated TCI states used for data receptions from two TRPs are based on the results of group-based L1-RSRP beam reporting. The UE will be configured with multiple RSs for L1 measurements.

Figure 1: Example of RSs configured L1 measurements

For example, as shown in Figure 1, the data receptions from two TRPs are based on the reported beam pair of CSI-RS1+CSI-RS2. The beam pair used for simultaneous data receptions from two TRPs is derived from the measured best beam pair of CSI-RS1+CSI-RS2, which means that CSI-RS1 or CSI-RS2 is QCLed with one of the activated TCI states for data receptions, UE can be assumed to perform simultaneous data reception and L1 measurement on CSI-RS1/CSI-RS2. For other CSI-RS resources, UE cannot perform simultaneous data reception and L1 measurement.

When only one TCI state is activated for data receptions, then best Rx beam for data receptions is derived based on non-group based L1-RSRP reporting. There is no necessary to consider

Proposal 3: For R18 multi-Rx DL receptions, the scheduling restrictions requirements due to L1 measurements on CSI-RS without repetition=on need to be applied when the CSI-RS is not QCLed with any one of the activated TCI states for data receptions.

Measurement Restrictions

The measurement restrictions requirements in TS 38.133 specifies the conditions when two RSs configured for L1 measurements cannot be measured simultaneously.

When SSB is used for L1 measurements, beam sweeping operation is assumed on the SSB resource for beam training. UE cannot guarantee the simultaneous L1 measurements on an SSB resource and another RS due to the dynamic beam change on the SSB resource. When CSI-RS with repetition on is used for L1 measurements, beam sweeping operation is also assumed for beam training. Similarly, simultaneous L1 measurements on an CSI-RS resource repetition on and another RS are not considered.

Based on RAN4 agreements in [1, 2], group-based reporting is considered for enabling simultaneous DL receptions with two different beams. So, simultaneous data receptions can be enabled by activating two TCI states according to the results of group-based L1-RSRP reporting. When two RS resources without beam sweeping for L1 measurements are separately QCL-ed with the two TCI states that are activated for simultaneous data receptions, then UE can be assumed to perform simultaneous L1 measurements on the two RS resources.

Proposal 4: Simultaneous L1 measurements on two RS resources can be considered under the following conditions:

- The two RSs are both CSI-RS resources without repetition ON, and

- The two RSs are separately QCLed to the two activated TCI states used for simultaneous data receptions.

Conclusions

In this paper we provided our discussion on scheduling/measurement restrictions for FR2 multi-Rx DL reception. The followings are provided.

Proposal 1: For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and L3 measurement, and the existing scheduling restrictions requirements due to L3 measurements still need to be applied.

Proposal 2: For R18 multi-Rx DL receptions, it is suggested not to consider simultaneous data reception and the L1 measurement with beam sweeping, and the scheduling restrictions requirements due to the L1 measurements on SSB or on CSI-RS with repetition=on still need to be applied.

Proposal 3: For R18 multi-Rx DL receptions, the scheduling restrictions requirements due to L1 measurements on CSI-RS without repetition=on need to be applied when the CSI-RS is not QCLed with any one of the activated TCI states for data receptions.

Proposal 4: Simultaneous L1 measurements on two RS resources can be considered under the following conditions:

- The two RSs are CSI-RS resource without repetition ON, and

- The two RSs are separately QCLed to the two activated TCI states used for simultaneous data receptions.

Reference

- R4-2303302, “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo

- R4-2303317, “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2)”, Qualcomm Incorporated