3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305322

Electronic Meeting, April 17th – April 26th, 2023

Title: 	Discussion on RRM impacts for R18 MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	5.30.3.1

Document for:	Discussion

Introduction

In RAN4#106 meeting, the RRM impacts of R18 MIMO evolution were discussed and the following agreements have been captured in WF [1].

RRM impact except timing requirements related to two Tas Agreements - RRM requirements impacts - Objective 1 (CSI reporting enhancements) - No RRM impact from Rel-16/17 Type-II codebook refinement objective - FFS whether “UE reporting of time-domain channel properties” has impact on RRM requirements - Objective 2 (Unified TCI framework) - Further study and if needed specify extension of unified TCI framework RRM requirements to M-TRP. - Objective 3 (DMRS Enhancement) - No RRM requirements impact - Objective 4 (enhancements of CSI acquisition for C-JT) - No RRM requirements impact - Objective 5 - SRI/TPMI enhancements: No RRM requirements impact - SRS enhancements: RRM impacts are FFS - Objective 6 - UL precoding indication: RRM impacts are FFS

In this contribution, we will further study whether the following aspects have RRM impacts.

- UE reporting of time-domain channel properties for CSI reporting enhancements

- SRS enhancements

- UL precoding indication for STxMP

Discussion

UE reporting of time-domain channel properties (TDCP)

For TDCP reporting, the following agreements were achieved in RAN1.

[RAN1#112] Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic [RAN1#112] Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives: - Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling - Alt2. The value of Y follows the delays from the configured TRS resource - Alt3. The value of Y is UE-selected and reported The value of Y is a UE capability

According to TS38.214, the higher layer parameter reportQuantity in a CSI report can be set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index' or 'ssb-Index-SINR- Index'. For TRS-based TDCP reporting, UE performs measurements and needs to report the wideband quantized normalized amplitude/phase value for each delay. Obviously, the wideband quantized normalized amplitude/phase for TDCP reporting is different with any type of the existing reportQuantity configured in a CSI report.

Observation 1: For TDCP reporting, UE performs TRS based measurements and report the wideband normalized amplitude/phase for each delay, where the reported normalized amplitude/phase is different with any type of the existing reportQuantity configured in a CSI report.

When reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' or 'cri-RSRP-Index' or 'ssb-Index-RSRP-Index' or 'cri-SINR-Index', or 'ssb-Index-SINR-Index', UE is required to perform L1-RSRP/L1-SINR measurements for beam management. In TS38.133, L1-RSRP/L1-SINR measurement requirements are defined for beam management. When reportQuantity is set to other types of reportQuantity, UE is required to perform measurements and calculate the CSI parameters for demodulation purpose. There is no RRM requirement defined for any measurements related for CSI parameters calculation.  For TDCP reporting, UE is required to report the quantized wideband normalized amplitude/phase for the purpose of codebook/SRS management. RAN4 need to study whether it is necessary to define amplitude/phase measurement requirements for TDCP reporting. If TDCP reporting for codebook/SRS management is considered to be similar with L1-RSRP/L1-SINR reporting for beam management, then RAN4 could consider to define TRS based measurement requirements for TDCP reporting. If the reported wideband normalized amplitude/phase of the time-domain correlation profile is considered as new type of CSI parameters for demodulation purpose, there is no need to define RRM requirements for TDCP reporting.

Proposal 1: RAN4 need to study whether it is necessary to define TRS based measurement requirements (e.g. measurement period and measurement accuracy) for TDCP reporting.

SRS enhancements

For SRS enhancement, companies had different views on whether there was RRM impacts. Based on RAN1 progress, there are aspects for SRS enhancement: SRS enhancement for TDD CJT and SRS enhancement for 8Tx operation.

For SRS enhancement for TDD CJT, based on current RAN1 discussion, the enhancement is mainly about interference randomization. Apparently, there is no RRM impacts for CJT enhancement.

Observation 2: There is no RRM impacts for SRS enhancement for CJT.

Some companies mentioned that 8Tx enhancement may have impacts on SRS antenna port switching requirements. Based on conclusion in Rel-17 FeRRM when the requirements for SRS AS was introduced, two scenarios were considered:

- Scenario 1: One SRS symbol is configured in a slot for antenna port switching.

- Scenario 2: Other SRS configurations for antenna port switching.

Besides, it was assumed that SRS symbols are allocated at the last 6 symbols in a slot.

Observation 3: In existing requirements for SRS antenna port switching, SRS resource for antenna port switching are assumed to be allocated in the last 6 symbols in a slot.

Based on the comments in the last meeting, some companies state that the above assumption needs to be revisited to support 8 Tx. The related RAN1 agreements are quoted as follows.

Agreement For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols - Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM) - FFS: m can be legacy values, i.e., 2,4,[8,10,12,14]. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2 - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Based on above RAN1 agreements, it could be observed that the 8 ports SRS resource mapping could be further categorized into following cases:

- Case 1: 8 ports SRS resource is mapped to each OFDM symbols

- Case 2: 8 ports SRS resource is mapped in TDM manner, where 8 ports are equally partitioned into subsets.

For case 1, there is no significant difference compared with legacy UE capabilities for SRS AS (e.g. 2t2r, 4t4r), the only difference is the number of ports mapped to the SRS resource. For case 2, it can also be covered by scenarios 2 as mentioned above.

Companies also commented that SRS location in any position within a slot shall be considered for 8 Tx. However, this functionality was already introduced since Rel-16 which is not directly related to the supporting of 8 Tx for SRS AS.

| srs-StartAnyOFDM-Symbol-r16 Indicates whether the UE supports transmitting SRS starting in all symbols (0 to 13) of a slot. This capability is also applicable to a frequency band that does not require shared spectrum access. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Observation 4: SRS located in any symbols within a slot was introduced in Rel-16 which is not directly related to the supporting of 8 Tx for SRS AS.

Based on the analysis above, it seems the existing categorization of SRS configuration for SRS AS can still fit the “t8r8” capable UE. Thus, there is no RRM impacts of SRS enhancement.

Proposal 2: There is no RRM impact of SRS enhancement.

UL precoding indication for STxMP

For uplink transmission enhancement, RAN1 agreed to support both single-DCI based STxMP PUSCH transmission and multi-DCI based STxMP PUSCH+PUSCH transmission for UL precoding indication.

Based on RAN1’s agreements, the following aspects are supported for single-DCI based STxMP PUSCH transmission

- SDM scheme for single-DCI based STxMP PUSCH transmission

  - Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission

  - Support RRC-based switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme

- SFN scheme for single-DCI based STxMP PUSCH transmission

  - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission

For single-DCI based STxMP PUSCH transmission, the enhancements are focused on DMRS port indication and SRS resource set indication.

Based on RAN1’s agreements, the following combinations are supported for multi-DCI based STxMP PUSCH+PUSCH transmission

- DG PUSCH + DG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission

- CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission

For multi-DCI based STxMP PUSCH transmission, the enhancements are focused on SRS resource set indication/configuration and the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH.

Observation 5: The enhancements for UL precoding indication for STxMP are mainly focused on DMRS port indication and SRS resource set indication/configuration.

It can be observed that the uplink transmission enhancements for UL precoding indication for STxMP PUSCH transmission have no impact on the exising RRM requirements.

Proposal 3: There is no RRM impact of UL precoding indication for STxMP PUSCH transmission.

Conclusions

In this paper we provided our analysis on RRM impacts for R18 MIMO evolution. The followings are provided.

Observation 1: For TDCP reporting, UE performs TRS based measurements and report the wideband normalized amplitude/phase for each delay, where the reported normalized amplitude/phase is different with any type of the existing reportQuantity configured in a CSI report.

Proposal 1: RAN4 need to study whether it is necessary to define TRS based measurement requirements (e.g. measurement period and measurement accuracy) for TDCP reporting.

Observation 2: There is no RRM impacts for SRS enhancement for CJT.

Observation 3: In existing requirements for SRS antenna port switching, SRS resource for antenna port switching are assumed to be allocated in the last 6 symbols in a slot.

Observation 4: SRS located in any symbols within a slot was introduced in Rel-16 which is not directly related to the supporting of 8 Tx for SRS AS.

Proposal 2: There is no RRM impact of SRS enhancement.

Observation 5: The enhancements for UL precoding indication for STxMP are mainly focused on DMRS port indication and SRS resource set indication/configuration.

Proposal 3: There is no RRM impact of UL precoding indication for STxMP PUSCH transmission.

Reference

- R4-2303258, “WF on R18 NR MIMO RRM requirements”, Samsung