3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305325

Electronic Meeting, April 17th – April 26th, 2023

Title: 	Discussion on RRM impacts for R18 sidelink relay enhancement

Source: 	Huawei, HiSilicon

Agenda item:	5.33.2

Document for:	Discussion

Introduction

In RAN4#106 meeting, the RRM impacts for R18 sidelink relay enhancement were initially discussed, and the following issues have been captured in WF [1] for further study.

Issue 1-1: Selection and reselection requirements of relay UE - Proposals - Option 1 (MTK, LGE, Nokia): The legacy requirements for selection/reselection of relay UE in clause 12.10 of TS 38.133 can be reused. But, depending on RAN2 progress/decision, it can be further discussed - Option 2 (Ericsson): - RAN4 to investigate whether the existing requirements in clause 12.10 (Selection / Reselection of relay UE) of TS 38.133 in Rel-17, can also apply to the selection/reselection of the relay UE in multi-path scenario i.e. when the remote/source UE has also direct path with gNB. - For single hop UE-to-UE scenario, - RAN4 to investigate measurement period within which the source UE should discover/identify the relay UE for establishing 1-hop UE-to-UE relay connection under certain side conditions. - RAN4 to investigate measurement period within which the source UE should select/reselect the relay UE under certain side conditions. <Way forward> - No decision. RAN4 continue discuss whether selection and reselection requirements of relay UE for multi-path case are in the WI scope. Issue 1-2: Interruption time - Proposals - Option 1 (Ericsson): RAN4 to investigate maximum allowed interruption requirements caused by the source/remote UE on the direct path when the source/remote UE performs the selection/reselection of the indirect path <Way forward> - Continue discussion whether to introduce interruption time requirement.

In this contribution, we will provide the further discussion on the RRM requirements of R18 sidelink relay enhancement.

Discussion

Based on the WID [2] on NR SL relay enhancements, RAN4 needs to specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay. In R17, the requirements on selection/reselection of relay UE have been defined for NR SL relay. The remote UE performs SD-RSRP/SL-RSRP measurements of the candidate relay UEs. The measurement period and the evaluation period of SD-RSRP/SL-RSRP measurements are defined in RAN4 specification for relay UE selection/reselection procedure. In R18, based on RAN2 progress, the remote UE still uses SD-RSRP/SL-RSRP measurements for relay UE selection/reselection. So, the current SD-RSRP/SL-RSRP measurement requirements for relay UE selection/reselection in R17 can be reused in R18. For relay UE selection/reselection in R18, RAN2 considered to introduce new event to trigger a measurement report. The introduction of new event only impacts on how UE to compare the measured SD-RSRP/SL-RSRP of candidate/serving relay UE, but the measurement period and the evaluation period of SD-RSRP/SL-RSRP can be reused.

For R18 sidelink relay enhancement, the mechanisms to support multi-path with relay need to be studied in RAN2 and RAN3, where the remote UE is connected to the same gNB using one direct path (via L2 UE-to-Network relay) and one indirect path (via another UE). For R17 sidelink relay solution, the remote UE is able to be connected to the gNB using either direct path or indirect path, and the switching between direct path and indirect path is also supported. The remote UE performs SD-RSRP/ SL-RSRP measurements for the switching between direct path and indirect path. From RAN4 perspective, whether to use direct path or indirect path has no impact on the relay UE selection/reselection requirements.

Proposal 1: For relay UE selection/reselection procedure, the current R17 requirements on measurement period and evaluation period of SD-RSRP/SL-RSRP can be reused in R18.

For Rel-17 relay solution, the remote UE is connected to the gNB using either direct path or indirect path. For multi-path relay solution in R18, the remote UE is connected to the gNB using both direct path and indirect path. How to configure the resource pools for discovery message and data transmission belongs to the resource allocation issue which shall be discussed in RAN2. When UE performs the selection/reselection of the indirect path, the resource pools for discovery message and data transmission may need to be updated accordingly, which belongs to the resource allocation issue rather than the interruption issue. There is no need to define interruption requirements due to the selection/reselection of the indirect path.

Proposal 2: For multi-path relay solution in R18, there is no need to define interruption requirements due to selection/reselection of the indirect path.

Conclusions

In this paper we provided our analysis on RRM impacts for R18 sidelink relay enhancement. The followings are provided.

Proposal 1: For relay UE selection/reselection procedure, the current R17 requirements on measurement period and evaluation period of SD-RSRP/SL-RSRP can be reused in R18.

Proposal 2: For multi-path relay solution in R18, there is no need to define interruption requirements due to selection/reselection of the indirect path.

Reference

- R4-2303266, “WF on R18 SL Relay RRM requirements”, LG Electronics