3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305330

Electronic Meeting, 17 – 26 April, 2023

Title: 	Discussion on collision handling for MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	5.26.2.2

Document for:	Discussion

Introduction

Collision handling related to MUSIM gaps are discussed in RAN4#106 and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- MUSIM gap priority

- Collision between different MUSIM gaps

- Collision between MUSIM and legacy MG

- Collision between MUSIM gaps and NW A signals

In this paper we will provide our views on open issues in collision handling related to MUSIM gaps.

Discussion

MUSIM gap priority

Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side - Agreements - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps - It is up to NW A on how to use this information Recommended WF - Focus and discuss how UE “UE can optionally indicate its preferred priority for all or a subset MUSIM gaps”

In our view, the set of priorities used in the UE indication is separate from the set of priorities used in NW configuration. The reason is that UE may not have same understanding as NW on the meaning of a priority level, and it is out of UE responsibility to suggest priority between a MUSIM gap and a legacy MG. The legacy MG may not be even configured when UE requests MUSIM gaps.

Therefore, the indication of preferred priority provides NW information about the relative priority between different MUSIM gaps. One simple example is to allow UE to indicate the suggested priority level among {1,2,3,4} since at most 4 MUSIM gaps can be configured. When UE indicates ‘1’ for a MUSIM gap, it does not mean NW should configure GapPriority = 1 for it, but it means this MUSIM gap is more important than another MUSIM gap for which UE indicates ‘2’.

Proposal 1: UE indicates relative priority (4 levels) among MUSIM gaps.

Issue 2-1-4: Constraints on MUSIM gap priority configuration from NW A - Proposals - When MUSIM gaps’ priority are up to NW-A configuration - P1: NW A maintaining the same relative priorities requested by the UE (Qualcomm vivo MTK) - P1a: If UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (Qualcomm) - P1b: If UE requests MUSIM gap1 with priority X1 and MUSIM gap2 with priority X2, where X1 > X2, and if network A configures both gaps, then both gaps must be assigned priorities X1’ and X2’ such that X1’ > X2’. X1’ may or may not be equal to X1. X2’ may or may not be equal to X2. (Qualcomm) - P3: NW A could allocate higher priority for MUSIM gaps with longer MGRP (vivo) - P4: NW A treat the MUSIM gaps with the highest/second highest priority indicated by UE as aperiodic MUSIM gap or MUSIM gap for paging purpose (implicitly indicated); NW A could configure relative higher priority for these MUSIM gaps (vivo) - P5: MUSIM paging gap and aperiodic gap can have higher priority than NW-A’s MGs (Ericsson) - P6: 1 single priority applicable for all periodic MUSIM gaps. 1 priority for each aperiodic MUSIM gap. Aperiodic MUSIM gaps can be assigned with different priorities to the priority of the periodic MUSIM gaps (Nokia) Recommended WF Continue discussion

We think it is reasonable that NW A follows the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps. This is elaborated in P1a and P1b of Issue 2-1-4. Otherwise it does not make much sense for UE to indicate the preferred priority for MUSIM gaps. It is out of NW A’s responsibility to decide the relative priority among MUSIM gaps.

Proposal 2: NW A should maintain the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps.

Based on the agreement for Issue 2-1-2, UE may not indicate preferred priority for all MUSIM gaps. RAN4 should discuss how NW A is supposed to configure priority for such gaps. At least for a periodic gap without UE preferred priority (aperiodic is discussed next), it is straightforward that the priority of this MUSIM gap relative to other MUSIM gaps are up to NW A configuration.

Proposal 3: If UE does not indicate preferred priority for a periodic MUSIM gap, the priority of this MUSIM gap relative to other MUSIM gaps is up to NW A configuration.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps - Proposals - P1: When collides with legacy measurement gaps or MUSIM gaps, aperiodic gap shall be kept (Apple ZTE oppo vivo Huawei Ericsson) - P2: Prefer to allocate priority level for aperiodic MUSIM gap (Charter xiaomi ZTE vivo Qualcomm Nokia) - P3: No need to assign priority of aperiodic MUSIM gap (Apple Huawei Ericsson ZTE) - P4: It is not mandatory to assign priority for an aperiodic MUSIM gap and the highest priority is assumed by default (oppo MTK) Recommended WF Continue discussion

Two options for handling collision between aperiodic MUSIM gap and other gaps are discussed.

- Option 1: aperiodic gap is always kept

- Option 2: use priority rule and treat periodic and aperiodic gaps equally

We think both options can work, and we slightly prefer option 1. In our view, AP gap by its nature should be always prioritized. It is noted that AP gap is still configured by NW A, i.e. NW A can choose to configure the requested AP gap or not. If NW A chooses to configure the AP gap, it does not make much sense to associate it with a lower priority than legacy MG, as it will cause the AP gap to be dropped, and it is same as not configuring the AP gap since AP gap has only one occasion. If option 1 is adopted, there is no need for UE to request or for NWA to configure a priority for AP MUSIM gap.

Proposal 4: When colliding with legacy MG or other MUSIM gaps, aperiodic gap shall be kept. No need for UE to request or for NWA to configure a priority for aperiodic gap.

Collision between different MUSIM gaps

Issue 2-2-1: Definition of the collision between different MUSIM gaps - Proposals - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (Apple CMCC ZTE oppo xiaomi vivo MTK) - Option 2: No definition for collisions between MUSIM gaps is needed. (Huawei Nokia) - Option 3: No collisions between MUSIM gaps that have the same priority level (Qualcomm). Recommended WF Continue discussion

Whether a definition of the collision between different MUSIM gaps is needed depends on the solution used to handle the collision. As discussed next, if keep solution is used then there is no need for such definition. Otherwise, we agree that option 1 is reasonable. We suggest to postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Proposal 5: Postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Proposals - Option 1: Priority based solution is used for collision between different MUSIM gaps (Apple oppo vivo MTK) - Option 1a: Priority based solution is used for collision between different MUSIM gaps, if multiple MUSIM gaps are assigned different priority levels (Qualcomm) - Option 2: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide (Huawei) - Option 2a: Keep solution is used under particular conditions (xiaomi vivo oppo Ericsson Qualcomm) - Option 3: Consider combine both option 1 and 2 as the solution (ZTE) - Option 3a (ZTE): - The aperiodic gap has higher priority than other periodic gaps, the priority handling rule shall be used if it collides with the periodic gaps (except the paging gap). - The paging gap should not be dropped, the kept/merged solution is used if the second gap is paging gap. - Otherwise, the priority handling rule will be used among MUSIM gaps. - Option 4: Collision between periodic and aperiodic MUSIM gaps are handled by priorities (Nokia) Recommended WF Continue discussion

In our view, MUSIM gaps should not be dropped due to collision with another MUSIM gap.

All the MUSIM gaps are requested by the UE, and if two MUSIM gaps are colliding based on UE request, it would mean that UE needs to use both of them. For example, one MUSIM gap may be used for serving cell measurement (sync/AGC) and another MUSIM gap may be used for paging, and depending on the distance between PO and SSB in NW B, it could happen that those two gaps are colliding, and in this case no gap should be dropped. We think it is a typical use case and should be supported.

Some companies raised the concern that two colliding MUSIM gaps may be used for accessing different frequency layers in NW B, or for other operations that cannot be done at same time. We understand UE would not request such colliding MUSIM gaps when it cannot use both of them, as it will only lead to less scheduling opportunity for this UE in NW A without any benefit for its operation in NW B. UE can make MUSIM gaps non-colliding by using larger gap periodicity or by utilizing different SMTC occasions with different offsets in an DRX cycle.

Therefore, we understand in most cases where UE requests colliding MUSIM gaps, UE would prefer to use all of them for NW B operation, and it is rare case where UE cannot avoid the collision and can only use one of the MUSIM gaps. We cannot agree to only use priority solution as in option 1, as it is too restrictive for UE implementation and precludes typical use cases.

Proposal 6: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide.

Issue 2-2-3: Conditions on “keep solution” is used during collision between different MUSIM gaps - Proposals - Keep solution (keep all collided MUSIM gap) is used when - P1: Conditions when “keep solution” are used (vivo): - when the collided MUSIM gaps are not physically overlapping and the distance between them is less than 4ms; - UE has the capability to handle the two collided MUSIM gaps when they are not overlapped however the distance between them is less than 4 ms - These “kept” MUSIM gaps measure MOs at the same frequency layer (xiaomi) - P2: Keep collided MUSIM gaps only when the involved MUSIM gaps are configured with the highest priority, and the time distance is smaller than X[ms]. FFS: the value of X (oppo) - P3: When the time duration between the two closest gap occasions within the two measurement gap patterns is shorter than [4]ms (Ericsson) - if the second gap occasion is for paging, UE should keep both gap occasions - P4: Keep all MUSIM gaps when these MUSIM gaps have the same priority level, regardless of proximity or overlap between them (Qualcomm) Recommended WF Continue discussion

If Proposal 6 is not agreeable, then a compromise is to both keep solution and priority solution are supported, and define applicability conditions for two solutions.

The proposed conditions in Issue 2-2-3 have some issues. On P1, it is undesirable to differentiate collision cases as it will complicate the discussion. Also, NW A is not aware of whether colliding MUSIM gaps are to measure MOs at the same frequency layer or not. On P2/3, they are restrictive in that keep solution is used only for collision with a specific MUSIM gap. On P4, priority is used to indicate the importance of the MUSIM gaps, and it may not be coupled with whether colliding MUSIM gaps can be kept or not where there is a collision. For example, MUSIM gaps for paging and SSB measurement may be suggested with different priority, but they can be used at same time.

Since whether UE can use all of the colliding MUSIM gaps depends on the usage of each colliding MUSIM gap, which is up to UE implementation, a reasonable way to select between keep and priority solutions is to base on UE request, i.e. UE can indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps.

Furthermore, based on agreements related to MUSIM gap priority in RAN4#106, it is possible that UE does not indicate preferred priority for one or more MUSIM gaps, and it is unclear how to apply the priority solution in this case. Of course, NW A may configure a priority for such MUSIM gaps, but it is unreasonable to have NW A to decide which one to keep/drop when different MUSIM gaps collide because none of the gaps is used for NWA operation. Also, it is unclear so far if UE can indicate equal preferred priority for multiple MUSIM gaps, and if possible, how to apply priority solution is also a question.

Proposal 7: If Proposal 6 is not agreeable, both keep solution and priority solution are supported. Whether keep solution or priority solution is used is based on UE request, i.e. UE can indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps.

- FFS how to apply the priority solution when colliding MUSIM gaps are with equal preferred priority or without preferred priority.

Collision between MUSIM and legacy MG

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority - Proposals - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (Huawei vivo Nokia) - P2: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm) - P3: Collision is be handled based on the MGRP of the collided gaps (Ericsson MTK) - P3-1: Prioritize the gap with longer MGRP for the following MUSIM collision scenarios (Ericsson) - Any of the collision gaps is Type-1 MG; - NW-A doesn’t configure a priority associated with any of the collision gaps - P4: The sharing rule solution could be considered. (xiaomi) Recommended WF FFS on solutions for this issue.

In last meeting, some companies raised the concern that some NW may not support the signaling to configure type-2 MG (e.g. NW does not support con-MG) but support configuring MUSIM gaps, so the collision handling between MUSIM gaps and type-1 MG should be considered. We think it is a valid concern, and prioritizing gap with longer MGRP is also reasonable because otherwise gap with longer MGRP will be completely dropped.

The next question is whether to extend the principle to scenario where MUSIM gaps are not configured with a priority. The case may happen when NW does not support the signaling to configure priority for MUSIM gaps, so we support to consider the extension.

The next question is what happens when the colliding gaps are with same MGRP. In our view, no requirements shall apply because the collision cannot be resolved in an easy way. Basically, when a legacy MG and a MUSIM gap collide, NW A should either configure priority for all colliding gaps, or to configure different MGRPs for all colliding gaps.

Proposal 8: When a MUSIM gap collides with a legacy MG and if any of the colliding gaps is not configured with priority, prioritize the gap with longer MGRP. No requirements apply if the two gaps have same MGRP.

Collision between MUSIM gaps and NW A signals

Issue 2-4-1: Definition of the collision between MUSIM gaps and L1/L3 measurement resources - Proposals - P1: Update agreement at RAN4 105 as the following: (xiaomi vivo Ericsson) - A L1/L3 measurement resource is considered to be [partially or fully] overlapped with a periodic MUSIM gap if it [partially or fully] overlaps a MUSIM gap occasion in time domain - A L1/L3 measurement resource is considered to be [partially or fully] overlapped with an aperiodic MUSIM gap if it [partially or fully] overlaps that aperiodic MUSIM gap occasion in time domain - P2 (Nokia): - A L1/L3 measurement resource is considered to be [partially or fully] overlapped with a MUSIM gap if it is [partially or fully] overlapping with the MUSIM gap occasion in time domain Recommended WF Continue discussion

We support to remove the [] in the collision definition. One minor issue is that based on Rel-15 spec, when we say L1/L3 measurement resource fully overlap with a gap, it means all occasions of the L1/L3 measurement resource are fully or partially overlapping with occasions of the gap. In this sense, L1/L3 measurement resource cannot fully overlap with an aperiodic gap because the latter has only one occasion.

Proposal 9: Update the RAN4#105 agreement as follows.

- A L1/L3 measurement resource is considered to be partially or fully overlapped with a periodic MUSIM gap if it partially or fully overlaps a MUSIM gap occasion in time domain

- A L1/L3 measurement resource is considered to be partially overlapped with an aperiodic MUSIM gap if it partially or fully overlaps that aperiodic MUSIM gap occasion in time domain

Issue 2-4-2: Priority of MUSIM against SMTC for L3/ L1 measurement - Proposals - P1: MUSIM gaps have higher priority when colliding with SMTC/SSB for L3/L1 measurement (collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and measurement gaps) (Apple xiaomi oppo vivo Huawei Ericsson MTK) - P2: RAN4 shall strike for optimization between MUSIM gaps and SMTC/L1 in NW A. (Apple) - P3: RAN4 to consider other options than only having a fixed MUSIM priority over SMTC, and other L3/ L1 measurement resources (Nokia, Ericsson) Recommended WF Continue discussion

We understand the issue is about priority of MUSIM against SMTC for L3/L1 measurement, i.e. periodic measurement. It is straightforward to apply the same principle as for collisions between L3/L1 measurement resources and measurement gaps. If too many L1/L3 measurement resource occasions are punctured by MUSIM gaps, NW A can just avoid configuring some MUSIM gaps.

Proposal 10: Collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and legacy MGs.

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation - Proposals Recommended WF - Agreements: - FFS on collision between SMTC and MUSIM gaps for handover and Scell activation Continue discussion Issue 1-1-1: Clarification on the scope - Proposals - P1: Add the following note for the sentence “Case 2: Collisions between MUSIM gap and SMTC” (Qualcomm vivo Huawei) - Note: The scope collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements Recommended WF No consensus Issue 1-1-2: Considerations on one-shot RRM mobility procedures - Proposals - P1: RAN4 to define the priorities for each procedure in either NW-A or NW-B in desending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens. - Level 1: One-shot RRM mobility procedures in NW-A, such as Handover/ Re-establishment/RRC redirection/SCell activation/SI update; - Level 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading; - Level 3: Measurements procedures for both NW-A and NW-B - P2: No need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection. (vivo) - Agreements: - No need to consider the collision between SMTC and MUSIM gaps for RRC Re-establishment, RRC Connection Release with Redirection - FFS on collision between SMTC and MUSIM gaps for handover and Scell activation

We do not support to define special collision between MUSIM gaps and SMTC for handover and SCell activation. While we can understand the point, we are not sure if RAN4 needs to specify such details for MUSIM gaps when the corresponding prioritization has not been specified for legacy MGs, i.e. it is already possible to have legacy MG colliding with those procedures, but RAN4 has not defined how to handle the collision between NW A MG and all the procedures.

For example, the SCell activation delay can be several SMTC periods, and when MG is configured, it could happen that MG is colliding with SMTC of the SCell. It is clear that UE cannot perform MG based RRM measurement and SCell activation at the same time, and as such the requirements for one of the procedures need to be relaxed, but so far RAN4 has not defined the what happens in this case. We believe collision with legacy MG is more relevant than collision with MUSIM gaps in real deployment.

In addition, besides legacy MG and MUSIM gaps, there are also other sources of interruption, e.g. measurement of deactivated SCell, SRS carrier or antenna switching, BWP switching, etc. The question is whether RAN4 should define requirements or prioritization for all collisions between all interruption sources and all important RRM procedures/important transmissions or receptions.

Based on above, we suggest not to define specific handling for collision between MUSIM gaps and specific RRM procedures / transmissions or receptions in NW A.

Proposal 11: No special handling is defined for collision between SMTC and MUSIM gaps for handover and SCell activation.

Conclusions

In this paper we provided our views on open issues in collision handling related to MUSIM gaps.

Proposal 1: UE indicates relative priority (4 levels) among MUSIM gaps.

Proposal 2: NW A should maintain the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps.

Proposal 3: If UE does not indicate preferred priority for a periodic MUSIM gap, the priority of this MUSIM gap relative to other MUSIM gaps is up to NW A configuration.

Proposal 4: When colliding with legacy MG or other MUSIM gaps, aperiodic gap shall be kept. No need for UE to request or for NWA to configure a priority for aperiodic gap.

Proposal 5: Postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Proposal 6: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide.

Proposal 7: If Proposal 6 is not agreeable, both keep solution and priority solution are supported. Whether keep solution or priority solution is used is based on UE request, i.e. UE can indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps.

- FFS how to apply the priority solution when colliding MUSIM gaps are with equal preferred priority or without preferred priority.

Proposal 8: When a MUSIM gap collides with a legacy MG and if any of the colliding gaps is not configured with priority, prioritize the gap with longer MGRP. No requirements apply if the two gaps have same MGRP.

Proposal 9: Update the RAN4#105 agreement as follows.

- A L1/L3 measurement resource is considered to be partially or fully overlapped with a periodic MUSIM gap if it partially or fully overlaps a MUSIM gap occasion in time domain

- A L1/L3 measurement resource is considered to be partially overlapped with an aperiodic MUSIM gap if it partially or fully overlaps that aperiodic MUSIM gap occasion in time domain

Proposal 10: Collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and legacy MGs.

Proposal 11: No special handling is defined for collision between SMTC and MUSIM gaps for handover and SCell activation.

Reference

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo