3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305337

Electronic Meeting, 17 – 26 April, 2023

Title: 	Initial simulation results for PRS measurement with 1RX

Source: 	Huawei, HiSilicon

Agenda item:	5.23.3.4

Document for:	Discussion

Introduction

RRM impacts for Rel-18 positioning are discussed in RAN4#106, and the outcomes are captured in [1]. One objective is to support RedCap positioning. For the case without FH, RAN4 agreed that existing requirements can be re-used for 2RX RedCap, while for 1RX RedCap further link level simulation is needed.

- Issue 4-1-1: PRS measurement requirements for 2Rx without frequency hopping: Agreements: - Reuse Rel. 17 core requirements to define core requirements for RedCap UE positioning in RRC_CONNECTED state for 2Rx RedCap UE for FDD/TDD without frequency hopping. FFS on RRC_INACTIVE state.

Issue 4-1-2: PRS measurement requirements for 1Rx without frequency hopping: Agreements: - RAN4 to evaluate whether it is feasible to reuse the existing PRS measurement periods in 38.133, Rel-17 for PRS measurements for 1Rx RedCap UE and the impact on side conditions and PRS measurement accuracies. - The evaluation is based on link level simulations.

RAN4 has agreed simulation assumption for 1RX in [2]. In this paper we will provide our initial simulation results for PRS measurement with 1RX.

Discussion

Simulation results for 1RX are provided for FR1 only. As discussed in our companion paper, 1RX is not applicable to RedCap in FR2. In addition, although in [2] the number of samples are defined as 4, we also simulated single sample because corresponding requirements exist for 2RX in the spec. The simulated cases include:

- RSTD

  - 4-sample, Es/Iot of (-6,-13)dB, AWGN

  - 4-sample, Es/Iot of (-6,-13)dB, TDL-A

  - 1-sample, Es/Iot of (-3,-6)dB, AWGN

- UE Rx-Tx

  - 4-sample, Es/Iot of -3dB, AWGN

  - 4-sample, Es/Iot of -13dB, AWGN

  - 4-sample, Es/Iot of -3dB, TDL-A

  - 4-sample, Es/Iot of -13dB, TDL-A

  - 1-sample, Es/Iot of 0dB, AWGN

  - 1-sample, Es/Iot of -6dB, AWGN

The simulation results for RSTD are in Table 1-3, and UE Rx-Tx in Table 4-9. Current requirements for 2RX in the spec are also listed for comparison. The value 9999 means for the TOA error is very large and like a random value, i.e. the first path is not detected.

Table 1: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, AWGN

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 180 | 132 |
|  |  | 52 | 1 | 136 | 98 |
|  |  | 104 | 1 | 75 | 42 |
|  | 30 | 24 | 4 | 112 | 75 |
|  |  | 48 | 1 | 75 | 48 |
|  | 60 | 24 | 4 | 78 | 50 |

Table 2: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, TDL-A

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 267 | 247 |
|  |  | 52 | 1 | 9999 | 140 |
|  |  | 104 | 1 | 9999 | 86 |
|  | 30 | 24 | 4 | 192 | 118 |
|  |  | 48 | 1 | 9999 | 109 |
|  | 60 | 24 | 4 | 153 | 147 |

Table 3: RSTD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 172 | 98 |
|  |  | 104 | 1 | 98 | 42 |
|  | 30 | 48 | 1 | 102 | 48 |

Table 4: UE Rx-Tx error for 4-sample, Es/Iot of -3dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 112 | 78 |
|  |  | 52 | 1 | 88 | 59 |
|  |  | 104 | 1 | 42 | 30 |
|  | 30 | 24 | 4 | 75 | 57 |
|  |  | 48 | 1 | 48 | 30 |
|  | 60 | 24 | 4 | 50 | 29 |

Table 5: UE Rx-Tx error for 4-sample, Es/Iot of -13dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 135 | 101 |
|  |  | 52 | 1 | 118 | 75 |
|  |  | 104 | 1 | 52 | 37 |
|  | 30 | 24 | 4 | 92 | 58 |
|  |  | 48 | 1 | 63 | 39 |
|  | 60 | 24 | 4 | 60 | 36 |

Table 6: UE Rx-Tx error for 4-sample, Es/Iot of -3dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 216 | 137 |
|  |  | 52 | 1 | 172 | 96 |
|  |  | 104 | 1 | 124 | 62 |
|  | 30 | 24 | 4 | 156 | 87 |
|  |  | 48 | 1 | 122 | 68 |
|  | 60 | 24 | 4 | 118 | 59 |

Table 7: UE Rx-Tx error for 4-sample, Es/Iot of -13dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 245 | 180 |
|  |  | 52 | 1 | 9999 | 98 |
|  |  | 104 | 1 | 9999 | 68 |
|  | 30 | 24 | 4 | 175 | 87 |
|  |  | 48 | 1 | 9999 | 85 |
|  | 60 | 24 | 4 | 144 | 139 |

Table 8: UE Rx-Tx error for 1-sample, Es/Iot of 0dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 75 | 59 |
|  |  | 104 | 1 | 47 | 30 |
|  | 30 | 48 | 1 | 46 | 30 |

Table 9: UE Rx-Tx error for 1-sample, Es/Iot of -6dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 96 | 75 |
|  |  | 104 | 1 | 58 | 37 |
|  | 30 | 48 | 1 | 55 | 39 |

Based on the results, we made the following observations.

Observation 1: For RSTD and UE Rx-Tx, with Es/Iot of -13dB and TDL-A channel, the first path detection performance is poor (detection rate <90%) with 1RX.

Observation 2: For RSTD and UE Rx-Tx, for other cases than Es/Iot of -13dB and TDL-A channel, the TOA estimation error is larger with 1RX.

The performance degradation with 1RX is due to the fact that UE cannot do combining across two RX branches, and the effective Es/Iot is lower compared to 2RX.

Conclusions

In this paper we provided our initial simulation results for PRS measurement with 1RX.

Observation 1: For RSTD and UE Rx-Tx, with Es/Iot of -13dB and TDL-A channel, the first path detection performance is poor (detection rate <90%) with 1RX.

Observation 2: For RSTD and UE Rx-Tx, for other cases than Es/Iot of -13dB and TDL-A channel, the TOA estimation error is larger with 1RX.

Reference

- R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson

- R4-2303318, Simulation assumptions for 1Rx RedCap UE PRS measurements, Ericsson