3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305343

Electronic Meeting, 17 – 26 April, 2023

Title: 	Discussion on option C for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	5.11.2.3

Document for:	Discussion

Introduction

A new Rel-18 WI [1] on support of BWP without restriction is approved in RAN#99 based on the following agreement.

Proposal: Noting that Option A is already “Mandatory with Capability signalling”, support the following optional UE features in Rel-18: - Option B-1-1 without early implementability; and - Option C; and - Option B-1-2 with the conditions that: - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability); and - The interruption related requirements will be decided and specified in RAN4

One objective of the WI is to support option C – NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD. In this paper we will provide our views on the RRM impact of option C.

Discussion

- For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1)

In our view, from L1 measurement perspective there is no difference between CD-SSB and NCD-SSB. In this sense, the existing L1 measurement requirements can be re-used. This is also the case for RedCap, i.e. no specific L1 measurement requirements are defined for NCD-SSB.

Proposal 1: RAN4 to confirm that existing L1 measurement requirements can be re-used for measurement of NCD-SSB within BWP with option C.

For L1 measurement, the impact on the RRM spec is mainly clarification of the applicability of existing requirements. Taking RLM requirements as example (BFR requirements are similar), the following applicability is defined in RedCap requirements.

- 8.1B.1 Introduction The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

In our view, the highlighted applicability can be re-used for non-RedCap UE that supports option C.

Proposal 2: Clarify in the spec that L1 measurement requirements apply for both CD- and NCD-SSB if UE supports [FG for C].

For RedCap, RAN4 defined specific transmission requirements for RLM and BFD when there is a BWP switching and the measured SSB changes from CD-SSB to NCD-SSB or vice versa.

- 8.1B.4 Minimum requirement at transitions When the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource during one evaluation period, the UE is allowed to restart the ongoing evaluation using the second SSB type after BWP switching. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource. The requirements in clause 8.1.4 shall also apply except the scenarios described above.

In our view, the requirements can be re-used for non-RedCap UE with option C.

Proposal 3: Re-use the requirements for transition between CD-SSB and NCD-SSB for RLM and BFD from RedCap.

For RedCap, RAN4 updated the applicability condition of UE timing requirements to include both CD- and NCD-SSB. Basically, both can be used for time tracking.

- 7.1A.2 Requirements The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

In our view, the applicability condition can be re-used for non-RedCap UE with option C.

Proposal 4: RAN4 to include NCD-SSB in the applicability condition for timing requirements, same as timing requirement for RedCap.

For RedCap, RAN4 updated the definition of intra- and inter-frequency measurements. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2]. Accordingly, RAN4 also defined requirements for transition of intra-frequency between CD-SSB NCD-SSB, transition of a measurement between intra- and inter-frequency.

### 9.2B.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2]. ### 9.1A.6 Minimum requirement at transitions When the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one cell identification period, the UE shall use a cell identification period that is the maximum of the cell identification periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a measurement period that is the maximum of the measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the measurement on one measurement object transitions from inter-frequency measurement to intra-frequency measurement or vice versa due to BWP switching during one cell identification period, the UE shall use a cell identification period that is the maximum of the cell identification periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the measurement on one measurement object transitions from inter-frequency measurement to intra-frequency measurement or vice versa due to BWP switching during one measurement period, the UE shall use a measurement period that is the maximum of the measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching . Subsequent to the above-mentioned measurement periods and/or cell identification periods, the cell identification and measurement period requirements on each measurement object are based on the SSB type after the BWP switch.

In our view, those requirements can be re-used for non-RedCap UE with option C.

Proposal 5: Re-use the intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap.

Besides, in HO requirements for RedCap, the applicable scenarios are clarified and HO to target cell’s specific Redcap BWP associated with NCD-SSB is supported in addition to HO to target cell’s initial BWP.

#### 6.1D.1.1 Introduction The purpose of NR handover is to change the NR PCell to another NR cell for RedCap UE. The requirements in this clause are applicable to SA NR. The requirements in this clause apply for the following handover scenarios: Handover to a target cell’s initial BWP associated with CD-SSB; Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP).

This means UE can perform RACH on the BWP containing either CD-SSB or NCD-SSB. In addition, for RedCap RAN4 agreed to support the scenario where the measured SSB and the target SSB for HO are different. In our view, those HO scenarios involving NCD-SSB can also be supported for non-RedCap UE with option C and same requirements from RedCap can be re-used.

Proposal 6: Support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap.

Conclusions

In this paper we provided our views on the RRM impact of option C.

Proposal 1: RAN4 to confirm that existing L1 measurement requirements can be re-used for measurement of NCD-SSB within BWP with option C.

Proposal 2: Clarify in the spec that L1 measurement requirements apply for both CD- and NCD-SSB if UE supports [FG for C].

Proposal 3: Re-use the requirements for transition between CD-SSB and NCD-SSB for RLM and BFD from RedCap.

Proposal 4: RAN4 to include NCD-SSB in the applicability condition for timing requirements, same as timing requirement for RedCap.

Proposal 5: Re-use the intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap.

Proposal 6: Support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap.

Reference

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Moderator (Vodafone), vivo