3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305348

Electronic Meeting, 17 – 26 April, 2023

Title: 	Reply LS on enhanced cell reselection requirements in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.1.2

Document for:	Approval

Introduction

RAN4 received LS [1] from RAN2#121 related to enhanced cell reselection requirements in NTN.

RAN2 respectfully asks RAN4 to check the recently agreed CR (in R2-2300470) and let RAN2 know if the feature definition for Enhanced RRM requirements for measurements in IDLE and INACTIVE still contains erroneous parts. In particular, please inform if INACTIVE requires separate description. RAN2 also asks RAN4 to pursue corresponding changes in RAN4 feature list.

In this paper we will provide our views on the questions asked in [1].

Discussion

Firstly, we think the change in RAN2 CR R2-2300470 is fine and aligned with the applicability of the normal and enhanced cell requirements in RAN4 spec.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, otherNTN measurement requirements (as specified in TS 38.133 [5], section 4.2C.2) are applied for both LEO and GEO.

Proposal 1: RAN4 to confirm the RAN2 CR R2-2300470 is aligned with the applicability of the normal and enhanced cell requirements in RAN4 spec.

Secondly, we think the same requirements apply to INACTIVE. Actually, the requirements for cell reselection in INACTIVE are missing in 38133. It should be added and the requirements for IDLE can be re-used.

Proposal 2: RAN4 to add requirements for cell reselection in INACTIVE in 38.133, and requirements for IDLE are re-used.

Thirdly, RAN2 asks RAN4 to update the FG 25-4 based on the RAN2 CR. We have provided the updated feature list in the Annex.

Conclusions

In this paper we provided our views on the question in the LS [1].

Proposal 1: RAN4 to confirm the RAN2 CR R2-2300470 is aligned with the applicability of the normal and enhanced cell requirements in RAN4 spec.

Proposal 2: RAN4 to add requirements for cell reselection in INACTIVE in 38.133, and requirements for IDLE are re-used.

A draft reply LS and the updated feature list is provided in the Annex.

Reference

- R2-2301966, Response LS on enhanced cell reselection requirements in NTN, RAN2

Annex A: draft reply LS on enhanced cell reselection requirements in NTN

3GPP TSG-RAN WG4 Meeting #106-bis-e R4-22xxxxx Electronic Meeting, 17 – 26 April, 2023 Title: Reply LS on enhanced cell reselection requirements in NTN Response to: R2-2301966 Release: Release 17 Work Item: NR_NTN_Solutions_Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the questions in LS R2-2301966. RAN4 confirms that the RAN2 CR R2-2300470 is aligned with the applicability of the normal and enhanced cell requirements in RAN4 spec. RAN4 also confirms that INACTIVE does not require separate description, and RAN4 will add requirements for cell reselection in INACTIVE in 38.133 by re-using the requirements for IDLE. RAN4 also updated the feature 25-4 which is provided in RAN4 feature list. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #107 May 22 – May 26, 2023 Incheon, South Korea RAN WG4 Meeting #108 Aug 21 – Aug 25, 2023 Toulouse, France

Annex B: Updated feature 25-4

| 25. NR_NTN_solutions | 25-4 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other NTN measurement requirements (as specified in TS 38.133 [5], section 4.2C.2) for both LEO and GEO. |  | No | N/A | UE does not support enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Per Band | FDD only | FR1 only | NA |  | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |