# 3GPP TSG-RAN WG4 Meeting # 106bis-e                              R4-2305351

# Electronic Meeting, 17th Apr–26th Apr, 2023

# Source: 	ZTE Corporation

# Title: 	Discussion on band definition for LTE based broadcast

# Agenda item:	6.3.2

# Document for:	Approval

# 1 Introduction

In RAN#95e meeting, the approved work item [1] includes the objective to specify band(s), within the portion of UHF spectrum allocated to broadcast systems (~470 - ~694/698 MHz, depending on the region), that support the channelization available to broadcasting operators. During the last RAN4 meeting, there were some open issues left for further discussion in [2]. In this contribution, we want to share some further views on band definition for LTE based broadcast.

From [1] two band plan options not mutually exclusive were identified - Define a single band covering the entire UHF frequency range 470 – [698/702] MHz - The requirements are relaxed for this band (TBD which requirements and how much) and/or - This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters) - Define smaller bands - At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter - Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz. - Other Agreement from online session: - Define the band plan based on available data for BS and UE. - Full UHF band, i.e., 470 – 698MHz, - with reduced UE blocking - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105. - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands. Way Forward - Further clarification and study is needed to elaborate on and quantify requirements with respect to reduced UE blocking for the full UHF band. - All technical requirements for these bands are still to be defined.

# 2 Discussion

## 2.1 Band definition

In last meeting, most companies tend to agree define a full band and a narrow band for LTE based broadcast. A new band type Standalone Downlink Only (SDO) was introduced during RAN4#104 e-meeting. In this paper, we want to give the following proposal for band definition.

Table 2.1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| [n107] | N/A | 470 MHz – 698 MHz | SDO |
| [n108] | N/A | 612 MHz – 652 MHz | SDO |

Based on the band definition given in Table 2.1, applicable NR-ARFCN can be defined as following.

Table 2.2: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n107 | 100 | N/A | 94000 – <20> – 139600 |
| n108 | 100 | N/A | 122400 – <20> – 130400 |

Proposal 1: To define the band for LTE based broadcast as in Table 2.1 and NR-ARFCN as in Table 2.2.

## 2.2 Reduced UE blocking

Since 5G broadcast band is Standalone Downlink Only band, the reduced blocking is feasible. According to EN 303 340, as shown in Table 2.3, the reduced blocking requirement can be defined as -25 dBm.

Table 2.3: Blocking requirements

| Test | Interferer(I) type | C Wanted signal centre frequency(MHz) | I centre frequency (MHz) | Crms (rms) (dBm) | Required blocking level Crms for DTT (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DVB-T | DVB-T2 |
| 1 | 10 MHz LTE 700 BS fully loaded | 690 | 763 | -71 | -25 |  |
| 2 | 10 MHz LTE 700 BS fully loaded | 690 | 763 | -69 |  | -25 |

Proposal 2: The reduced blocking is feasible and can be defined as -25 dBm.

# 3 Conclusion

In this contribution, we shared some views on band definition for LTE based broadcast and the proposals are made as following:

Proposal 1: To define the band for LTE based broadcast as in Table 2.1 and NR-ARFCN as in Table 2.2.

Proposal 2: The reduced blocking is feasible and can be defined as -25 dBm.

# 4 Reference

- RP-220518, “Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm

- R4-2303489, “WF on RF requirements for 5G Broadcast”, Qualcomm